-

The LONGLIB Graphics Library

Version 5.0

March 16, 1988

David Long

Jet Propulsion Laboratory

4800 Oak Grove Drive

Pasadena, California   91109

Page 2

CONTENTS

Table of Contents

CHAPTER 1       INTRODUCTION

1.1     GENERAL  . . . . . . . . . . . . . . . . . . . . . 1-1

1.2     MACHINE DEPENDENCE . . . . . . . . . . . . . . . . 1-2

1.3     USING LONGLIB WITH GRAPHICS TERMINALS  . . . . . . 1-3

1.3.1     Terminal Specific Application Notes  . . . . . . 1-4

1.3.1.1   VT100 (w/Selanar Graphics Cards) . . . . . . . . 1-4

1.3.1.2   VT125  . . . . . . . . . . . . . . . . . . . . . 1-5

1.3.1.3   VT240  . . . . . . . . . . . . . . . . . . . . . 1-5

1.3.1.4   VT220 (w/Selanar Graphics Card)  . . . . . . . . 1-5

1.3.1.5   Tektronix 4010/4014 Compatible Terminals . . . . 1-6

1.3.1.6   Tektronix 4107/4109 Compatible Terminals . . . . 1-6

1.3.1.7   Graphon Terminals  . . . . . . . . . . . . . . . 1-6

1.4     THE LONGLIB METAFILE (HARDCOPY OUTPUT) . . . . . . 1-6

1.4.1     Dot Matrix Printers  . . . . . . . . . . . . . . 1-7

1.4.1.1   Subprocesses . . . . . . . . . . . . . . . . . . 1-8

1.4.1.1.1 SUBROUTINE PSUBPRO . . . . . . . . . . . . . . . 1-8

1.4.1.1.2 SUBROUTINE SUBPROC . . . . . . . . . . . . . . . 1-8

1.4.2     HPGL Compatible Pen Plotter  . . . . . . . . . . 1-9

1.4.3     QMS QUIC Laser Printer . . . . . . . . . . . . . 1-9

1.4.4     PostScript . . . . . . . . . . . . . . . . . .  1-10

1.4.5     Plotting The LONGLIB Metafile To A Screen

Device . . . . . . . . . . . . . . . . . . . .  1-10

1.5     PLOTTING ON THE RAMTEK SCREEN GRAPHICS DEVICE  .  1-10

1.5.1     Ramtek Emulation File Plotting . . . . . . . .  1-11

1.5.1.1   Using The REF Routines . . . . . . . . . . . .  1-11

1.5.1.2   REF File Output  . . . . . . . . . . . . . . .  1-12

1.5.1.3   REF Terminal Or Metafile Output  . . . . . . .  1-12

1.6     LIBRARY LOCATION . . . . . . . . . . . . . . . .  1-12

1.6.1     LONGLIB Source Code  . . . . . . . . . . . . .  1-13

1.7     USER NOTES -- VAX/VMS ENVIRONMENT  . . . . . . .  1-13

1.7.1     Using The LONGLIB VAX/VMS On-line HELP Library  1-13

1.7.2     Using The LONGLIB VAX/VMS Shareable Image

Library  . . . . . . . . . . . . . . . . . . .  1-14

1.8     DISCLAIMER . . . . . . . . . . . . . . . . . . .  1-14

CHAPTER 2       PROGRAMMING EXAMPLES

2.1     FORTRAN PROGRAMMING EXAMPLES . . . . . . . . . . . 2-1

2.1.1     Simple MASTER Routine Example  . . . . . . . . . 2-1

2.1.2     Lower Level Routines . . . . . . . . . . . . . . 2-2

2.2     C LANGUAGE PROGRAMMING EXAMPLE . . . . . . . . . . 2-3

2.3     MASTER ROUTINE EXAMPLES (FORTRAN)  . . . . . . . . 2-3

2.3.1     Adding Additional Annotation To A MASTER Routine

Plot . . . . . . . . . . . . . . . . . . . . . . 2-4

2.3.2     Using Multiple MASTER Routines In The Same

Program  . . . . . . . . . . . . . . . . . . . . 2-4

2.3.3     MASTER Routine Color Example . . . . . . . . . . 2-5

2.3.4     Using Multiple MASTER Plots On Same Page/Screen  2-6

CHAPTER 3       DESCRIPTION OF PLOTTING ROUTINES

3.1     SUBROUTINE ABSPLT  . . . . . . . . . . . . . . . . 3-1

3.2     SUBROUTINE ARROW . . . . . . . . . . . . . . . . . 3-2

3.3     SUBROUTINE AXIS  . . . . . . . . . . . . . . . . . 3-2

3.4     SUBROUTINE AXIS2 . . . . . . . . . . . . . . . . . 3-3

3.5     SUBROUTINE AXIS3 . . . . . . . . . . . . . . . . . 3-4

3.6     SUBROUTINE CIRCLE  . . . . . . . . . . . . . . . . 3-5

3.7     SUBROUTINE CSHADE  . . . . . . . . . . . . . . . . 3-6

3.8     SUBROUTINE CTERM . . . . . . . . . . . . . . . . . 3-6

3.9     SUBROUTINE DASHL   . . . . . . . . . . . . . . . . 3-7

3.10    SUBROUTINE ELLIPSE . . . . . . . . . . . . . . . . 3-7

3.11    SUBROUTINE FACTOR  . . . . . . . . . . . . . . . . 3-8

3.12    SUBROUTINE FRAME . . . . . . . . . . . . . . . . . 3-8

3.13    SUBROUTINE GRID  . . . . . . . . . . . . . . . .  3-10

3.14    SUBROUTINE HLT3D . . . . . . . . . . . . . . . .  3-10

3.15    SUBROUTINE HELPME  . . . . . . . . . . . . . . .  3-11

3.16    SUBROUTINE JPLTAG  . . . . . . . . . . . . . . .  3-11

3.17    SUBROUTINE LGAXS . . . . . . . . . . . . . . . .  3-11

3.18    SUBROUTINE LGLIN . . . . . . . . . . . . . . . .  3-12

3.19    SUBROUTINE LGRID . . . . . . . . . . . . . . . .  3-13

3.20    SUBROUTINE LINE  . . . . . . . . . . . . . . . .  3-13

3.21    SUBROUTINE LINSEQ  . . . . . . . . . . . . . . .  3-14

3.22    SUBROUTINE NEWPAGE . . . . . . . . . . . . . . .  3-15

3.23    SUBROUTINE NEWPEN  . . . . . . . . . . . . . . .  3-15

3.24    SUBROUTINE NUMBER  . . . . . . . . . . . . . . .  3-16

3.25    SUBROUTINE PFACTOR . . . . . . . . . . . . . . .  3-17

3.26    SUBROUTINE PLOT  . . . . . . . . . . . . . . . .  3-17

3.27    SUBROUTINE PLOTND  . . . . . . . . . . . . . . .  3-18

3.28    SUBROUTINE PLOTS . . . . . . . . . . . . . . . .  3-18

3.29    SUBROUTINE PLOTRM  . . . . . . . . . . . . . . .  3-18

3.30    SUBROUTINE PLOTVT  . . . . . . . . . . . . . . .  3-19

3.31    SUBROUTINE PLRAX . . . . . . . . . . . . . . . .  3-20

3.32    SUBROUTINE PLRLN . . . . . . . . . . . . . . . .  3-20

3.33    SUBROUTINE PLT3D . . . . . . . . . . . . . . . .  3-20

3.34    SUBROUTINE PPLOT . . . . . . . . . . . . . . . .  3-21

3.35    SUBROUTINE PPEN  . . . . . . . . . . . . . . . .  3-22

3.36    SUBROUTINE RECT  . . . . . . . . . . . . . . . .  3-23

3.37    SUBROUTINE RESPL . . . . . . . . . . . . . . . .  3-23

3.38    SUBROUTINE RFACTOR . . . . . . . . . . . . . . .  3-23

3.39    SUBROUTINE RMPEN . . . . . . . . . . . . . . . .  3-24

3.40    SUBROUTINE RTERM . . . . . . . . . . . . . . . .  3-24

3.41    SUBROUTINE SAVPL . . . . . . . . . . . . . . . .  3-24

3.42    SUBROUTINE SCALE . . . . . . . . . . . . . . . .  3-25

3.43    SUBROUTINE SCALG . . . . . . . . . . . . . . . .  3-25

3.44    SUBROUTINE SHADE . . . . . . . . . . . . . . . .  3-26

3.45    SUBROUTINE SYMBOL  . . . . . . . . . . . . . . .  3-26

3.46    REAL FUNCTION SYMS . . . . . . . . . . . . . . .  3-27

3.47    REAL FUNCTION SYMSS  . . . . . . . . . . . . . .  3-30

3.48    SUBROUTINE VFACTOR . . . . . . . . . . . . . . .  3-30

3.49    SUBROUTINE VPEN  . . . . . . . . . . . . . . . .  3-31

3.50    SUBROUTINE WHERE . . . . . . . . . . . . . . . .  3-31

3.51    SUBROUTINE WHEREPR . . . . . . . . . . . . . . .  3-31

iii

3.52    SUBROUTINE WHERERM . . . . . . . . . . . . . . .  3-32

3.53    SUBROUTINE WHEREVT . . . . . . . . . . . . . . .  3-32

CHAPTER 4       DESCRIPTION OF 3-D PLOTTING ROUTINES

4.1     INIT3D ROUTINES  . . . . . . . . . . . . . . . . . 4-1

4.1.1     SUBROUTINE INIT3D  . . . . . . . . . . . . . . . 4-2

4.1.2     SUBROUTINE AXIS3D  . . . . . . . . . . . . . . . 4-2

4.1.3     SUBROUTINE NUM3D . . . . . . . . . . . . . . . . 4-3

4.1.4     SUBROUTINE PLOT3D  . . . . . . . . . . . . . . . 4-3

4.1.5     SUBROUTINE SYM3D . . . . . . . . . . . . . . . . 4-4

4.1.6     SUBROUTINE WHERE3D . . . . . . . . . . . . . . . 4-4

4.2     INIT3DH ROUTINES . . . . . . . . . . . . . . . . . 4-4

4.2.1     SUBROUTINE INIT3DH . . . . . . . . . . . . . . . 4-5

4.2.2     SUBROUTINE AXIS3DH . . . . . . . . . . . . . . . 4-6

4.2.3     SUBROUTINE CUBE  . . . . . . . . . . . . . . . . 4-7

4.2.4     SUBROUTINE NUM3DH  . . . . . . . . . . . . . . . 4-7

4.2.5     SUBROUTINE PLT3DH  . . . . . . . . . . . . . . . 4-7

4.2.6     SUBROUTINE SKETCH  . . . . . . . . . . . . . . . 4-8

4.2.7     SUBROUTINE SYM3DH  . . . . . . . . . . . . . . . 4-9

4.2.8     SUBROUTINE WHERE3H . . . . . . . . . . . . . . . 4-9

4.2.9     SUBROUTINE XFRM3D  . . . . . . . . . . . . . . . 4-9

CHAPTER 5       CURSOR ROUTINES

5.1     SUBROUTINE BITCURSOR . . . . . . . . . . . . . . . 5-1

5.2     SUBROUTINE CURLOCATE . . . . . . . . . . . . . . . 5-2

5.3     SUBROUTINE CURMOTION . . . . . . . . . . . . . . . 5-2

5.4     SUBROUTINE CURBAND . . . . . . . . . . . . . . . . 5-3

5.5     SUBROUTINE CURRECT . . . . . . . . . . . . . . . . 5-3

5.6     SUBROUTINE GETCURSOR . . . . . . . . . . . . . . . 5-4

CHAPTER 6       MAP SUBROUTINES

6.1     EARTH OUTLINE MAPS . . . . . . . . . . . . . . . . 6-1

6.1.1     SUBROUTINE EARTH3D . . . . . . . . . . . . . . . 6-2

6.1.2     SUBROUTINE LANDMAP . . . . . . . . . . . . . . . 6-2

6.1.3     SUBROUTINE POLARMAP  . . . . . . . . . . . . . . 6-2

6.1.4     SUBROUTINE SPRECT1 . . . . . . . . . . . . . . . 6-3

6.2     LAND AREA MAP ROUTINES . . . . . . . . . . . . . . 6-3

6.2.1     LNDSEA1.DAT Format . . . . . . . . . . . . . . . 6-3

6.2.2     SUBROUTINE BITMAP  . . . . . . . . . . . . . . . 6-4

6.2.3     INTEGER FUNCTION LNDSEA  . . . . . . . . . . . . 6-5

CHAPTER 7       MASTER SUBROUTINES

7.1     MASTER ROUTINE INDEX . . . . . . . . . . . . . . . 7-2

7.1.1     Pie Chart  . . . . . . . . . . . . . . . . . . . 7-2

7.1.2     Bar Chart  . . . . . . . . . . . . . . . . . . . 7-2

iv

7.1.3     Single Linear/linear Line  . . . . . . . . . . . 7-2

7.1.4     Two Linear/linear Lines On The Same Plot . . . . 7-2

7.1.5     Multiple Linear/linear Lines On The Same Plot  . 7-3

7.1.6     Multiple Log/linear Or Log/log Lines On The Same

Plot . . . . . . . . . . . . . . . . . . . . . . 7-3

7.1.7     Scatter Plot . . . . . . . . . . . . . . . . . . 7-3

7.1.8     1-d Histogram  . . . . . . . . . . . . . . . . . 7-3

7.1.9     Lines/points With Error Bars . . . . . . . . . . 7-3

7.1.10    Special Plot Formats . . . . . . . . . . . . . . 7-4

7.1.11    Contour Plot With Equally Spaced Data  . . . . . 7-4

7.1.12    Contour Plots With Unequally Spaced Data . . . . 7-4

7.1.13    3-d Surface Slices . . . . . . . . . . . . . . . 7-4

7.1.14    3-d Surface Mesh (no Hidden Line Removal): . . . 7-4

7.1.15    3-d Surface Mesh (hidden Line Removal) . . . . . 7-4

7.1.16    3-d Surface Mesh With Contour Plot (hidden Line

Removal) . . . . . . . . . . . . . . . . . . . . 7-5

7.1.17    3-d Surface Triangular Mesh (hidden Line

Removal) . . . . . . . . . . . . . . . . . . . . 7-5

7.1.18    Unequally Sampled 3-d Surface (hidden Line

Removal):  . . . . . . . . . . . . . . . . . . . 7-5

7.1.19    3-d Histogram With Hidden Line Removal . . . . . 7-5

7.1.20    3-d Contour Plot With Equally Spaced Data (no

Hidden Line Removal) . . . . . . . . . . . . . . 7-5

7.1.21    4/5-d Surface Plots: . . . . . . . . . . . . . . 7-5

7.2     SUBROUTINE BARCHR  . . . . . . . . . . . . . . . . 7-6

7.3     SUBROUTINE CNT3D . . . . . . . . . . . . . . . . . 7-8

7.4     SUBROUTINE CNT3DX  . . . . . . . . . . . . . . . . 7-8

7.5     SUBROUTINE CNTLN . . . . . . . . . . . . . . . .  7-11

7.6     SUBROUTINE CNTRPLT . . . . . . . . . . . . . . .  7-13

7.7     SUBROUTINE CVAX3D  . . . . . . . . . . . . . . .  7-16

7.8     SUBROUTINE CVAX3DX . . . . . . . . . . . . . . .  7-16

7.9     SUBROUTINE GLPLOT  . . . . . . . . . . . . . . .  7-20

7.10    SUBROUTINE HIST3D  . . . . . . . . . . . . . . .  7-22

7.11    SUBROUTINE LCNTR . . . . . . . . . . . . . . . .  7-24

7.12    SUBROUTINE LSPLOT  . . . . . . . . . . . . . . .  7-26

7.13    SUBROUTINE MESH3D  . . . . . . . . . . . . . . .  7-29

7.14    SUBROUTINE MESH3DX . . . . . . . . . . . . . . .  7-29

7.15    SUBROUTINE MVAX3D  . . . . . . . . . . . . . . .  7-32

7.16    SUBROUTINE MVAX3DX . . . . . . . . . . . . . . .  7-32

7.17    SUBROUTINE MVAX5D  . . . . . . . . . . . . . . .  7-35

7.18    SUBROUTINE PHIST . . . . . . . . . . . . . . . .  7-38

7.19    SUBROUTINE PICHRT  . . . . . . . . . . . . . . .  7-40

7.20    SUBROUTINE PLOTLG  . . . . . . . . . . . . . . .  7-42

7.21    SUBROUTINE PLOTLG2 . . . . . . . . . . . . . . .  7-44

7.22    SUBROUTINE PLOTLGX . . . . . . . . . . . . . . .  7-46

7.23    SUBROUTINE PLOTLGL . . . . . . . . . . . . . . .  7-48

7.24    SUBROUTINE PLOTSC  . . . . . . . . . . . . . . .  7-51

7.25    SUBROUTINE PLOTSC2 . . . . . . . . . . . . . . .  7-53

7.26    SUBROUTINE SCATPL  . . . . . . . . . . . . . . .  7-55

7.27    SUBROUTINE SEISPL  . . . . . . . . . . . . . . .  7-57

7.28    SUBROUTINE SPLOTS  . . . . . . . . . . . . . . .  7-59

7.29    SUBROUTINE SPLOTSX . . . . . . . . . . . . . . .  7-59

7.30    SUBROUTINE T3DH  . . . . . . . . . . . . . . . .  7-62

v

7.31    SUBROUTINE TRIG3DH . . . . . . . . . . . . . . .  7-64

7.32    SUBROUTINE VAX3D . . . . . . . . . . . . . . . .  7-66

7.33    SUBROUTINE VAX3DX  . . . . . . . . . . . . . . .  7-66

7.34    SUBROUTINE VAX5D . . . . . . . . . . . . . . . .  7-69

CHAPTER 8       MISCELLANEOUS ROUTINES

8.1     INTEGER FUNCTION IPCLIP  . . . . . . . . . . . . . 8-1

8.2     SUBROUTINE GCONTR  . . . . . . . . . . . . . . . . 8-1

8.3     INTEGER FUNCTION INXTCHR . . . . . . . . . . . . . 8-2

8.4     INTEGER FUNCTION IRMCHAN . . . . . . . . . . . . . 8-3

8.5     SUBROUTINE MATMUL4 . . . . . . . . . . . . . . . . 8-3

8.6     SUBROUTINE MTV4  . . . . . . . . . . . . . . . . . 8-3

8.7     SUBROUTINE NXTVU . . . . . . . . . . . . . . . . . 8-3

8.8     SUBROUTINE PAUSE . . . . . . . . . . . . . . . . . 8-4

8.9     SUBROUTINE PAUSEP  . . . . . . . . . . . . . . . . 8-4

8.10    SUBROUTINE RAMCLOSE  . . . . . . . . . . . . . . . 8-5

8.11    SUBROUTINE REFDIS  . . . . . . . . . . . . . . . . 8-5

8.12    SUBROUTINE RMCLEAR . . . . . . . . . . . . . . . . 8-6

8.13    SUBROUTINE RMDIR . . . . . . . . . . . . . . . . . 8-6

8.14    SUBROUTINE RMFNTSIZE . . . . . . . . . . . . . . . 8-6

8.15    SUBROUTINE RAMOPEN . . . . . . . . . . . . . . . . 8-7

8.16    SUBROUTINE RAMOUT  . . . . . . . . . . . . . . . . 8-7

8.17    SUBROUTINE RMPAN . . . . . . . . . . . . . . . . . 8-7

8.18    SUBROUTINE RMPLOT  . . . . . . . . . . . . . . . . 8-7

8.19    SUBROUTINE RMREADBYTE  . . . . . . . . . . . . . . 8-8

8.20    SUBROUTINE RMREADCOL . . . . . . . . . . . . . . . 8-8

8.21    SUBROUTINE RMREADCURSOR  . . . . . . . . . . . . . 8-8

8.22    SUBROUTINE RMREADWORD  . . . . . . . . . . . . . . 8-9

8.23    SUBROUTINE RMSETCUR  . . . . . . . . . . . . . . . 8-9

8.24    SUBROUTINE RMSTART . . . . . . . . . . . . . . . . 8-9

8.25    SUBROUTINE RMTEXT  . . . . . . . . . . . . . . . . 8-9

8.26    SUBROUTINE RMTEXTURE . . . . . . . . . . . . . .  8-10

8.27    SUBROUTINE RMWIND  . . . . . . . . . . . . . . .  8-10

8.28    SUBROUTINE RMWRITEBYTE . . . . . . . . . . . . .  8-10

8.29    SUBROUTINE RMWRITECOL  . . . . . . . . . . . . .  8-10

8.30    SUBROUTINE RMWRITEWORD . . . . . . . . . . . . .  8-11

8.31    SUBROUTINE RMZOOM  . . . . . . . . . . . . . . .  8-11

8.32    SUBROUTINE VTPLOT  . . . . . . . . . . . . . . .  8-11

8.33    SUBROUTINE TRIANGC . . . . . . . . . . . . . . .  8-12

8.34    REAL FUNCTION XVMUL3D  . . . . . . . . . . . . .  8-12

CHAPTER 9       LONGLIB LIBRARY NAMES

9.1     SUBROUTINE CALLS . . . . . . . . . . . . . . . . . 9-1

9.2     COMMON BLOCK NAMES . . . . . . . . . . . . . . .  9-18

CHAPTER 10      PLOT EXAMPLES AND PLOTTING SYMBOLS

10.1    PLOTTING SYMBOLS AND CHARACTER FONTS EXAMPLES  .  10-1

vi

10.2    LINE TYPE/WIDTH EXAMPLES . . . . . . . . . . . .  10-2

10.3    MASTER ROUTINE OUTPUT EXAMPLES . . . . . . . . .  10-3

10.4    3-D ROUTINE OUTPUT EXAMPLES  . . . . . . . . . .  10-4

10.5    CURSOR ROUTINE TEST PROGRAM  . . . . . . . . . .  10-5

10.6    REF OUTPUT EXAMPLE . . . . . . . . . . . . . . .  10-6

CHAPTER 11      INDEX

7

CHAPTER 1

INTRODUCTION

1.1  GENERAL

This manual was prepared to  document  the  LONGLIB  graphics

library.  This latest revision of the library has been extensively

modified though every effort has been made to make any changes  or

additions  to the library compatible with previous versions of the

LONGLIB graphics  library.   It  is  anticipated  that  additional

MASTER  subroutines will be incorporated in the future as they are

needed.

The LONGLIB graphics library is a set of FORTRAN  subroutines

for  vector  plotting  (line graphics).  The library is similar to

CALCOMP; however, a great many extensions  are  incorporated  into

the  LONGLIB  library  including viewport clipping, plot rotation,

etc.  In addition, the LONGLIB library includes  a  large  set  of

routines  for  such  things  as  3-d  plotting  (with  hidden line

removal), extended character sets, MASTER routines, graphics input

routines, map routines, etc.

The LONGLIB library is designed with three internal  packages

for  plotting  on  3  major  classes  of  output graphics devices:

graphics terminals, Ramtek display screens, and  hardcopy  devices

via   metafiles.   A  large  variety  of  graphics  terminals  are

supported.  Graphics output to each type of  device  is  discussed

below.   LONGLIB  achieves  virtual  output device independence by

using only  a  minimum  of  low-level  commands.   These  include:

initialization,  a  screen  clear/new  page command, and drawing a

vector.  In  addition,  line  types  and  color  are  utilized  if

available.   Graphics  terminal  and  Ramtek  output  also support

additional features.  Hardcopy devices which require rasterization

are also supported using the Metafile.

The library is principally designed for  vector  (i.e.   line

plotting).   Lines  are  drawn  by  specifying  the movement of an

"electronic pen".   The  color,  width,  and  pattern  of  up/down

motions  in  a line (the line type) may be specified.  The library

consists of various subroutines  which,  when  called  by  a  user

written  program,  will  produce  line  drawings  on  the  desired

1-1

INTRODUCTION

graphics device(s).  The system is interactive in the  sense  that

the user can see and modify plots immediately (on screen devices).

The library  can  also  produce  a  metafile  which  can  then  be

processed by programs supplied with the LONGLIB library to produce

hardcopy output on a variety of hardcopy devices.  Provisions have

been incorporated for device-dependent graphics input to a program

(see CURSOR routines).

The library is  designed  with  several  levels  of  graphics

routine  users in mind.  For the user who simply wants to obtain a

plot of an array of data with a  minimum  of  effort,  the  MASTER

routines   offer   a   simple  solution.   These  routines  handle

opening/closing the plot package, scaling,  and  axis  generation,

etc,  a  wide variety of formats.  For the user interested in more

elaborate plots, access to several levels of the plotting routines

are provided.

The LONGLIB graphics library supports both line type (such as

pen  width  and  dot-dash  patterns)  and  color  attributes.   In

general, these attributes are device dependent and  specific  line

types or colors used on one plotting device may appear differently

on a second  device.   Typically,  line  width  is  simulated  for

graphics  terminals  while line typing relies on hardware support.

A routine for software generation of line types (LINSEQ) has  been

provided.

1.2  MACHINE DEPENDENCE

LONGLIB was developed in a VAX/VMS environment and is written

exclusively in FORTRAN.  While some machine-specific routines have

been retained, the present version of the library has been made as

FORTRAN-77  compatible  as  practical.   Where  the  code  is  not

FORTRAN-77 compatible, it is so noted in the source  code.   These

exceptions  were  made  for efficiency's sake and can be easily be

modified or be  left  out  of  the  final  library.   The  largest

FORTRAN-77  incompatibilities  (outside  of  the  Ramtek packages)

occur when treating character string data (see notes below).

The auxilary, MASTER, and 3d plotting routines are FORTRAN-77

compatible with a few minor exceptions (e.g.  INTEGER*2 is used to

save space and some routine names  exceed  6  characters).   These

exceptions are noted in the source code file headers.  Routines in

RAMLIB and REFLIB tend to be very machine specific and may require

extensive   modification   for  use  on  other  machines.   Unless

specificially stated, integers are all assumed to be the  standard

default  machine  length.   On  the  VAX this is INTEGER*4.  A 2's

complement representation is assumed for  negative  integers  when

specifying  the  line  type  bit  pattern  (various  routines  and

programs) and in storing the  pen  motion  data  in  the  routines

SYMBOL, SYM3D, and SYM3DH.

1-2

INTRODUCTION

Some machine dependent constructs are used in the library for

detection  of  missing  (and/or  default)  parameters  (mainly  in

SYMBOL, NUMBER, and AXIS ROUTINES).  These can easily  be  modifed

and/or adapted for other machines.

Note that when  a  subroutine  is  called  with  a  Hollerith

constant,  CALL  SUB('string'), VAX FORTRAN passes the string as a

BYTE array of ascii values.  (see the VAX FORTRAN  User's  Guide).

CHARACTER variables are passed using a different mechanism and can

not be substituted.  If you wish to use a CHARACTER variable,  use

the       %REF()       function.        i.e.,       use       CALL

SUB(%REF(CHARACTER_VARIABLE)).

The VAX extended FORTRAN data  type  "BYTE"  (for  an  8  bit

variable)  is  used  in  the  REF and Ramtek library package.  The

terminal  output  is  via  a  WRITE(*,<format  statement  number>)

command.   The cursor routines CURMOTION, CURRECT, CURBAND use the

machine dependent routine INXTCHR to read  escape  sequences  from

the terminal.  Note, however, that the library can be used without

these cursor routines.

A VAX routine %LOC() is used in MVAX5D and VAX5D to determine

the  address  of  an  array.   These  routines  may be modified or

deleted as desired.

The  library  contains  files  for   VAX   environment   help

libraries, etc.  These files can be deleted if not used.

1.3  USING LONGLIB WITH GRAPHICS TERMINALS

One of three internal, separate packages used by  LONGLIB  is

the  terminal  screen  graphics  package.   The  LONGLIB  graphics

library supports a variety of graphics terminal devices which  use

Tektronix graphics formats including:

1.  VT100 equipped with a Selanar GR100  or  GR100+  (Selanar

Corporation, Santa Clara, Ca.)

2.  VT125 (VT100 + retrographics)

3.  VT240 and compatibles

4.  VT220 equipped with a Selanar SG220

5.  Tektronix 4010/4014 and compatibles

6.  Tektronix 4107/4109 and compatibles

7.  Graphon GO-235

Other graphics terminals can be used by  appropriate  modification

of the terminal driver routines (CTERM, VTPLOT, NEWVPEN, NEWVCOL).

The routines FRAME and VPLOTS  will  also  need  to  be  modified.

Escape   sequences  need  for  terminal  initialization  and  mode

switching are handled in the subroutine CTERM.  When  plotting  is

not  done  to  the  terminal  CTERM  and  other  terminal specific

1-3

INTRODUCTION

routines are a dummy call.  Note:  plotting  to  the  terminal  is

done  by  accumulating a set of connected line segments created by

PLOT or PLOTVT.  The line segments are  output  to  terminal  only

when  the  buffer  length  of 32 is exceeded or a "pen up" occurs.

Consequently, a "pen up" should be issued to force all stored plot

segments to be output to the terminal prior to viewing screen.

It is assumed that all terminals are configured in a standard

fashion.   Because  of  the  wide  variety  of terminal types, the

specific settings will not be given in this document.

In  designing  the  terminal  driver  routines,  the   design

philosophy  has been to take advantage of the capability that many

graphics  terminals  have  to  display  both  text  and   graphics

independently  on  the  screen.   That  is,  the text and graphics

"screens" can be independently  cleared,  etc.   This  feature  is

heavily  exploited  in  this library.  Of course not all terminals

support this feature.  Terminals such as the VT240  and  Tektronix

4010's  do  not  support  this  feature.   In this case, the CTERM

routine operates somewhat differently in that it sets the terminal

to graphics mode and leaves it there.

Note that in the VAX environment, the SET TERM options of the

terminal  driver  must  be  properly configured.  For example, you

must execute a SET TERM/FORM while in DCL  to  enable  the  screen

clear  command to work properly.  This permits a form feed to pass

through to the terminal.  The NO ESCAPE qualifier should  also  be

set to read from the terminal.

A command file, LONGLOC:CLEAR.COM, has been  provided  which,

when  executed,  will  clear the screen of a VT100 equipped with a

Selanar card and set the VT100 into text mode.  A simliar  routine

for  a  VT125  (TCLEAR.COM)  and  a  VT220  (VCLEAR.COM)  is  also

provided.  These reset the graphics mode back to terminal mode and

clear   the   screens.    The  are  very  helpful  during  program

development.

LONGLIB also  supports  color  on  graphics  terminals  which

permit color plotting.

1.3.1  Terminal Specific Application Notes

The following sections describe the operation of the  LONGLIB

graphics  routines  with  some  specific  graphics terminal types.

This list is not comprehensive.

1-4

INTRODUCTION

1.3.1.1  VT100 (w/Selanar Graphics Cards)

A reference to a VT100 indicates  a  VT100  equipped  with  a

Selanar equipped (GR100, or GR100+) VT100.  A SG100 equipped VT100

has three modes:  the terminal (VT100) mode, the Selanar  terminal

mode, and the Selanar graphics mode.  The routine CTERM is used to

switch the terminal between modes.  A call to FRAME will  set  the

mode  to the Selanar terminal mode.  Whenever a line is plotted to

the screen the mode is change to Selanar graphics and then back to

the  Selanar  terminal  mode.   If a program error occurs when the

terminal is in the Selanar graphics mode, it is  possible  to  not

see  any  output.   Running  the CLEAR command file will reset the

mode  back  to  terminal.   The  Selanar  card  does  not  provide

linetypes or XOR capability in the 4010 operation mode.

1.3.1.2  VT125

A VT100 upgraded with the DEC upgrage package to a VT125  has

two  modes used by the LONGLIB library:  the normal terminal mode,

and the Tektronix 4010 graphics mode.  Both text and graphics  can

be  displayed  on the same screen but independently manipulated by

swithing between modes.  Running  the  TCLEAR  command  file  will

reset the mode back to terminal.

1.3.1.3  VT240

Unlike other  terminals,  the  VT240  when  switched  between

terminal  modes  clears  the screen.  Thus CTERM sets the terminal

mode to  the  Tektronix  4010  mode  when  terminal  graphics  are

initiated.   The  mode  is  not  changed.   Limited  linetypes are

supported though XOR plot mode is not.

1.3.1.4  VT220 (w/Selanar Graphics Card)

Reference to a VT220 terminal is intended to indicate a VT220

terminal  equipped  with  a  Selanar SG220 graphics upgrade board.

The SG220 board operates through the  VT220's  printer  port.   In

addition,  it  uses some of the keys on the VT220 keyboard.  It is

possible to enable the graphics board when  the  printer  port  is

disabled.   This  causes  some of the keys (notably the keypad and

cursor keys) to not work  for  editing,  etc.   To  correct  this,

deselect the graphics board using CNTRL-SELECT.

In order for the graphics board  to  plot  graphics  commands

from  the  library  routines,  the  printer  port  must  be in the

controller mode and the board enabled.  All this is  automatically

handled  by the LONGLIB graphics library routines.  Note, however,

1-5

INTRODUCTION

that you will need to use the CTERM commands correctly  to  obtain

proper operation.

The DCL command file VCLEAR will reset  printer  port,  clear

the graphics card and terminal the screens under most situations.

When using GETCURSOR, a large cross-hair will appear  on  the

screen.   Use  the  cursor  keys  and  the  shift  key to move the

intersection point to the desired location and press a key.   This

ends  the  graphics  input.   The cross-hair will disappear, there

will be a pause, and the program will continue.  NOTE:  DON'T  USE

THE  RETURN  KEY  TO  END  GRAPHICS  INPUT  AS THIS WILL CAUSE THE

PROGRAM TO NOT READ THE  LOCATION  AND  MESS  UP  YOUR  TYPE-AHEAD

BUFFER.

The SG220 board stores the starting and ending points of each

vector  in  memory  then  dynamically  redraws the screen for each

refresh (this is how it can do zooming and  windowing).   However,

this  also  limits  the number of vectors that can be drawn on the

screen at once.  If a very large number of vectors are plotted,  a

zoom in/out may cause a portion of them to not reappear.

Note:  Since the  actual  SG220  screen  resolution  is  only

832x350  and  the  board is emulating a Tektronix 4014 (4096x3200)

there is some quantization error associated  with  the  cross-hair

operation.

Note:  When the  manual  zoom  in/out  feature  is  used  the

cross-hair  cursor  does  not  work  properly.   The cross-hair is

initially plotted then erased.  When you move the  cross-hair,  it

is  replotted.   A  copy  of  the  cross-hair  stays  at the start

location.  Otherwise, it is useable.

1.3.1.5  Tektronix 4010/4014 Compatible Terminals

A generic Tektronix 4010/4014 compatible terminal is  defined

in  LONGLIB.   Linetypes  are  supported.   Since  most  Tektronix

4010/4014 compatible terminals do not support  dual  text/graphics

screens, this feature is unavailable.

1.3.1.6  Tektronix 4107/4109 Compatible Terminals

The Tektronix 4107/4109 terminals support a  dual  (text  and

graphics)  screen mode which is exploited by LONGLIB.  Most of the

advanced features available on these terminals are not  exploited.

Color  and  linetypes  are  supported.   The  command  file TEKCLR

(executed from DCL) can be used to clear the screen and reset  the

terminal into the ANSI mode.

1-6

INTRODUCTION

1.3.1.7  Graphon Terminals

The  Graphon  GO-235  is  a  terminal   compatible   with   a

VT100/VT200 and Tektronix 4014.  It permits both text and graphics

on the same screen in separate planes  when  properly  configured.

This  configuration  is  assumed  by the LONGLIB graphics library.

The Graphon terminal supports linetypes, graphics input,  and  XOR

plot  mode.  A command file GCLEAR (executed from DCL) is provided

to clear the screen and reset the text mode.

1.4  THE LONGLIB METAFILE (HARDCOPY OUTPUT)

The second internal package used by LONGLIB is  the  metafile

or  printer  history  file  package.   The  library can optionally

produce a plot metafile  containing  scaled,  clipped  pen  motion

commands.   This "history" file can be processed by other programs

to produce hardcopy output on the appropriate device.   Using  the

REPLOT  program,  the  plot  history  file can be redisplayed on a

screen device.  Raster scan converter programs produce a dot-image

printable  file  from  the  metafile  for printing to a dot matrix

printer.  Other metafile processing programs convert  the  LONGLIB

metafile   to   other  graphics  description  languages  including

POSTSCRIPT, HPGL, QUIC, etc.

Many of  the  LONGLIB  metafile  processing  programs  permit

"striping"  of  the  graphics  image.   When  the  graphics  image

contained in the LONGLIB metafile exceeds the  size  of  a  single

page (or whatever) of the output device, the the metafile image is

"cut" into "strips" which fit on the output page.  Then each  page

is  output.   Normally,  blank  pages are suppressed.  At the same

time redundant pen motions, changes, etc.  are filtered out.

1.4.1  Dot Matrix Printers

Dot matrix printers, in general, require graphics data  in  a

bit-image  image  format.   This  requires  converting the LONGLIB

metafile pen motion file into a bit-image file using a raster scan

process.    Using  one  of  the  supplied  raster  scan  converter

programs, the LONGLIB metafile  can  be  processed  to  produce  a

printable  file  of  graphics  for  several  types  of  dot matrix

printers.  The raster scan converter programs  supports  linetypes

and  widths.   Currently,  raster  scan  converter  programs which

including "stripping" exist for the following printers:

1.  Printronix

2.  Trilog TIP-300

3.  DEC LA50 (or compatible)

1-7

INTRODUCTION

The raster scan converter programs using "stripping"  with  a

strip  size which depends on the printer.  Ordinarily the strip is

56.5 by 13.2 inches (or  the  width  of  the  printer  page).   To

generate  a  raster  scan data file suitable for printing on a dot

matrix printer from the vector plot command file, the raster  scan

converter  program  must  be used.  Versions of this program exist

for LA50 printers (LA50), Printronix printers (PRNTRX), and Trilog

Printers (TRILGLO or TRILGHI--see below).

Two raster scan programs have been provided  for  the  Trilog

printer  to  take  advantage  of  its  higher printing resolution.

TRILGLO prints the same resolution on the Trilog printer as on the

Printronix   printer.    TRILGHI   plots   at   almost  twice  the

across-the-page resolution and the same down-the-page  resolution.

Execution time is longer for the high resolution program.

An example of the use of  the  TRILGLO  program  in  the  VAX

environment  follows.   The  LONGLIB metafile input is FOR003.DAT.

The output is OUT.LIS.

$ TRILGLO :== $LONGLOC:TRILGLO

$ TRILGLO FOR003.dat            ! run raster scan converter

$ print/que=Trilog OUT.LIS      ! print output

VAX DCL command files in the  directory  pointed  to  by  the

logical  name  LONGLOC:   contain  command  files for execution of

these commands.  (PLOT183.COM = printronix printer,  PLOTLO.COM  =

trilog printer lo-res PLOTHI.COM = trilog printer hi-res).

1.4.1.1  Subprocesses

Two subroutines have been provided to spawn a subprocess from

a FORTRAN program (in a VAX VMS environment) to execute the raster

scan converter programs.

1.4.1.1.1  SUBROUTINE PSUBPRO

CALL PSUBPRO(i,opt,ip)

i   (I): raster scan conversion program type

= 0 printronix printer

= 1 low res trilog printer

= 2 hi res trilog printer

opt (I): concurrent process option

= 0 wait for completion

= 1 no wait

ip  (I): print flag

= 0 do not print

1-8

INTRODUCTION

= 1 print

PSUBPRO creates a subprocess using SUBPROC to run the raster  scan

conversion  program.   PSUBPRO should be used with FOR003.dat file

after the plot package has been closed.   The  output  is  OUT.LIS

which  can  be  printed  if  desired.   The command files used are

located in LONGLOC:.

1.4.1.1.2  SUBROUTINE SUBPROC

CALL SUBPROC(in,out,i)

in   (C) : input command file name (CHARACTER variable)

out  (C) : output log file (CHARACTER variable)

i    (I) : concurrent processing option

= 0 wait for completion

= 1 no wait

SUBPROC creates a subprocess which executes the command  file

IN.   The  output  is  places  in the file OUT (if OUT is NL:  the

output is discarded).

1.4.2  HPGL Compatible Pen Plotter

Metafile  conversion  programs  which  convert  the   LONGLIB

metafile  into an HPGL (Hewlett-Packard graphics language) command

data file are included in LONGLIB.  Three programs are  available,

HPGL, HPGL2, and HPGLS.

HPGL  reads  the  LONGLIB   metafile,   processes   it   into

appropriate  commands  and  sends  the  commands  to  the terminal

printer port (either a VT100 or VT220  compatible  terminal  or  a

VT100  equipped  with  a Selanar GR100 graphics board) to which is

connected an HPGL-compatible pen plotter.  No "stripping"  of  the

image  is  done.   The  user  is  prompted  for page changes.  Pen

changes on the plotter occur in response to a color change in  the

metafile.  Line types are supported but line widths are ignored.

HPGL2 reads the LONGLIB  metafile  and  produces  a  separate

output  file  of  HPGL  commands for each page of LONGLIB metafile

input.  HPGL2 does not include "stripping" of  the  image.   HPGL2

plots  all  of  the vectors of a given color before changing pens.

Line types are supported but line widths are ignored.

HPGLS is similar to  HPGL2  but  includes  stripping  of  the

image.  HPGL2 produces a separate output file of HPGL commands for

each strip and page of LONGLIB metafile input.

1-9

INTRODUCTION

An example of the  use  of  the  HPGL2  program  in  the  VAX

environment  follows.   The  LONGLIB metafile input is FOR003.DAT.

The output is a file,  OUT.LIS  which  can  then  be  sent  to  an

HPGL-compatible plotter.

$ HPGL2 :== $LONGLOC:HPGL2

$ HPGL2 FOR003.dat            ! run HPGL conversion program

! several out.lis files may be produced

$ print/que=HPGL out.lis;*    ! send output file(s) to plotter

1.4.3  QMS QUIC Laser Printer

Three programs, LASER, LASERS, and RLASER,  are  included  to

convert the LONGLIB metafile into a printable file in the QMS QUIC

laser printer control language.  Line width  and  line  types  are

supported though color is not.  Programs LASERS and RLASER "strip"

the metafile into 8.5 by 11 page strips while LASER  does  not  do

stripping.    LASER   and  LASERS  produce  a  full  size,  normal

orientation output, while RLASER scales  the  output  by  3/4  and

rotates the page -90 deg.

An example of the  use  of  the  LASER  program  in  the  VAX

environment  follows.   The  LONGLIB metafile input is FOR003.DAT.

The output is OUT.LIS.

$ QMS :== $LONGLOC:LASER

$ QMS FOR003.dat               ! run QMS metafile conversion

$ print/que=QMSLASER OUT.LIS   ! print output to printer

1.4.4  PostScript

A program titled POSTSCRIPT is included  which  will  convert

the  LONGLIB  meta  file  into  a PostScript page language format.

This can then be printed to PostScript compatible printer (such as

an  AppleWriter).   No "strips" are used.  Linetypes and width are

supported.  Input color changes are output as grey tones.

An example of the use of the POSTSCRIPT program  in  the  VAX

environment  follows.   The  LONGLIB metafile input is FOR003.DAT.

The output is OUT.LIS.

$ POST :== $LONGLOC:LASER

$ POST FOR003.dat                ! run POSTSCRIPT conversion

$ print/que=POSTSCRIPT OUT.LIS   ! print output to printer

1-10

INTRODUCTION

1.4.5  Plotting The LONGLIB Metafile To A Screen Device

A program REPLOT  has  been  provided  which  will  read  the

LONGLIB  metafile  created by an eariler program and plot the file

to a screen device (terminal or Ramtek).  This program may be  run

from DCL in the VAX environment by:

$ REPLOT :== $LONGLOC:REPLOT

$ REPLOT <printer graphics file name>

Note that the default meta file name is FOR003.DAT.

1.5  PLOTTING ON THE RAMTEK SCREEN GRAPHICS DEVICE

The third internal package (which will not  be  available  in

some  installations)  used by LONGLIB is the Ramtek Screen display

package.  The plotting device is the Ramtek  family  of  displays.

Currently,  only some (but not all) of the functions of the Ramtek

9460 (1280x1024) or  9050  (512x512)  Ramtek  screen  display  are

supported.   The  Ramtek  screen  display  can  be replaced by the

Ramtek Emulation File (REF) package  which  simulates  the  Ramtek

display  as  an  internal image array.  The LONGLIB library can be

configured without the Ramtek package.

The Ramtek package is initialized by a call to  FRAME  (which

calls  RPLOTS).   When the Ramtek package is not being used, calls

to Ramtek routines are dummy calls.  Note:  plotting to the Ramtek

is  done  by  accumulating  a  sequence of connected line segments

(defined by PLOT  or  PLOTRM)  in  a  storage  buffer.   The  line

segments  are  output to the Ramtek only when the buffer length of

128 is exceeded or a "pen up" occurs.  Consequently, a final  "pen

up"  should  be issued to make all plot segments are output to the

display prior to viewing screen.

The current version of the software drivers  for  the  Ramtek

display  are machine specific to VAX/VMS.  The REF subpackage uses

some VAX FORTRAN specific constructs (for  efficiency)  but  could

easily be modified for other machines.

Ramtek display drivers  expects  the  logical  name  "RM"  be

assigned to Ramtek device (example:  assign RMA0:  RM:).  When the

Ramtek device supports multiple diplays, interaction  between  the

displays  complicates  matters.   To  distiguish between different

displays on the same device it is suggested that the 0th device be

named  xxx0:, second device xxx1:, etc.  Color plotting is done on

the Ramtek using the previously loaded color table.   The  LONGLIB

"color"  is the color table index.  The default Ramtek color table

index is 255.  The main LONGLIB package uses only vector plotting.

However, auxilary routines have been included which permit writing

image mode data to  the  Ramtek  (or  REF)  and  for  color  table

manipulation (see RMWRITEWORD or RMWRITECOL).

1-11

INTRODUCTION

The  Ramtek  supports  line  types  and  scale  factors   but

simulates line widths.

1.5.1  Ramtek Emulation File Plotting

When a Ramtek  display  is  not  available  or  for  off-line

plotting  to  a "simulated" Ramtek device, a set of routines known

as the Ramtek Emulation File (REF) routines can  be  used.   These

routines  replace the Ramtek communications routines (RAMLIB) in a

version of the LONGLIB graphics library named LONGLIBR.OLB.   This

software emulates many (but not all) of the important functions of

the Ramtek communication routines to a 9460 or 9050 Ramtek display

using an internal byte array.

1.5.1.1  Using The REF Routines

To use the REF routines, link to the LONGLIBR version of  the

LONGLIB  library, and plot to the Ramtek device.  The REF routines

use a 1280x1024 BYTE array as the simulated Ramtek display.   Each

byte  of  the array stores the color table index for each pixel of

the simulated Ramtek display.  An empty "screen" consists  of  all

zeros.   When  a line is drawn to the display using PLOT or PLOTRM

the appropriate pixel bytes  are  set  to  the  line  color.   REF

routines  also  permit image mode writing/reading of horizontal or

vertical lines of bytes (see RMWRITEWORD, for example).  When  the

plot  package  is  closed  (by  a call to PLOTND) the user will be

prompted to output the internal BYTE array to a specific device (a

graphics  terminal,  a metafile, or a special REF file).  To avoid

the  interative  prompt,  the  user  can  call  REFDIS  (with  its

appropriate  arguments)  prior  to  the  PLOTND  call.   This will

disable the prompts.  See REFDIS for additional details.

Note:  the Ramtek color table routines  and  cursor  routines

are dummy calls when using the REF routine library.

1.5.1.2  REF File Output

The REF file consists of a direct access file with  a  record

length  of  one  horizontal line of pixels.  The number of records

and record size depends on the type of Ramtek  chosen  (RECL=1280,

1024  records  for  the large Ramtek and RECL=512, 512 records for

the small Ramtek).  REFDIS places  each  horizontal  line  of  the

output  array  into each record of the REF file.  The REF file can

subsequently be read by the REFTERM program  and  plotted  to  the

"real"  Ramtek  device.   To  produce a hardcopy output of the REF

file on a QMS laser printer, the program REFLAS reads the file and

produces  a simulated gray scale image with 4 grey levels.  REFLAS

1-12

INTRODUCTION

prompts the user for  the  appropriate  inputs.   In  the  VAX/VMS

environment,  the  program  REFLAS2 can be used.  REFLAS2 provides

significantly faster run times by using the VAX paging utility and

downloadable fonts.

1.5.1.3  REF Terminal Or Metafile Output

When outputing the byte array data to a terminal  or  printer

history  file,  the  array  is converted to a series of horizontal

vectors.  As the array scanned left to right, top to  bottom,  all

ajacent  pixels  of  the  same  color  are  combined into a single

vector.  When a pixel of a  different  color  is  encountered  the

vector  is  output.   Pixels  of  color  "0"  (background) are not

output.  See REFDIS for additional details.

1.6  LIBRARY LOCATION

All LONGLIB libraries, source files, and support programs are

located  in  the  same  directory.   In  the  VAX environment, The

logical name LONGLOC:  should be assigned to  be  this  directory,

i.e.,

$ assign disk:[directory] LONGLOC:

The LONGLIB object library is self contained.   In  order  to

link to the normal library use the command (see also the shareable

image library section below),

$ link <your_prog>,LONGLOC:LONGLIB/LIB

If your installation has a Ramtek display, this link command  will

include  the  Ramtek  display routines.  If your installation does

not have a Ramtek display, this link  command  will  include  only

dummy  routines  for Ramtek routines.  In order to link to the REF

version  the  library  (which  is  normally  available  on   every

installation) use the LONGLIBR.OLB object library,

$ link <your_prog>,LONGLOC:LONGLIBR/LIB

1.6.1  LONGLIB Source Code

The source code for LONGLIB is all in FORTRAN and is included

in  the  distribution  package.   Source  code may be found in the

following files:

LONGLIB.FOR (device dependent routines)

AUXLIB.FOR (auxilary device independent routines)

1-13

INTRODUCTION

MLIB.FOR (MASTER routines)

MLIB2.FOR (more MASTER routines)

HIDELIB.FOR (3-d hidden line routines)

LONGLIB3D.FOR (3-d w/no hidden lines)

RAMLIB.FOR (Ramtek communication code)

MAPLIB.FOR (map routines)

CURSORLIB.FOR (cursor routines)

REFLIB.FOR (Ramtek emulation file routines)

etc.

In addition, the LONGLIB support program such as the  example

programs,  metafile  processing  programs,  etc.,  are  located in

LONGLOC:.

1.7  USER NOTES -- VAX/VMS ENVIRONMENT

In the VAX/VMS environment LONGLIB provides an  on-line  help

library  accessable from either a user program or DCL as well as a

shareable image library.

1.7.1  Using The LONGLIB VAX/VMS On-line HELP Library

In  the  VAX/VMS  environment  an  on-line  help  library  is

available  in  addition  to  this documentation.  The On-line help

library can be accessed from VAX DCL by,

$ HELP @LONGLIB

or

$ HELP @LONGLOC:LONGLIB.HLB

A subroutine for accessing the library is  also  included  in  the

LONGLIB package HELPME.  From a user program calling HELPME with a

CHARACTER string corresponding to the  initial  help  string  will

call  the  VAX  librarian  for  an  interactive search through the

LONGLIB help library.

1.7.2  Using The LONGLIB VAX/VMS Shareable Image Library

A shareable image library for the VAX/VMS may be created  for

the  LONGLIB  graphics library.  To obtain the most advantage from

using the shareable image library, the library must be "installed"

by  the  system  manager.   Not  all  installations  will  have an

installed LONGLIB library.  Linking to the shareable image library

(LONGLIB.EXE  rather  than  LONGLIB.OLB) produces executable files

which are small and run faster.  The main  difference  is  in  how

they are linked.  To link to the "normal" library,

1-14

INTRODUCTION

$ LINK your_file_name.OBJ,LONGLIB/LIB

To link to the shareable image library you have to  use  a  linker

option file:

$ LINK your_file_name.OPT/OPT

where your_file_name.OPT contains:

your_file_name.obj,LONGLIB/SHARE

or you may use the longlib linker option file:

$ LINK your_file_name,LONGLIB/OPT

This second option for the linker option file is  the  easiest  to

use.   Note  that the LONGLIB/OPT should be last on the link list.

On most installations, the REF version of the  library,  LONGLIBR,

is not installed as a shareable image library.

Although the linking is somewhat more complex, the  shareable

image  file  does  allow program using the graphics library to run

slightly faster and the executeable to be significantly smaller.

The shareable  image  library  is  designed  to  be  upwardly

compatible.

1.8  DISCLAIMER

LONGLIB is very powerful graphics package for  line-graphics.

Unfortunately,  the desire to maintain compatibility with earilier

LONGLIB  versions  has  resulted  in  less  than  optimal   design

decisions.    There  are,  however,  some  incompatibilities  with

earlier versions of LONGLIB; notably the color array used in AXIS,

AXIS2,  and  AXIS,  the  working space specifications of PLT3D and

NXTVU, and metafile internal format which results in the fact that

previous  metafile  processing  programs  will  not  work with new

metafiles and vice versa.

LONGLIB was developed in the VAX VMS environment.   With  the

possible  exception  of the Ramtek package routines, every attempt

has  been  to  keep  LONGLIB  FORTRAN-77  compatible  and  machine

independent  with  the  exceptions  previously noted.  No claim is

made  regarding  the  transportability  of  LONGLIB   to   non-VAX

machines.   It  is  felt,  however,  that only small modifications

would be required.

As with any software system, there may be bugs in either  the

documentation  or  software though every attempt to eliminate them

has been made.

1-15

CHAPTER 2

PROGRAMMING EXAMPLES

In this chapter several examples of using some of  the  basic

plotting   routines   of  this  library  are  included.   The  VAX

environment permits the linking of object code  segments  compiled

from  different  languages.  Thus, examples for both FORTRAN and C

routines are shown.

2.1  FORTRAN PROGRAMMING EXAMPLES

For the user interested in obtaining  very  simple  plots  of

curves  without  being  concerned with the details of LONGLIB, the

MASTER routines provide the most simple approach.  Greater control

of  the output plot can be obtained by customizing user version of

MASTER-like routines from the auxilary routines.

2.1.1  Simple MASTER Routine Example

A very simple example of using the MASTER  routines  of  this

library  is  shown  below.   For  more  detailed  examples see the

following sections.

This example plots a damped sine wave on  both  the  terminal

screen,  the  Ramtek  (in  color),  and the LONGLIB metafile.  The

master routine  PLOTSC  handles  all  graphics  intialization  and

closing.   (See  the  chapter  on  MASTER  routines for additional

details).

2-1

PROGRAMMING EXAMPLES

DIMENSION X(100),Y(100)

DATA P,U,PL,A,B,PHI/112,85,25.,.013,.3/

DATA PI/3.141593/

C  FILL DATA ARRAYS

DO 10 I=1,100

X(I)=I-1

Y(I)=SIN((I-1)*PI/A+PHI)*EXP(-I*B)

10    CONTINUE

C CALL LONGLIB PLOTTING ROUTINE TO PLOT CURVE.

C OUTPUT ONLY 35 OF THE 100 POINTS ON A 6X5 PLOT WITH TITLE

C AND A GRID.  iflag IS SET TO PROMPT FOR A SCREEN OUTPUT DEVICE.

C NOTE THAT WHEN A MASTER ROUTINE INITIALIZES LONGLIB, A

C METAFILE IS ALWAYS PRODUCED.

CALL PLOTSC(X,Y,100,5,6.,5.,'X TITLE',7,'Y TITLE',7,

1        'TOP TITLE',-10)

STOP

END

2.1.2  Lower Level Routines

An example of using the auxilary routines of LONGLIB  library

from  FORTRAN  is  illustrated  below.   An example for using this

library from C follows.

This example plots a damped sine wave on  both  the  terminal

screen,  the  Ramtek  (in  color),  and  the LONGLIB metafile.  No

MASTER routines are used.

PROGRAM DEMO

DIMENSION X(100),Y(100),ICOL(3)

DATA P,U,A,B,PHI/112,85,25.,.013,.3/

C ICOL IS THE COLOR ARRAY FOR AXES

DATA PI/3.141593/,ICOL/1,2,3,4/

C FILL DATA ARRAYS WITH Y=F(X)

DO 10 I=1,100

X(I)=I-1

Y(I)=SIN((I-1)*PI/A+PHI)*EXP(-I*B)

10  CONTINUE

C INITIALIZE LONGLIB WITH SCREEN PROMPT OPTION

C AND CREATE METAFILE TO FORTRAN UNIT 3.

CALL FRAME(3,0,2.,2.,1.)

C COMPUTE SCALING FACTORS FOR X AND Y

CALL SCALE(X,8.,100,1,1,XMIN,DX)

CALL SCALE(Y,6.,100,1,1,YMIN,DY)

Y0=-YMIN/DY

C PLOT COORDINATE AXISES WITH COLOR OPTION ENABLED

CALL AXIS(0.,Y0,'X-AXIS',-6-100000,20.,0.,

1  XMIN,DX,N1,N2,ICOL)

CALL AXIS(0.,0.,'SINE',4+100000,17.,90.,

2-2

PROGRAMMING EXAMPLES

1  YMIN,DY,N1,N2,ICOL)

C SET LINE COLOR

CALL PLOT(5.,0.,0)

C PLOT DATA POINTS AS A LINE WITH SYMBOLS

CALL LINE(X,Y,100,1,5,2,1,1,XMIN,DX,YMIN,DY)

C PICK UP PEN AT END OF LINE (FORCES OUTPUT TO SCREENS)

CALL PLOT(0.,0.,3)

C PROMPT FOR SCREEN CLEAR ON RAMTEK/TERMINAL

CALL CTERM(2)

CALL RTERM(2)

C CLOSE LONGLIB

CALL PLOTND

STOP

END

2.2  C LANGUAGE PROGRAMMING EXAMPLE

The following is an illustration of how to use lower level LONGLIB

routines to produce a plot from VAX C.

/* Example in VAX C */

main

{

real x[100],y[100],dx,xmin,dy,ymin;

real exp(),sin(),y0;

int icol[4] = 1 , 2, 3, 4;

int i,n1,n2;

real pi = 3.141592654;

real p = 112., u = 85., a = 25., b = .013, phi = .3;

for (i = 1; i < 100 ; i++) {

x[i] = i - 1;

y[i] = sin( (i-1) * pi / a + phi) * exp(-i * b);

}

frame(&3,&-3,&0.,&0.,&1.);          /* Initialize */

scale(x,&8.,&100,&1,&1,&xmin,&dx);

scale(y,&6.,&100,&1,&1,&ymin,&dy);

y0 = -ymin / dy;

plot(&2.,&2.,&-3);                  /* new origin */

axis(&0.,&y0,"X-AXIS",&-6-100000,&20.,&0.,&xmin,&dx,&n1,&n2,icol);

axis(&0.,&0.,"SINE",&4+100000,&17.,&90.&ymin,&dy,&n1,&n2,icol);

plot(&4.,&0.,&0);                   /* new color */

line(x,y,&100,&1,&5,&2,&1,&1,&xmin,&dx,&ymin,&dy);

plot(&0.,3&0.,&3)                   /* pen up */

cterm(&2);                          /* ask if screen clear */

plotnd;                             /* terminate plotting */

}

2-3

PROGRAMMING EXAMPLES

2.3  MASTER ROUTINE EXAMPLES (FORTRAN)

The subsections that follow illustrate additional examples of

how to use MASTER routines from FORTAN.

2.3.1  Adding Additional Annotation To A MASTER Routine Plot

The following is an example of using a MASTER subroutine more

than  once  and/or adding additional text or another plotting line

and/or two MASTER subroutines:  (see also the program PLOTTESTS)

...

C INCLUDE PLOTSC ROUTINE COMMON BLOCK WHICH RETURNS

C SCALE FACTORS USED IN PLOTTING

COMMON /CPLOTSC/XMR,DXR,YMR,DYR

...

C CALL PLOTSC WITH -10000 iflag < 0 TO INITIALIZE LONGLIB

C BEFORE CALL BUT NOT CLOSE LONGLIB AFTER CALL

C SET iflag TO PROMPT FOR SCREEN DEVICE TYPE WITH TICKED GRID

CALL PLOTSC(X,Y,25,4,8.,6.,'X AXIS',6,

1    'Y AXIS',6,'TITLE',-5,ICOL)

C PUT GRAPHICS TERMINAL IN GRAPHICS MODE

CALL CTERM(-1)

C PLOT ADDITIONAL TEXT AFTER PLOT TITLE

CALL SYMBOL(999.,999.,0.15,' TEXT',0.,5,-1)

C PLOT ADDITIONAL ANNOTATION ABOVE PLOT

CALL SYMBOL(0.0,6.5,0.15,'NUMBER=',0.,7,-1)

C ADD NUMBER AFTER ANNOTATION WITH 3 DIGITS AFTER DECIMAL PT.

CALL NUMBER(999.0,999.0,0.15,3.1415,0.,3,-1)

C CHANGE LINE TYPE TO DOTTED

CALL NEWPEN(1)

C ADD ANOTHER LINE OF DATA ON PLOT USING PLOTSC SCALE FACTORS

CALL LINE(X,Y2,N,1,0,0,1,1,XMR,DXR,YMR,DYR)

...

C ASK IF SCREEN CLEAR ON TERMINAL (METAFILE NOT AFFECTED)

CALL CTERM(2)

CALL PLOTND

...

2.3.2  Using Multiple MASTER Routines In The Same Program

The following is an example of  how  to  use  several  MASTER

subroutines in the same program.  (see also the program PLOTTESTS)

The basic idea is to use the first call to  a  MASTER  routine  to

initialize the LONGLIB graphics library.  On later calls to MASTER

routines you must use "iflag"  to  instruct  the  routine  NOT  to

re-initialize  LONGLIB.   The last call to a MASTER routine closes

LONGLIB.

2-4

PROGRAMMING EXAMPLES

C FIRST CALL TO MASTER ROUTINE, SET -10000 < iflag < 0

C TO INITIALIZE LONGLIB BUT NOT CLOSE IT

C SET iflag TO PROMPT FOR SCREEN DEVICE TYPE WITH

C AXIS ON TOP AND SIDES AND NO GRID

CALL PLOTSC(X,Y,N,-1,...)

...

C SECOND CALL TO MASTER ROUTINE, SET iflag < -10000 TO

C PREVENT INITIALIZING LONGLIB OR CLOSING IT

C SET iflag TO PRODUCE AXIS TICKED GRID (SINCE

C LONGLIB OPEN, NO PROMPT FOR SCREEN DEVICE)

CALL PLOTSC(X,Y,N,-10004,...)

...

C LAST CALL TO MASTER ROUTINE, SET iflag > 10000 TO

C PREVENT INITIALIZING LONGLIB AND CLOSE IT AFTER PLOTTING

C SET iflag TO PRODUCE AXIS TICKED GRID AND LOG/LOG FORM

C (SINCE LONGLIB OPEN, NO PROMPT FOR SCREEN DEVICE)

CALL PLOTLG(X,Y,N,10053,...)

2.3.3  MASTER Routine Color Example

The following is a very elaborate example of how to use color

in a particular MASTER routine (PLOTSC).

C DIMENSION AND LOAD COLOR ARRAY

DIMENSION ICOL(6)

DATA ICOL/1,2,3,4,5,6/

...

C CALL PLOTSC WITH NUMBER OF TITLE CHARACTERS NEGATIVE TO

C USE COLOR ARRAY.  SET iflag TO INITIALIZE/CLOSE LONGLIB

C SET iflag TO PRODUCE AXIS TICKED GRID

CALL PLOTSC(X,Y,25,4,8.,6.,'X AXIS',6,

1    'Y AXIS',6,'TITLE',-5,ICOL)

...

2-5

PROGRAMMING EXAMPLES

2.3.4  Using Multiple MASTER Plots On Same Page/Screen

The following is a very elaborate example of how to to  place

several  MASTER  routine  plots  on  the  same Ramtek and terminal

screen but different metafile pages.

C OPEN LONGLIB OUTSIDE OF MASTER ROUTINE

C SELECT BOTH TERMINAL AND RAMTEK SCREEN OUTPUT WITH NO

C SCREEN CLEAR ON RAMTEK OR TERMINAL

CALL FRAME(3,-3,1.,1.,1.)

C CHANGE COLOR AND ADD A SEPARATE PLOT TITLE

CALL PLOT(7.,0.,0)

CALL SYMBOL(6.,11.5,.3,'TITLE',0.,5,-1)

C CHANGE ORIGIN AND SHRINK SUBSEQUENT PLOTS ON RAMTEK/TERMINAL

C BUT NOT METAFILE

CALL PLOT(1.,1.,-3)

CALL VFACTOR(.5)

CALL RFACTOR(.5)

C LOOP TO PLOT FOUR MASTER PLOTS ON ONE SCREEN

DO 10 I=1,4

C CALL PLOTSC WITH iflag < 10000 TO NOT INITIALIZE

C LONGLIB OR CLOSE IT AFTER PLOT

C SET iflag TO PRODUCE AXIS TICKED GRID

C (SINCE LONGLIB OPEN, NO PROMPT FOR SCREEN DEVICE)

CALL PLOTSC(X,Y,25,-10004,8.,6.,'X AXIS',6,

1    'Y AXIS',6,'TITLE',5)

C MOVE ONLY RAMTEK AND TERMINAL ORIGINS BUT NOT METAFILE

CALL PLOTRM(0.,2.*5.,-3)

CALL PLOTVT(0.,2.*5.,-3)

IF (I.EQ.2) CALL PLOTRM(11.,-20.,-3)

IF (I.EQ.2) CALL PLOTVT(11.,-20.,-3)

C NEWPAGE ON METAFILE (DOES NOT AFFECT TERMINAL/RAMTEK)

CALL NEWPAGE

10     CONTINUE

...

C PROMPT FOR SCREEN CLEAR ON RAMTEK/TERMINAL

CALL CTERM(2)

CALL RTERM(2)

C CLOSE LONGLIB

CALL PLOTND

...

2-6

CHAPTER 3

DESCRIPTION OF PLOTTING ROUTINES

This chapter details the  central  LONGLIB  graphics  library

routines.  A brief description of each routine is shown along with

an  example  of  its  call  and  a  description  of  the  required

parameters.  Optional parameters are shown in <> brackets.

The name of each parameter is followed by a letter indicating

the  variable  type.  Unless otherwise specified, integers are the

default integer length of the machine (which is INTEGER*4  on  the

VAX).

B = BYTE array string (see notes in section 1.2)

I = INTEGER

R = REAL

C = CHARACTER

L = LOGICAL

NOTE:  In  VAX  FORTRAN,  when  a  subroutine  is  called  with  a

Hollerith  constant  such  as  'string',  CALL SUB(...  ,'string',

...), VAX FORTRAN passes the string  as  a  BYTE  array  of  ASCII

values.  A CHARACTER variable can not be substituted.  If you wish

to use a CHARACTER variable, use %REF() to convert  the  CHARACTER

variable    to    a    BYTE    array,    i.e.,    call    SUB(...,

%REF(CHARACTER_VARIABLE), ...).

Lengths are given in the standard plot units of inches.   The

routine FACTOR can be used to change the plot unit length.  Angles

are specified in degrees counter-clockwise from the x axis.

Unless  specifically  noted  all  parameters  passed  into  a

routine  via  the  call  statement  will  be used on a "read-only"

basis, i.e., they will not be modified by the routine.

3.1  SUBROUTINE ABSPLT

The routine ABSPLT changes the ABSOLUTE (versus  relative  as

in  PLOT)  rotation  angle  and  scale  factor  of  the  metafile,

3-1

DESCRIPTION OF PLOTTING ROUTINES

terminal, and  ramtek  plot  routines  to  the  values  specified.

Normally,  the  routine  PLOT should be used to change the origin,

rotation angle, or scale factor.  This routine is provided  for  a

skilled  user  to  use  in error recovery.  Note, however, that no

error checking is provided in ABSPLT.

CALL ABSPLT (x,y,a,z)

x,y  (R): coordinates of new absolute origin (in inches)

a    (R): new absolute angle CCW from horizontal in degrees

z    (R): new absolute scale factor

3.2  SUBROUTINE ARROW

ARROW plots a vector arrow at any desired point and with  any

desired angle.  The type of arrowhead drawn may be selected.

CALL ARROW (x,y,al,a,p,b)

x,y  (R): location coordinates for vector-arrow tail

al   (R): length of arrow in plot units

if al=0, only a single point at (x,y) is output

a    (R): angle from horizontal at which the arrow is to be drawn

(deg counterclockwise)

p    (R): length of arrowhead in plot units

> 0 open arrowhead    ( --> )

= 0 no arrowhead

< 0 closed arrowhead  ( -|> )

b    (R): angle between arrowhead side and arrow line (deg)

3.3  SUBROUTINE AXIS

AXIS plots a single coordinate axis with  numeric  labels  at

any  desired  location  and  angle.   The routine has to be called

separately  for  the  x  and  y  axis.   A  possible  exponent  is

determined  and placed at the end of the axis title in the form of

10**n.  In order to leave room for the labelling the  axis  should

be removed at least by 3/4 inch from the edge of the plotter page.

The length of the axis should be integer-valued.  The  (i)th  (i=0

to  ml-1) axis major tick is labeled with the value, xm+dx*i.  For

a log axis see LGAXS.

CALL AXIS (x,y,s,n,al,a,xm,dx<,nm,ml<,ic>>)

x,y  (R): location of starting point of the axis

s    (B): alpha string containing the axis title

n    (I): number of characters in the string s

> 0 : axis labelling on positive side (anti-clockwise)

< 0 : axis labelling on negative side (clockwise)

3-2

DESCRIPTION OF PLOTTING ROUTINES

(100's digit)    = 0 : coordinate line, ticks and labels drawn

= 1 : line and ticks only--no labeling

(1000's digit)   = 0 : numeric labels paralel to axis line

= 1 : numeric labels orthogonal to axis line

(10000's digit)  = 0 : additional optional parameters ignored

= 1 : additional optional parameters used

(100000's digit) = 0 : color list ignored

= 1 : color list used

al   (R): length of axis in plot units (real number, integer-valued)

> 0 : tick marks placed on same side of axis as title

= 0 : no action (return with no plotting)

< 0 : tick marks placed on opposite side of axis from title

a    (R): angle at which the coordinate axis is to be drawn

xm   (R): value of first marking on the axis

dx   (R): increment for numeric axis labels

(NOTE: the following optional parameters are needed only if

the magnitude of n is > 10000)

mn   (I): minor tick marks between labeled major ticks

if not specified, 0 is used

ml   (I): number of labeled major tick marks

if not specified, int(l) is used

ic   (I): array of color values, if color not enabled,

current color is unchanged

ic(1) : color value for axis line and ticks

ic(2) : color value for numbers on axis

ic(3) : color value for axis label

(color upon return if no exponent plotted)

ic(4) : color for auto exponent scale

(color upon return if exponent shown)

3.4  SUBROUTINE AXIS2

AXIS2 is similar to AXIS but allows additional flexibility to

draw  different  tick  sizes  and  types.   Optionally, a possible

exponent is determined and placed at the end of the axis title  in

the  form  of 10**n.  In order to leave room for the labelling the

axis should be removed at least by 3/4 inch from the edge  of  the

plotter  page.   The  length of the axis should be integer-valued.

The (i)th (i=0 to ml-1) axis major tick is labeled with the value,

xm+dx*i.   AXIS2  plots  a coordinate axis and its markings at any

desired location and angle.  For a log axis see LGAXS.

CALL AXIS2 (x,y,s,n,al,a,xm,dx<,nm,nn,ml,ts,nd,sm<,ic>>)

x,y  (R): location of starting point of the axis

s    (B): alpha string containing the axis title

n    (I): number of characters in the string

> 0 : axis labelling on positive side (anti-clockwise)

< 0 : axis labelling on negative side (clockwise)

(100's digit)    = 0 : coordinate line, ticks and labels drawn

= 1 : line and ticks only--no labeling

3-3

DESCRIPTION OF PLOTTING ROUTINES

(1000's digit)   = 0 : numeric labels paralel to axis line

= 1 : numeric labels orthogonal to axis line

(10000's digit)  = 0 : optional parameters ignored

= 1 : optional parameters used

(100000's digit) = 0 : color list ignored

= 1 : color list used

al   (R): length of axis in plot units (real number, integer-valued)

> 0 : tick marks placed on same side of axis as title

= 0 : no action (return with no plotting)

< 0 : tick marks placed on opposite side of axis from title

a    (R): angle at which the axis is to be drawn

xm   (R): value of first marking on the axis

dx   (R): increment for the axis markings

(NOTE: the following optional paramters are needed only if

the magnitude of n is > 10000)

nm   (I): number of minor tick marks between major ticks,

if not specified, 0 is used

nn   (I): nn-th minor tick is high-lited in length, if

not specified, 0 is used (none)

ml   (I): number of labeled major tick marks, if not specified

then one major tick per inch is used

< 0 use following additional optional parameters used

> 0 use following additional parameters ignored

(NOTE: the following optional paramters are needed only if

the magnitude of n is > 10000 and ml < 0)

ts   (R): character size of title and numbers, if not

specified, 0.15 is used

> 0 auto exponent scaling (x10 to power) is enabled

< 0 auto exponent scaling (x10 to power) disabled

nd   (I): number of digits to right of decimal point, if not

specified 1 is used

sm   (R): major tick length, if not specified 0.1 is used

note that minor tick length is 1/2 major tick length

ic   (I): array of color indexes for axis colors

ic(1) : color value for axis line and ticks

ic(2) : color value for numbers on axis

ic(3) : color value for axis label

(color upon return if no exponent plotted)

ic(4) : color for auto exponent scale

(color upon return if exponent shown)

3.5  SUBROUTINE AXIS3

AXIS3 plots a single axis and its  markings  at  any  desired

location and angle.  Optionally, a possible exponent is determined

and placed at the end of the axis title in the form of 10**n.  The

length  of  the  axis can take any value.  The (i)th (i=0 to ml-1)

axis major tick is labeled with the value, xm+i*(xx-xm)/le.   This

version  of  axis is more flexible than other versions and permits

specifying the axis number labeling format.  For a  log  axis  see

LGAXS.

3-4

DESCRIPTION OF PLOTTING ROUTINES

CALL AXIS3 (x,y,s,n,al,a,xm,xx,t,c,f,ic)

x,y  (R): starting location of the axis

s    (B): alpha string containing the axis title

n    (I): number of characters in the string

> 0 : axis labelling on positive side (anti-clockwise)

< 0 : axis labelling on negative side (clockwise)

(100's digit)    = 0 : coordinate line, ticks and labels drawn

= 1 : line and ticks only--no labeling

(1000's digit)   = 0 : numeric labels paralel to axis line

= 1 : numeric labels orthogonal to axis line

(100000's digit) = 0 : color list ignored

= 1 : color list used

al   (R): length of axis

> 0 : tick marks placed on same side of axis as title

= 0 : no action

< 0 : tick marks placed on opposite side of axis from title

a    (R): angle at which the axis is to be drawn

xm   (R): value of first marking on the axis

xx   (R): value of last marking on the axis

t    (R): number of tick marks

specification is coded in the form MMM.mmss where

MMM is the number of major tick marks ( MMM > 0), mm is

the number of minor tick marks between major tick marks

(100 > mm => 0), and ss is the number of subminor tick

marks between minor tick marks (100 > ss => 0).

(example 1.0102 produces I_._._i_._._I)

c    (R): size of characters

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

f    (R): axis number label format (see NUMBER)

Note: if an integer -1 is passed in place of f, AXIS3

will used f=1003.00 if autoscaling is enabled.  If an

integer 0<n<13 is passed in place of f, AXIS3 will use

f=2+n*1.01 if autoscaling is enabled.

ic   (I): array of color indexes for axis colors

ic(1) : color value for axis line and ticks

(color upon return if no labels)

ic(2) : color value for numbers on axis

ic(3) : color value for axis label

(color upon return if no exponent plotted)

ic(4) : color for auto exponent scale

(color upon return if exponent shown)

3.6  SUBROUTINE CIRCLE

CIRCLE  plots  circles,  arcs  and  spirals.   The  curve  is

approximated  by  small  straight  lines.  The radius of the curve

determines the number of line segments used.  A  solid  circle  of

radius 2.0 centered at the origin could be generated by the call,

3-5

DESCRIPTION OF PLOTTING ROUTINES

CALL CIRCLE(0.0,0.0,0.0,360.0,2.0,2.0,0.0).

while a spiral centered at (1,1) would be created by the call,

CALL CIRCLE(1.0,1.0,90.0,800.0,3.0,1.0,0.0).

CALL CIRCLE (x,y,aa,ao,ra,ro,d)

x,y  (R): coordinates for the center of the circle

aa   (R): angle in degrees of starting point of curve

ao   (R): angle of end point relative to start point

ra   (R): curve radius at starting point

ro   (R): curve radius at the end point

d    (R): =  0 : solid curve

= .5 : dashed curve (software line type)

3.7  SUBROUTINE CSHADE

The CSHADE subroutine fills in an area defined by  a  segment

of  a  circle  using  equally spaced lines at a given angle with a

specified line type.  A full circle may be used.

CALL CSHADE (x,y,r,a1,a2,s,l,d,t,w,m1,m2)

x,y   (R): location of circle center

r     (R): segment or circle radius

a1,a2 (R): segment start and stop angles (a2>=a1)

if a2-a1=360 a full circle is used otherwise

the area is a pie segment

l     (I): shade format control

= -3 : clear area and outline

= -2 : clear area

= -1 : clear outline

=  0 : no action

=  1 : draw outline

=  2 : shade area

=  3 : shade area and outline

d     (R): distance between shading lines

t     (R): angle of shading lines in degrees

w     (R): working array dimensioned at least 3*n where

n=int((a2-a1)/(180*atan(s/r)/pi)+1)

m1    (R): line type of shading

< 0 : shading done with current line type

=> 0 : new line type (see NEWPEN)

m2    (R): line type for area outline

< 0 : use prior line type

=> 0 : new line type (see NEWPEN)

3-6

DESCRIPTION OF PLOTTING ROUTINES

3.8  SUBROUTINE CTERM

CTERM is the central subroutine for controling the  state  of

the  graphics  terminal.  It is used to switch a graphics terminal

in and out of the graphics and text modes.  It  is  a  dummy  call

when not in terminal plotting mode.  Note that not all options are

available on all terminals.  Results of operation of this  routine

is  thus  terminal dependent.  In some cases the graphics and text

modes are the same while in others, the mode is switched  only  at

init/de-init.   See  the  introduction  to the documentation under

terminal types.

Note:  for CTERM(2), the user will be prompted for  a  "clear

screen".   The reply may be "Y" for yes, "N" for no (the default),

"Q" (quit), "S" (skip one), "P" (pass many), or "D"  (dump).   "Y"

and  "N"  clear  the screen as indicated and the program continues

normally.  A reply of "Q"  permanently  disables  terminal  screen

plotting until the package is reinitialized by FRAME.  A replot of

"S"  disables  screen  plotting  until  the  next  CTERM(2)   call

whereupon  the "clear screen" prompt reappears as before.  A reply

of "P" is similar to "S", but the user will be  prompted  for  the

number  of CTERM(2) calls to pass before reissuing the prompt.  In

this manner multiple terminal screen "pages" can  be  skipped.   A

reply  of  "D"  will  enable  a dump of the screen to the attached

terminal    printer    (if    supported    on    the    particular

terminal/printer).   Note  that  during a skip that any changes of

origin by PLOT, scale factors by FACTOR or VFACTOR, or  pen  color

or linetype will NOT take place.

CALL CTERM (iarg)

iarg (I): terminal operation code

= 0 : initialize graphics mode on terminal

= 1 : return terminal from graphics to text mode

=-1 : return terminal text to graphics mode

= 2 : return terminal to text mode, and prompt

user for clear screen (see note above)

=-2 : return terminal to text mode, clear graphics screen

= 3 : clear text screen, leave in text mode

=-3 : clear graphics screen, leave in terminal mode

= 4 : dump graphics screen to printer

=-4 : clear text, graphics, leave in graphics mode

= 5 : turn off graphics screen, return to text mode

=-5 : turn on graphics screen, return to graphics mode

= 6 : toggle reverse video

= 8 : de-initialize graphics terminal

3.9  SUBROUTINE DASHL

DASHL plots a  software-generated  dashed  line  through  the

coordinate points stored in x and y.  See also LINE.

3-7

DESCRIPTION OF PLOTTING ROUTINES

CALL DASHL (x,y,n,k,j,l,ix,iy,xm,dx,ym,dy)

Parameter list description same as LINE.

3.10  SUBROUTINE ELLIPSE

ELLIPSE will plot all or part of a parametric ellipse of  the

form,

(x,y) = (maj*cos(t), min*sin(t))

where  maj  and  min  are  the  major  and  minor  ellipse   axes,

respectively  and  where  t is the parametric angle specification.

Part of the ellipse can be plotted by specifying only part of  the

range of t.  The major axis center line is offset by the angle am.

The angles ast and aend define the starting and ending angles  for

t.   Note  that  aend  should be greater than ast.  The ellipse is

approximated by short, straight line  segments.   The  number  and

length of the segments depend on the size of the ellipse.

CALL ELLIPSE (x,y,maj,min,am,ast,aend)

x,y  (R): center of ellipse (halfway between foci)

maj  (R): length of semi-major axis (distance between center and

curve through one focus)

min  (R): length of minor axis (distance between center and

curve perpendicular to semi-major axis)

am   (R): angle of semi-major axis from horizontal

(deg counter-clockwise)

ast  (R): start angle of curve from semi-major axis

(deg counter-clockwise)

aend (R): end angle of curve from semi-major axis

(deg counter-clockwise) note: aend > ast

3.11  SUBROUTINE FACTOR

Positional information is usually  expressed  in  inches.   A

conversion factor can be given for other units, such as "cm".  The

new coordinate values are derived from the product "fac*x" where x

is  the  input value.  A negative value or zero resets the scaling

factor to unity.  Note that the initial factor can be specified in

FRAME.   The subroutine FACTOR calls PFACTOR, RFACTOR, and VFACTOR

which change the scale  factors  for  the  metafile,  Ramtek,  and

terminal  packages,  respectively.   These  routines can be called

separately to affect only  the  scale  factor  of  the  particular

package.

CALL FACTOR (fac)

3-8

DESCRIPTION OF PLOTTING ROUTINES

fac (R): new conversion factor for coordinate values (all packages)

<= 0 : scale factor reset to unity

>  0 : new scale factor

3.12  SUBROUTINE FRAME

FRAME  initializes  the   graphics   metafile/terminal/Ramtek

software  and  resets internal variables.  This routine is usually

the first LONGLIB routine  called  (also  see  PLOTS  which  calls

FRAME).   FRAME must be activated before plotting calls are issued

by the user program.  FRAME should normally be called only once in

a program.  MASTER routines optionally handle the FRAME call.

FRAME intializes the graphics output  device  packages.   The

FORTRAN   unit  number  used  for  the  LONGLIB  metafile  can  be

specified.  Normally, unit 3 is used.  If  unit  0  is  specified,

FRAME  will  prompt  the  user  for  a yes/no response to create a

metafile.  A negative unit number disables the  metafile  package.

All  calls  to  metafile  routines (such as PPLOT) are dummy calls

when the metafile package is disabled.

FRAME also initializes the screen graphics device.  A  screen

device  code  is  used to determine whether the Ramtek or Terminal

screen device packages are open.  If  a  negative  value  for  the

screen  device code is used the screen device is not cleared prior

to use, otherwise it is.  If the screen device code is +/-  4,  no

screen  device  package  is  used and all calls to screen specific

routines (such as PLOTRM or PLOTVT) are dummy calls.  If a  screen

device code of 0 is used, the user is prompted for a screen device

to use.  The screen devices may then be selected using a character

code.  A reply of "?" will list the available options.

FRAME  also  intializes  the  origin  and  scale  factor  for

plotting.   The  default  origin on the terminal and the Ramtek is

the lower left of screen (0.,0.).  The X  axis  runs  horzontally,

while  the  Y  axis  runs  vertically.   The default origin on the

printer is the upper  left  corner  of  page.   The  X  axis  runs

vertically  down  page,  while the Y axis runs horizontally across

the page.

When the Ramtek Screen device  is  selected,  FRAME  opens  a

communications  channel to the Ramtek.  If no channel is available

or the Ramtek is in use an error message is typed and the  calling

program is terminated.

CALL FRAME (pl,id,vpx,vpy,zom)

pl   (I): Fortran file unit number (normally 3) used for LONGLIB

Metafile.  If pl < 0 then no metafile is generated.

If pl = 0 then the user will be prompted for a yes/no

metafile.  In this case unit number used will be 3.

3-9

DESCRIPTION OF PLOTTING ROUTINES

id   (I): Screen device code number.

< 0 : Do not clear Ramtek/terminal screens prior to use.

> 0 : Clear Ramtek/terminal screens prior to use.

= 0 : Prompts user for which screen device to use.

A ? response will list the available devices.

= 1 : Use VT100 as Screen Output (only) (Selanar GR100)

= 2 : Use Ramtek as Screen Output (only).

= 3 : Use both Ramtek and VT100 as Screen Output.

= 4 : Do not produce Screen output.

= 5 : Use VT125 as Screen Output (only)

= 6 : Use VT100 as Screen Output (only) (Selanar GR100)

= 8 : Use VT240 as Screen Output (only)

= 9 : Use VT220 as Screen Output (only) (Selanar SG220)

=10 : Use Tektronix 4010/4014 as Screen Output

=11 : Use Tektronix 4107/4109 as Screen Output

(color Tektronix)

=12 : Use Graph-On GO-235 as Screen Output

vpx, (R): relative x,y offset of the bottom left hand corner of

vpy       the screen/page.  Equivalent to PLOT(vpx,vpy,-3).

zom  (R): The value of 'zom' scale factor. (see FACTOR)

3.13  SUBROUTINE GRID

GRID plots a Cartesian grid or solid lines or ticks  at  grid

intersections.  See also LGRID.

CALL GRID (x,y,dx,dy,nx,ny)

x,y   (R): coordinates in the bottom left corner of grid

dx,dy (R): spacing of grid lines in x and y directions

nx,ny (I): number of grids in x and y direction

if nx > 0 and ny > 0 then solid grid plotted

if nx < 0  or ny < 0 then tick grid plotted

if nx < 0 and ny < 0 then boxed tick grid plotted

3.14  SUBROUTINE HLT3D

HLT3D plots a 2 dimensional array to produce a 2-d  histogram

similar  to  PLT3D.   Hidden  lines are supressed.  Transformation

from the array indices (i,j) to (x,y,z) is:

x = xl * .5 * float(2*j-n-1)/float(n-1)

y = yl * .5 * float(2*i-m-1)/float(m-1)

z = zs * (a(i,j) + z0)

Thus,

(1,1) is (-xl/2,-yl/2)   (m,1) is (-xl/2,+yl/2)

(1,n) is (+xl/2,-yl/2)   (m,n) is (+xl/2,+yl/2)

3-10

DESCRIPTION OF PLOTTING ROUTINES

xplotted = x*cos(az) - y*sin(az) + x0

yplotted = x*sin(az)*sin(al) + y*sin(az)*sin(al) + z*cos(al) + y0

The  common  block   PLT3B   returns   these   transformation

parameters  so  that the plotted location (xp,yp) of the corner of

the cube corresponding to the point (i,j,zr) may be computed as:

xp = a1 * j + a2 * i + a3

yp = b1 * j + b2 * i + b3 * zr + b4

The dimension of  the  working  array  is  dependent  on  the

surface  complexity  --  the  greater  the surface complexity, the

greater l2 must be.  As a minimum,  l2  >  4*min(m,n).   See  also

NXTVU and PLT3D.

CALL HLT3D (a,md,nd,m,n,w,l,w2,l2,al,az,xl,x0,yl,y0,zs,z0,ierr)

a     (R): array of values to be plotted dimensioned a(md,nd)

md,nd (I): array dimensions

m,n   (I): size of data in array to be plotted

w     (R): dummy variable so that call is compatible with PLT3D

l     (I): dummy variable so that call is compatible with PLT3D

w2    (R): working storage array dimensioned w2(l2)

l2    (I): working storage array dimension (see note)

al    (R): viewing altitude angle (deg)

az    (R): viewing azimuth angle (deg)

xl,yl (R): length of unprojected axes in plot units

x0,y0 (R): plot origin in plot units

zs    (R): z coordinate scale factor

z0    (R): z coordinate offset

ierr  (I): (returned) error code

= 0 : ok

= 1 : l2 not large enough in NXTVU

COMMON /PLT3B/ a1,a2,a3,b1,b2,b3,b4

3.15  SUBROUTINE HELPME

HELPME calls the VAX VMS interactive  help  utility  for  the

LONGLIB  help  library  with  an initial help request string.  The

librarian  is  interactive  so  that  the  user  is  prompted  for

additional input until a control-Z is typed at which point control

is returned to the calling program.

CALL HELPME(s)

s     (C): initial help request string

3-11

DESCRIPTION OF PLOTTING ROUTINES

3.16  SUBROUTINE JPLTAG

JPLTAG plots the JPL logo (JPL).  Optionally, it can  produce

only  a line outline or fill the outline with a line pattern using

SHADE.

CALL JPLTAG (x,y,h,a,i,d,m,sa,w)

x,y   (R): coordinates of the lower left corner of the initial j

h     (R): letter height in plot units

a     (R): angle of baseline relative to horizontal

i     (I): option flag

(1's digit)  = 0 no action

= 1 draw outline only

= 2 draw shading only

= 3 shade area and draw outline

(10's digit) = pen color (if = 0, pen color is not changed)

d     (R): distance between shading lines

m     (I): shading line type

=>  0 line type (see NEWPEN)

sa    (R): angle of shading lines

w     (R): working array dimensioned at least w(56+54*H)

3.17  SUBROUTINE LGAXS

LGAXS plots a single logarithmic coordinate  axis.   Complete

decades are produced.  See also AXIS and LGLIN.

CALL LGAXS (x,y,s,n,al,a,nmin,dx<,ic>)

x,y   (R): location starting point of the axis

s     (B): axis label string

n     (I): number of characters in string

> 0 :  label on positive side

< 0 :  label on negative side

(100's digit)   = 0 : coordinate line, ticks and labels drawn

= 1 : line and ticks only--no labeling

(1000's digit)  = 0 : numeric labels paralel to axis line

= 1 : numeric labels orthogonal to axis line

(10000's digit) = 0 : color list ignored

= 1 : color list used

al    (R): length of axis

> 0 : axis ticks placed on same side of axis as title

= 0 : no action (return with no plotting)

< 0 : ticks placed on opposite side of axis from title

a     (R): angle at which the axis should be plotted

nmin  (R): number to be printed at the first axis tick (power of ten)

dx    (R): scaling factor in the form dx=(nmax-nmin)/l where

nmax, nmin are the exponent powers at the start

and end of the axis

ic    (I): color array (required if mag(n)>10000))

3-12

DESCRIPTION OF PLOTTING ROUTINES

ic(1) : color for axis line and ticks

ic(2) : color for numbers

ic(3) : color for axis title

3.18  SUBROUTINE LGLIN

LGLIN plots a solid curve through a set of values from  x  to

y.   Either  x  or  y  or  both may in the process be converted to

logarithmic form.  Symbols may  be  used  at  selected  intervals.

(see  also  SCALG  and  LINE).   The plotted x values are computed

according to,

for logarithmic scaling,

xplotted= (alog10(abs(x(i))+1.e-38)-xm)/dx

for linear scaling,

xplotted= (x(i)-xm)/dx

and similarily for y.

CALL LGLIN (x,y,n,k,j,l,lg,ix,iy,xm,dx,ym,dy)

x     (R): array containing the x coordinates

y     (R): array containing the y coordinates

n     (I): number of data points in x and y

(the number of data points in x,y must be equal)

k     (I): take every first (k=1), second (k=2) value etc.

(normally k=1)

j     (I): plotting symbol spacing flag

> 0 : symbol is plotted at every jth plotted point

connected by lines

= 0 : no symbols, lines only

< 0 : symbol is plotted at every abs(j)th plotted point

with no connecting lines

l     (I): plot symbol number (see SYMBOL)

lg    (I): log option

= - 2 : x and y are plotted using logarithmic scaling

= - 1 : x logarithmic, y linear

=   1 : x linear, y logarithmic

ix,iy (I): start index if arrays (normally ix,iy=1)

xm    (R): minimum value scale factor for x array

dx    (R): increment scale factors for x array

ym    (R): minimum value scale factor for y array

dy    (R): increment scale factors for y array

3-13

DESCRIPTION OF PLOTTING ROUTINES

3.19  SUBROUTINE LGRID

LGRID plots a logarithmic or linear grid using  solid  lines,

dotted lines, or ticks.  See also GRID.

CALL LGRID (x,y,dx,dy,nx,ny,i)

x,y   (R): location coordinates for the bottom-left corner of grid

dx,dy (R): spacing of major grid lines in x and y directions

nx    (I): number of major grid lines in x direction

< 0 : log spacing of minor lines

> 0 : no minor lines/ticks

ny    (I): number of major grid lines in y direction

< 0 : log spacing of minor lines

> 0 : no minor lines/ticks

i     (I): option flag

= 0 : solid major/minor lines

= 1 : dotted major/minor lines

= 2 : solid major lines with minor ticks

3.20  SUBROUTINE LINE

LINE plots a solid curve through a set  of  coordinate  pairs

stored  in  two  arrays.   Symbols  may  be  inserted  at selected

intervals and points of curves may be  skipped.   The  coordinates

are calculated as follows:

xplotted(i)=(x(i)-xm)/dx, i=ix to n, step k

yplotted(m)=(y(m)-ym)/dy, m=iy to n, step k

The routine SCALE may be used to compute the scale factors xm, dx,

ym, and dy from the x and y arrays.

CALL LINE (x,y,n,k,j,l,ix,iy,xm,dx,ym,dy)

x     (R): array containing the x coordinates

y     (R): array containing the y coordinates

n     (I): number of data points in x and y

(the number of data points in x, y should be equal)

k     (I): plot every first (k=1), second (k=2) value etc

(normally k=1)

j     (I): plotting symbol spacing flag

> 0 : symbol is plotted at every jth plotted point

connected by lines

= 0 : no symbols, lines only

< 0 : symbol is plotted at every abs(j)th plotted point

with no connecting lines

l     (I): plot symbol number (see SYMBOL)

ix,iy (I): starting indexs in array (normally ix,iy=1)

xm    (R): minimum value scale factor for x array

dx    (R): increment scale factors for x array

3-14

DESCRIPTION OF PLOTTING ROUTINES

ym    (R): minimum value scale factor for y array

dy    (R): increment scale factors for y array

3.21  SUBROUTINE LINSEQ

LINSEQ plots line curves using software generated line types.

Cubic  spline  interpolation  is  used  in  the  generation of the

specified  long  dash/short  dash  sequence.   The  cubic   spline

"smoothing"  prior  to  plotting  may  also  be specified.  Closed

curves and loops are permitted.  A word of caution:   due  to  the

limitations  of  cubic  splines, the plotted curve may not exactly

represent the underlying function.  The code used by this  routine

is an adaptation of the COSMIC routine LAR-11123.

LINSEQ can be used  to  plot  the  same  curve  at  different

locations  by  first  calling  LINSEQ  with the desired data, then

calling LINSEQ again with n set to zero and h changed.

CALL LINSEQ (x,y,z,n,ns,h,s,l1,l2,l3,l4,l5)

x     (R): array containing the x coordinates

y     (R): array containing the y coordinates

z     (R): working array dimensioned at least z(3*n+3)

n     (I): number of points in x,y arrays

if n=0 then arc parameters from the last

call to LINESEQ are used again

(used to replot the same curve at different location)

ns    (I): number of smoothing passes (normally 0)

h     (R): x axis plotting offset (normally 0).  Setting h <> 0

and n=0 permits replotting last curve, shifted right

the specifed amount.

s     (R): approximate interval between interpolated points on

curve.  Actual interval is internally computed.

If s<0 pre-interpolation to 2*n-1 points is done.

l1-l5 (I): dash pattern control flags

l1 = Number of INTERVALS BETWEEN dashes

l2 = Number of LONG dashes per CYCLE

l3 = Number of INTERVALS per LONG dash

l4 = Number of SHORT dashes per CYCLE

l5 = Number of INTERVALS per SHORT dash

Note: if l1=0, l2-l5 are ignored and a solid curve is drawn

if l4=0, l5 is ignored. All dashes same length.

3.22  SUBROUTINE NEWPAGE

NEWPAGE inserts  a  change  page  command  into  the  LONGLIB

metafile.   The  hardcopy  conversion  program use the change page

command to issue a form feed to the metafile  output.   Note  that

NEWPAGE only affects the metafile page and does not change origin,

3-15

DESCRIPTION OF PLOTTING ROUTINES

etc.  It is a dummy call for Ramtek or  terminal  plotting.   This

command is equivalent to CALL PLOT(0.,0.,10).

CALL NEWPAGE

(no arguments)

3.23  SUBROUTINE NEWPEN

NEWPEN calls PPEN, RMPEN, and VPEN which change the  hardware

line  type  and/or  width  of  the  plotting  line  for subsequent

plotting on the metafile, Ramtek,  and  terminal  output  devices.

The  precise  effects  depend  on  the particular graphics device.

There are 10 standard line types  which  are  shown  in  the  last

chapter.     The    output    device    will   use   the   nearest

hardware-supported line type to  the  standard  line  type.   Some

devices   support   additional   types   including   permitting  a

specification of the scale factor of the line type (the length  of

the  dot/dash  pattern).  If a device does not support line types,

the default type is used.  Line widths  are  not  support  on  the

Ramtek  packages  and are simulated in softwared for the terminal.

The metafile package supports all features although  the  metafile

processing  programs  may  only  use the features supported by the

particular hardcopy graphics output device.  Default line type  is

a solid line of width 1 dot.

CALL NEWPEN (i)

i  (I): selects a line type for all additional plotting

for all output devices

< 0 : resets line type to solid line of unit width.

= 0 : line type 0 no change in line width

> 0 : line type and width changed according to,

(1's digit)   : line type (0-9)

(10's digit)  : line width (1-7) (value of 0 does not change width)

(100's digit) : line type pattern scaling (1-7) (0 is no change)

3.24  SUBROUTINE NUMBER

NUMBER plots a floating point number in  a  specified  format

using  a  fortran  format-like  specification.   It  also  permits

free-format and  exponential  notation  formats.   The  number  is

converted  to  an ASCII string plotted at a specified location and

baseline angle using SYMBOL.  The following table illustrates  the

dependence  of  the  output  string on the type (integer/real) and

value of the parameter e.  The table shows the output for an input

f=103.356 and i=-1.

3-16

DESCRIPTION OF PLOTTING ROUTINES

Output    integer e   real e

--------   ---------- --------

103          -1       1003.0

103.          0        0.0

103.          0        3.00

x103.36                7.02

103.36        2        0.02

103.356000    6       10.06

xx103                 1005.0

**                    1002.0 (format overflow)

x103.4                 6.01

*.****                 6.04  (format overflow)

.103E+02              -8.03

*.***                 -5.03  (format overflow)

note: x=space, * indicates overflow

CALL NUMBER (x,y,h,f,a,e,i)

x,y   (R): location position (x,y returned if i=-2 or -3)

If x=999 then x is continued from lower right of

prior call to SYMBOL or NUMBER.  If y=999 then

y is continued.

h     (R): size (height) of digits

f     (R): floating point number to be plotted

a     (R): baseline angle at which to plot

e     (R): output format (e=n.j)

(similar to format statement Fn.j)

n is the total number of characters (max 18)

including the decimal point and j is a two digit number

specifying the number of digits to the right of

the decimal point (to get F6.4 use e=6.04)

if e < 0 number is plotted in exponential notation.

if e = 0 then number is output in free format

if n = 0 then number is output in free format

integer with no decimal point shown

The number will be shown as an m digit integer

if using e=1000+m.

NOTE for VAX usage: if an integer in the range (-1 to 12) is passed

for e, NUMBER detects and uses the integer value

to specify a free-format specifying

the number of digits to the right of the decimal point.

a -1 inhibits decimal point

i     (I): centering flag (see SYMBOL)

= -3 : same as -2 but string is not plotted and

last position is not affected

= -2 : same as -1 but returns end point in x,y

= -1 : (x,y) is lower left corner of plotted array

=  0 : (x,y) is center of plotted array

=  1 : (x,y) is lower right corner of plotted array

=  2 : no action

3-17

DESCRIPTION OF PLOTTING ROUTINES

3.25  SUBROUTINE PFACTOR

PFACTOR is  called  by  FACTOR  to  change  the  input  scale

conversion  factor  for  the  LONGLIB metafile package.  It may be

called separately if desired.  Only the metafile plotting  package

is  affected.   The routines RFACTOR and VFACTOR may be separately

called to change the input scale conversion factor on  the  Ramtek

and terminal packages, respectively.  See FACTOR.

CALL PFACTOR (fac)

fac (R): new conversion factor for coordinate values

(only the metafile scaling is affected)

<= 0 : scale factor reset to unity

>  0 : new scale factor

3.26  SUBROUTINE PLOT

PLOT is the central routine for controlling the motion of the

electronic  pen  for  all  LONGLIB graphics device packages.  PLOT

calls the Ramtek, metafile  and  terminal  PLOT  routines  PLOTRM,

PPLOT,  PLOTVT based on the graphics devices initialized by FRAME.

These individual "package PLOT" routines can be called  separately

if  desired  to  affect  only  the  particular package.  Via these

package-specific plot routines PLOT can moves the pen, change  the

origin or pen color, issue a page change command, etc.  A relative

rotation angle for all successive plotting may also be  specified.

LONGLIB  defines  a plotting window or viewport, the size of which

is device dependent, within which pen motions  are  clipped.   The

viewport  may  be  set  to  an  arbitrary  size  within the device

plotting window (e.g.  terminal screen).  An attempt to  make  the

viewport  bigger than the device output will force the viewport to

be the device output window (this is the default).

CALL PLOT (x,y,i)

x,y   (R): coordinate values

i     (I): plot function parameter

=  0: color control

x is the new line color

if x < 0 the screen is cleared

if x >= 0 then plotting angle becomes y

=  2: draw to (x,y) with 'pen down'

= -2: same as i=2. (x,y) becomes new origin

=  3: move to (x,y) with 'pen up'

= -3: same as i=3. (x,y) becomes new origin

=  4: upper right corner of viewport set to (x,y)

= -4: lower left corner of viewport set to (x,y)

=  5: pick pen up at last point

=  9: erase to (x,y) (plot with color 0)

= -9: erase to (x,y) (x,y) becomes new origin

3-18

DESCRIPTION OF PLOTTING ROUTINES

= 10: issue change page command to metafile

= 11: end plot (close LONGLIB)

=999: end plot (close LONGLIB)

3.27  SUBROUTINE PLOTND

PLOTND is  used  to  signal  LONGLIB  that  all  plotting  is

complete.   It  sends  the  final output buffers to the respective

graphics devices,  closes  files  and  channels,  and  resets  the

terminal  to the normal text mode.  PLOTND is equivalent to a CALL

PLOT(0.,0.,11) command.

CALL PLOTND

(no arguments)

3.28  SUBROUTINE PLOTS

PLOTS initializes the LONGLIB graphics package via a call  to

FRAME.   PLOTS  provides  compatibility  with  existing CALCOMP or

PLOTS-10 compatible code.  It simply calls FRAME  with  argurments

which cause FRAME to prompt for optional usage of the Longlib meta

file output and a graphics screen.  For new code use FRAME.

CALL PLOTS (...)

(arguments are ignored)

3.29  SUBROUTINE PLOTRM

PLOTRM is the central routine for controlling the plotting of

lines  to the Ramtek or REF package.  PLOTRM is called by the PLOT

routine but can be called separately.  Any call to PLOTRM when the

Ramtek  package  is  not initialized is a dummy call.  Options for

PLOTRM are similar  to  PLOT  (see  documentation  on  PLOT).   An

attempt  to make the viewport bigger than the Ramtek screen window

will force the viewport to be the size of the Ramtek screen window

(this  is  the  default).   Typically  the Ramtek screen window is

either 13.75 or 11 by 11 inches depending on the  type  (1280x1024

and  512x512,  respectively),  with the lower left corner at (0,0)

and the upper right corner at (13.75,11).

CALL PLOTRM (x,y,i)

x,y   (R): coordinate values

i     (I): plot function parameter

=  0: Ramtek color control

x is the Ramtek color table index

3-19

DESCRIPTION OF PLOTTING ROUTINES

if x < 0 the Ramtek screen is cleared

if x >= 0 then rotation angle becomes y

=  2: Ramtek draw to (x,y) with 'pen down'

= -2: same as i=2. point (x,y) becomes new origin

=  3: Ramtek move to (x,y) with 'pen up'

= -3: same as i=3. Point (x,y) becomes new origin

=  4: upper right corner of viewport set to (x,y)

= -4: lower left corner of viewport set to (x,y)

=  5: pick pen up at last point

=  9: draw to (x,y) 'pen down' color 0 (erase)

= -9: same as i=9. Point (x,y) becomes new origin

= 11: end plot (close Ramtek package)

=999: end plot (close Ramtek package)

3.30  SUBROUTINE PLOTVT

PLOTVT is the central routine for controlling the plotting of

lines to the terminal screen device REF package.  PLOTVT is called

by the PLOT routine but can be called  separately.   Any  call  to

PLOTVT  when  the  terminal  package is not initialized is a dummy

call.  Options for PLOTVT are similar to PLOT  (see  documentation

on  PLOT).   An  attempt  to  make  the  viewport  bigger than the

terminal screen window will force the viewport to be the  size  of

the  terminal  screen window (this is the default).  Typically the

terminal screen  window  is  9.5  by  either  9.5  or  7.2  inches

(depending  on  the  terminal) with the lower left corner at (0,0)

and the upper right corner at (9.5,9.5).

CALL PLOTVT (x,y,i)

x,y   (R): coordinate values

i     (I): plot function parameter

=  0: if x < 0 the terminal graphics screen is cleared

if x >= 0 then rotation angle becomes y

if x = 0 set terminal to erase mode plotting

if x <> 0 and x <> 999 set line color to x

if x = 999 set terminal to XOR mode plotting

(only if terminal has capability)

=  2: draw to (x,y) with 'pen down' on terminal

= -2: same as i=2. Point (x,y) becomes new origin

=  3: move to (x,y) with 'pen up' on terminal

= -3: same as i=3. Point (x,y) becomes new origin

=  5: pick pen up at last point

=  4: upper right corner of viewport set to (x,y)

= -4: lower left corner of viewport set to (x,y)

=  9: erase to (x,y) on terminal (if supported)

= -9: same as i=9. Point (x,y) becomes new origin

= 11: end plot (close terminal package)

=999: end plot (close terminal package)

3-20

DESCRIPTION OF PLOTTING ROUTINES

3.31  SUBROUTINE PLRAX

PLRAX plots a circular axis for plotting in  polar  form.   A

series  of concentric circles are drawn around (x,y) at increasing

radi to the maximum radius.   Provisions  are  included  for  half

circle,  quarter  circle,  etc.   Labeling  of starting points and

ending points may be changed.

CALL PLRAX (x,y,r,as,ae,a0,a1)

x     (R): x coordinate of center of polar axis

y     (R): y coordinate of center of polar axis

r     (R): radius of polar axis in plot units

as    (R): starting angle of axis in degrees from horizontal

ae    (R): ending angle of axis in degrees from horizontal

Note: as=0 and ae=360 yields full circle axis

a0    (R): number label of starting angle

a1    (R): number label of ending angle

Note: if a0=a1 then angles are not labeled.

3.32  SUBROUTINE PLRLN

PLRLN plots a solid  curve  through  the  set  of  coordinate

points  in  polar  form  stored in r and t arrays.  Symbols may be

inserted at selected intervals.  Angle  values  stored  in  the  t

array  are  in  degrees referenced to horizontal.  Coordinates are

calculated as follows:

r=(r-rmin)/dr

xplotted=r*cos(t*pi/180)

yplotted=r*sin(t*pi/180)

CALL PLRLN (r,t,n,j,l,ir,rmin,dr)

r     (R): array of radial values to be plotted

t     (R): array of angle values

n     (I): number of points to plot

j     (I): plotting symbol option flag

> 0 : symbol plotted every jth point with connecting lines

= 0 : line plotted only with no symbols

< 0 : symbols plotted only, no connecting line

l     (I): symbol number (see SYMBOL)

ir    (I): start index in r and t arrays

rmin  (R): minimum radius scale factor

dr    (R): scale factor

3-21

DESCRIPTION OF PLOTTING ROUTINES

3.33  SUBROUTINE PLT3D

PLT3D plots a 2 dimensional array as a surface scribed with a

linear  grid  parallel  to the x and y axes, i.e., a mesh.  Hidden

lines are supressed.  Transformation from the array indices  (i,j)

to (x,y,z) is:

x = xl * .5 * float(2*j-n-1)/float(n-1)

y = yl * .5 * float(2*i-m-1)/float(m-1)

z = zs * (a(i,j) + z0)

Thus,

(1,1) is (-xl/2,-yl/2)   (m,1) is (-xl/2,+yl/2)

(1,n) is (+xl/2,-yl/2)   (m,n) is (+xl/2,+yl/2)

xplotted = x*cos(az) - y*sin(az) + x0

yplotted = x*sin(az)*sin(al) + y*sin(az)*sin(al) + z*cos(al)+y0

The  common  block   PLT3B   returns   these   transformation

parameters  so  that  the  plotted  location  (xp,yp)  of  a point

(i,j,zr) may be computed as:

xp = a1 * j + a2 * i + a3

yp = b1 * j + b2 * i + b3 * zr + b4

The  dimension  of  the  second  set  of  working  arrays  is

dependent  on  the  surface  complexity -- the greater the surface

complexity, the greater l2 must be.  As a minimum, l2  >  l.   See

also NXTVU and HLT3D.

CALL PLT3D (a,md,nd,m,n,w,l,w2,l2,al,az,xl,x0,yl,y0,zs,z0,ierr)

a     (R): array of values to be plotted dimensioned a(md,nd)

md,nd (I): array dimensions

m,n   (I): size of data in array to be plotted

w     (R): working array dimensioned w(l) l=>4*min(m,n)

l     (I): working array dimension

w2    (R): working storage array dimensioned w2(l2)

l2    (I): working storage array dimension (see note)

al    (R): viewing altitude angle

az    (R): viewing azimuth angle

xl,yl (R): length of unprojected axes (plot units)

x0,y0 (R): plot origin

zs    (R): z coordinate scale factor

z0    (R): z coordinate offset

ierr  (I): (returned) error code

= 0 : ok

= 1 : l2 not large enough in NXTVU

= 2 : l not large enough

COMMON /PLT3B/ a1,a2,a3,b1,b2,b3,b4

3-22

DESCRIPTION OF PLOTTING ROUTINES

3.34  SUBROUTINE PPLOT

PPLOT is the central routine for controlling the plotting  of

lines  to  the  LONGLIB  metafile package.  PPLOT is called by the

PLOT routine but can be called separately.  Any call to PPLOT when

the  metafile package is not initialized is a dummy call.  Options

for PPLOT are similar to PLOT (see  documentation  on  PLOT).   An

attempt  to  make  the  viewport bigger than the metafile plotting

window will force the viewport to be  the  size  of  the  metafile

plotting  window  (this  is  the  default).  The metafile plotting

window is 56.5 by 56.5 inches with the lower left corner at  (0,0)

and  the  upper  right corner at (56.5,56.5).  Since most hardcopy

devices can not produce such a  large  output  page,  the  LONGLIB

metafile  processor  programs  which  convert  the  metafile to an

output  file,  "strips"  the  metafile   window   into   separate,

overlapping  output  pages.  Only non-blank page strips are output

to the device.

CALL PPLOT (x,y,i)

x,y   (R): coordinate values

i     (I): plot function parameter

=  0: line color control

x is the new line color

if x >= 0 then plotting angle becomes y

=  2: draw to (x,y) with 'pen down'

= -2: same as i=2. (x,y) becomes new origin

=  3: move to (x,y) with 'pen up'

= -3: same as i=3. (x,y) becomes new origin

=  4: upper right corner of viewport set to (x,y)

= -4: lower left corner of viewport set to (x,y)

=  5: pick pen up at last point

=  9: erase to (x,y)

= -9: erase to (x,y), (x,y) becomes new origin

= 10: issue change page command to metafile

= 11: end plot (close metafile package)

=999: end plot (close metafile package)

3.35  SUBROUTINE PPEN

PPEN is called by NEWPEN to change  the  hardware  line  type

and/or  width  of the plotting line for subsequent plotting on the

metafile.  It may be called separately to change only the metafile

line  type.   Ramtek  and terminal output device line types may be

changed using RMPEN and VPEN, respectively.   While  the  metafile

output  device  supports line types, widths and type scale factors

(the length of the linetype dot/dash pattern), the programs  which

process  the  LONGLIB  metafile into the output format required by

the hardcopy device may not support  all  features.   The  precise

effects  depend  on  the  particular graphics device.  Raster scan

converter programs for dot matrix printers support all options and

3-23

DESCRIPTION OF PLOTTING ROUTINES

the  10  standard  line types shown in the last chapter.  On other

hardcopy devices, the metafile processing  program  uses  the  the

nearest  hardware-supported  line  type to the standard line type.

Line widths are defined in terms of the minimum line widths.   The

default line type is a solid line of width 1 dot.

CALL PPEN (i)

i     (I): selects a line type for all additional plotting

to the LONGLIB metafile

< 0 : resets line type to solid line of unit width.

= 0 : line type 0 no change in line width

> 0 : line type and width changed according to,

(1's digit)   : line type (0-9)

(10's digit)  : line width (1-7) (value of 0 does not change width)

(100's digit) : line type pattern scaling (1-7) (0 is no change)

3.36  SUBROUTINE RECT

RECT plots a rectangle defined by the lower  left  and  upper

right  hand  corners.   The pen moves UP to lower left hand of the

rectangle, plots the rectangle, and leaves the  pen  DOWN  at  the

lower left corner.

CALL RECT (x1,y1,x2,y2)

x1,y1 (R): lower left hand corner coordinates

x2,y2 (R): upper right hand corner coordinates

3.37  SUBROUTINE RESPL

RESPL performs a penup, then restores  the  current  plotting

origin,  color,  line type, scale, and plotting angle saved by the

SAVPL command.  In conjunction with SAVPL (which saves a  previous

condition) context changes can be easily made.  RESPL calls PRESPL

(metafile restore), RRESPL (Ramtek restore), and VRESPL  (terminal

restore) which can be independently used if desired.  If the stack

is empty, no restore occurs.  (NOTE:  graphics devices must be  in

the  graphics  mode  when  this  routine is executed).  When using

PRESPL, RRESPL, or VRESPL a pen up operation  should  be  executed

imediately prior to the call.

CALL RESPL

(no arguments)

3-24

DESCRIPTION OF PLOTTING ROUTINES

3.38  SUBROUTINE RFACTOR

RFACTOR is  called  by  FACTOR  to  change  the  input  scale

conversion  factor  for  the  LONGLIB metafile package.  It can be

called separately if desired.  Only the Ramtek plotting package is

affected.   The  routines  PFACTOR  and  VFACTOR may be separately

called to change the input scale conversion factor on the metafile

and terminal packages, respectively.  See FACTOR.

CALL RFACTOR (fac)

fac (R): new conversion factor for coordinate values

(only the Ramtek scaling is affected)

<= 0 : scale factor reset to unity

>  0 : new scale factor

3.39  SUBROUTINE RMPEN

RMPEN is called by NEWPEN to change the  hardware  line  type

and/or  width  of the plotting line for subsequent plotting on the

Ramtek.  It may be called separately to  change  only  the  Ramtek

line  type.  Metafile and terminal output device line types may be

changed using PPEN  and  VPEN,  respectively.   While  the  Ramtek

output  device  supports  line  types  and type scale factors (the

length of the linetype dot/dash pattern) it does not support  line

widths  in  hardware.   The  default  line type is a solid line of

width 1 dot.

CALL RMPEN (i)

i     (I): selects a line type for all additional plotting

to the LONGLIB Ramtek

< 0 : resets line type to solid line of unit width.

= 0 : line type 0 no change in line width

> 0 : line type and width changed according to,

(1's digit)   : line type (0-9)

(10's digit)  : line width (0-7) (ignored)

(100's digit) : line type pattern scaling (1-7) (0 is no change)

3.40  SUBROUTINE RTERM

RTERM is designed to be similar to the CTERM routine but  for

use  with  the  Ramtek.   It  is  a  dummy call when not in Ramtek

plotting mode.  When using the REF package, it calls REFDIS.

CALL RTERM (iarg)

iarg (I): operation code

= 0 : clear Ramtek screen

3-25

DESCRIPTION OF PLOTTING ROUTINES

= 2 : ask if clear screen desired.

reply should be: "Y" or "N".  Default is "N".

NOTE: a reply of "Q" will execute RTERM(3).

a reply of "S" will close channel and stop

Ramtek plotting until next RTERM(2) (see CTERM)

=-2 : clear Ramtek screen

= 3 : close Ramtek plotting (closes old channel)

=-3 : reopen Ramtek plotting (opens new channel--does

not reinitalize Ramtek plotting package)

=-4 : clear Ramtek screen

3.41  SUBROUTINE SAVPL

SAVPL performs a penup,  then  stores  the  current  plotting

origin, color, line type, scale, etc.  on a stack which will store

up six calls.  In conjunction with RESPL (which restores  previous

condition) context changes can be easily made.  SAVPL calls PSAVPL

(metafile save), RSAVPL (Ramtek save), and VSAVPL (terminal  save)

which  can  be  independently  used if desired.  When the stack is

full no save is performed.

CALL SAVPL

3.42  SUBROUTINE SCALE

SCALE  calculates  the  minimum  and  a   scaled   (smoothed)

increment from an array of values.  The maximum and minimum of the

array are computed and the  difference  is  divided  by  a  length

parameter.   The  resulting  values are "smoothed" so that numeric

labels  appear  "nice"  when  labeled  at  xm  and  at  one   inch

increments.  The smoothed numbers are taken from the set of values

1,2,4,5,8 * 10n that equals or is smaller  than  the  true  value.

SCALE  is useful in determaning the scale factors for LINE.  It is

used extensively in  the  MASTER  routines.   SCALE  is  not  very

intelligent  and does not always make good choices.  SCALE selects

xm and dx so that the plotted x values may be computed  using  the

following   formula   such   that   if  x=xm,  xplotted=0  and  if

x=xlen*dx+xm, xplotted=xlen.

xplotted = ( x - xm ) / dx

CALL SCALE (x,xlen,n,k,ix,xm,dx)

x     (R): array of data points from which scale is determined

xlen  (R): scale length

n     (I): number of data points in x (n>1)

k     (I): use every first (k=1) value, second (k=2) value, etc.

normally k=1.

3-26

DESCRIPTION OF PLOTTING ROUTINES

ix    (I): first data point index for x (normally ix=1)

xm    (R): contains smoothed minimum after the call (returned)

dx    (R): contains smoothed increment after call (returned)

3.43  SUBROUTINE SCALG

SCALG  calculates  a  scaled  minimum  and  increment  factor

similar     to     SCALE    but    uses    the    log    (actually

alog10(abs(val)+1.e-38)) of the input array.  See also  SCALE  and

LGLIN.   SCALG  selects xm and dx so that the plotted x values may

be  computed  using   the   following   formula   such   that   if

alog10(abs(x)+1.e-38)=xm      then      xplotted=0      and     if

alog10(abs(x)+1.e-38)=xlen*dx+xm then xplotted=xlen.

xplotted = ( alog10(abs(x)+1.e-38) - xm ) / dx

CALL SCALG (x,xlen,n,k,ix,xm,dx)

x     (R): array of data points from which scale is determined

xlen  (R): length

n     (I): number of data points in x (n>1)

k     (I): use every first (k=1) value, second (k=2) value, etc.

normally k=1

ix    (I): first data point index for x (normally ix=1)

xm    (R): contains smoothed minimum after the call (returned)

dx    (R): contains smoothed increment after call (returned)

3.44  SUBROUTINE SHADE

The SHADE subroutine fills in the area inclosed by  the  line

defined by the x and y arrays with equally spaced lines at a given

angle using a specified line type.  The first and  last  point  of

the x and y array are assumed to be connected.

CALL SHADE (x,y,n,i,l,d,t,w,ma,xm,dx,ym,dy)

x     (R): array of x values

y     (R): array of y values

n     (I): number of points in array

i     (I): increment between points

l     (I): shade format control

= -3 : clear area and outline

= -2 : clear area

= -1 : clear outline

=  0 : no action

=  1 : draw outline

=  2 : shade area

=  3 : shade area and outline

3-27

DESCRIPTION OF PLOTTING ROUTINES

d     (R): distance between shading lines

t     (R): angle of shading lines in degrees

w     (R): working array dimensioned at least 3*n

ma    (R): line type of shading

=> 0 : line type (see NEWPEN)

xm    (R): minmum scale factor for x (see LINE)

dx    (R): x increment scale factor

ym    (R): minmum scale factor for y

dy    (R): y increment scale factor

3.45  SUBROUTINE SYMBOL

The SYMBOL routine plots an ASCII string.   Upper  and  lower

case characters can be plotted as well as special plotting symbols

and characters.  A list of symbols is shown in the  last  chapter.

The   plotting  symbols  0  to  16  are  centered  vertically  and

horizontally, while other symbols have a reference  point  on  the

lower  left edge of the character.  Hence, i=-1 should be used for

centered plot symbols 0 thru 16.  The number of characters in  the

input  string  which  should  be  plotted can be specified.  If an

ASCII null (0) is encountered  in  the  string  beyond  the  first

position, the routine terminates.

The  string  can  be  plotted  left-justified,  centered,  or

right-justified.   When the string is left-justified, the location

of the end of the plotted string can be optionally returned.   The

length  of  the  plotted  string can be computed by calling SYMBOL

first with i=-3 and computing the difference between the start and

ending points of the string.

More elaborate characters and different fonts may be obtained

using  SYMS.   MASTER  routines, AXIS routines, and NUMBER all use

SYMBOL characters.  If the user desires to use SYMS  in  place  of

SYMBOL  throughout  a  program,  the  user  can  add the following

routine to the program.  The subroutine should  be  titled  SYMBOL

with  the  arguments  described below.  This routine simply passes

the arguments (in the same order) to SYMS.

SUBROUTINE SYMBOL(x,y,h,s,a,n,i)  ! symbol replacement

INTEGER S(1)

A = SYMS(x,y,h,s,a,n,i)           ! call syms

RETURN

END

Note:  VAX FORTRAN CHARACTER types can not be  used  directly

since   calling  subroutines  with  CHARACTER  types  is  done  by

DESCRIPTOR.  Use BYTE or INTEGER arrays for s or  use  %REF()  for

character  variables,  i.e., call SYMBOL(0., 0., .25, %REF(S), 0.,

10,-1).  See the section on machine dependency.

CALL SYMBOL (x,y,h,s,a,n,i)

3-28

DESCRIPTION OF PLOTTING ROUTINES

x,y   (R): location position (x,y returned if i=-2 or -3)

If x=999 then x continued from last position in

prior SYMBOL or NUMBER call. If y=999 then y continued.

h     (R): height of the string to be printed

s     (B): alpha string containing the text to be plotted

a     (R): angle at which the string is to be plotted

n     (I): number of characters in string s to plot

= -2 : draws pen down to (x,y) before symbol plotted

= -1 : plots a single symbol

>  0 : number of characters to plot

i     (I): location flag

= -3 : same as -2 but string is not plotted and

last position is not affected

= -2 : same as -1 but returns end point in x,y

= -1 : (x,y) is lower left corner of plotted string

=  0 : (x,y) is center of plotted array

=  1 : (x,y) is lower right corner of plotted string

=  2 : no action

3.46  REAL FUNCTION SYMS

SYMS is similar to SYMBOL in  that  it  will  plot  an  ASCII

string  (i.e.   a  byte array).  However, SYMS provides additional

math and plotting symbols  as  well  as  several  character  fonts

(including   Greek   characters).   SYMS  has  several  additional

enhancements to permit complicated equations to be  plotted.   All

but  the  9th  font have variable width characters.  Several fonts

are designed to produce solid characters when the  plotting  scale

is  small.  The number of characters in the input string s plotted

and/or interpreted can be specified.  If  an  ASCII  null  (0)  is

encountered  in  the  string after the first position, the routine

terminates.

The ASCII character "|" (decimal 124) is used  as  a  control

character  to change fonts or subscripting options.  The character

following the "|" character is used  according  to  the  following

table:

3-29

DESCRIPTION OF PLOTTING ROUTINES

SYMS Options

ASCII Character  Decimal       Effect

________________ _________     ________

0-9           48-57         Change to font (0-9)

W             87           Move forward one space

X             88           Move forward 1/2 space

Y             89           Move backward one space

Z             90           Move backward 1/2 space

[             91           Reset to default

\             92           Begin subscripting

]             93           "Back up" one character

^             94           Begin superscripting

_             95           Un sub/super

`             96           Change scale (requires

another character-- "+"

indicates increase size by 2,

"-" indicates shrink by 1/2.

a             97           Begin over printing

b             98           Begin under printing

The number of characters in  the  string  should  include  control

characters.

The width of one  space  is  moved  when  moving  forward  or

backward.   "Back  up" returns the positioning to the start of the

last character.  Up  to  6  "back  up"  commands  can  be  issued.

Super/sub  scripting  can  be done recursively.  Only one level is

"popped off" by the un-sub/super command.  Scale  changes  require

an  additional  character  (either  a  "+"  or "-" to indicate the

direction).  Over/under printing  permit  summation  and  integral

limits  to  be added.  Available fonts are listed in the following

Table:

Fonts Available in SYMS

Font          Characters              Description

______        ____________            _____________

0 [default]  ASCII 0-31              Plotting Symbols

ASCII 32-127            Simplex font--variable width

1            ASCII 32-127            Roman

2            ASCII 64-127            Greek Simplex

3            ASCII 32-127            Roman Italic

4            ASCII 32-127            Duplex bold

5            ASCII 64-127            Special math symbols

6            ASCII 32-127            Greek bold

7            ASCII 32-127            Simplex Italic

8            ASCII 32-127            Crude Simplex--fixed width

3-30

DESCRIPTION OF PLOTTING ROUTINES

For example, the following summation using Greek  characters,

a math symbol, and a super scripted variable can be wrtten:

alph string = 'A = |6R a|^2|_|]|]|]|]|b|1|2n=1|]|]|]|_|a|5K'

infinity    2

A = SIGMA alpha

zeta=1

Note:  SYMS can be called as a real function.   The  returned

value  is  the  final  length  of the plotted string.  When i=2 no

plotting is done but the length is returned.  When i=-2 the string

is  plotted  and  the  lower  left  corner  of  the next character

position after the end of the string is returned in x,y.

Note:  VAX FORTRAN CHARACTER types can not be  used  directly

since   calling  subroutines  with  CHARACTER  types  is  done  by

DESCRIPTOR.  Use BYTE or INTEGER arrays for s or  use  %REF()  for

character variables, i.e., call SYMS(0., 0., .25, %REF(S), 0., 10,

-1).  See the section on machine dependency.

rlen = SYMS (x,y,h,s,a,n,i)

x,y    (R): string position (x,y returned if i=-2 or i=-3)

If x=999 then x is continued from last position in

prior SYMS call.  If y=999 then y continued.

h      (R): height of the string to be printed

s      (B): alpha array containing the text to be plotted

a      (R): angle at which the string is to be plotted

n      (I): number of characters in string s

i      (I): centering flag

= -3 : same as -2 but string is not plotted and

last position is not affected

= -2 : same as -1 but returns end point in x,y

= -1 : (x,y) is lower left corner of plotted string

=  0 : (x,y) is center of plotted array

=  1 : (x,y) is lower right corner of plotted string

=  2 : no plotting, plotted length of string returned

rlen   (R): (returned) length of plotted text string

3.47  REAL FUNCTION SYMSS

SYMSS is identical to SYMS except that  it  plots  "smoothed"

characters  using the software linetype generation routine LINSEQ.

Smoothing improves the appearance of most of the  fonts  when  the

characters  are  plotted  at large scale.  See SYMS and LINSEQ for

additional details.

rlen = SYMSS (x,y,h,s,a,n,i,al,l1,l2,l3,l4,l5)

3-31

DESCRIPTION OF PLOTTING ROUTINES

x,y   (R): string position (x,y returned if i=-2 or i-3)

If x=999 then x continued from from last position in

prior SYMS call. If y=999 then y continued.

h     (R): height of the string to be printed

s     (B): alpha array containing the text to be plotted

a     (R): angle at which the string is to be plotted

n     (I): number of characters in string s

i     (I): centering flag

= -3 : same as -2 but string is not plotted and

last position is not affected

= -2 : same as -1 but returns end point in x,y

= -1 : (x,y) is lower left corner of plotted array

=  0 : (x,y) is center of plotted array

=  1 : (x,y) is lower right corner of plotted array

=  2 : no plotting, plotted length of string returned

al    (R): smoothed arc length (typically 0.05 to 0.01)

l1-5  (I): LINSEQ arc type, use l1=0 for a solid line.

rlen  (R): (returned) length of plotted text string

3.48  SUBROUTINE VFACTOR

VFACTOR is  called  by  FACTOR  to  change  the  input  scale

conversion  factor  for  the  LONGLIB metafile package.  It may be

called separately if desired.  Only the terminal plotting  package

is  affected.   The routines PFACTOR and RFACTOR may be separately

called to change the input scale conversion factor on the metafile

and Ramtek packages, respectively.  See FACTOR.

CALL VFACTOR (fac)

fac (R): new conversion factor for coordinate values

(only the terminal scaling is affected)

<= 0 : reset scale factor to unity

>  0 : new scale factor

3.49  SUBROUTINE VPEN

VPEN is called by NEWPEN to change  the  hardware  line  type

and/or  width  of the plotting line for subsequent plotting on the

terminal screen device.  It may be  called  separately  to  change

only  the  terminal  line  type.  Metafile and Ramtek and terminal

output device line types may be changed separatly using  PPEN  and

RMPEN,  respectively.   While  the  terminal  output device driver

supports line types in hardware,  line  widths  are  supported  in

software  by  outputing  multiple single-width lines offset by one

pixel.  Line type  scale  factors  (the  length  of  the  linetype

dot/dash  pattern)  are  not  used.  Not all terminals support all

standard line types.  If a particular terminal  does  not  support

3-32

DESCRIPTION OF PLOTTING ROUTINES

the  requested  line  type,  normally  a  solid line is used.  The

default line type is a solid line of width 1 dot.

CALL VPEN (i)

i     (I): selects a line type for all additional plotting

to the terminal screen output device

< 0 : resets line type to solid line of unit width.

= 0 : line type 0 no change in line width

> 0 : line type and width changed according to,

(1's digit)   : line type (0-9)

(10's digit)  : line width (1-7) (0 value does not change width)

(100's digit) : line type pattern scaling (ignored)

3.50  SUBROUTINE WHERE

WHERE returns the location from the last call to  PLOT.   The

Zoom  scale  factor  value  is  returned from the LONGLIB graphics

device packages in the priority order:  terminal if open or Ramtek

if open or else from the metafile.  Does nothing when no device is

open.

CALL WHERE (x,y,z)

x,y   (R): (returned) values of x,y from last call to plot

z     (R): (returned) zoom value.

3.51  SUBROUTINE WHEREPR

WHEREPR  returns  information  on   the   metafile   plotting

parameters.  If lu <= 0 then metafile has not been initialized.  A

routine FIXPR0 (which has the same parameters) may be used to  set

these variables to absolute values without error checking.

CALL WHEREPR (x,y,ax,ay,z,a,rx,ry,lu,m,iw,ic)

x,y   (R): (returned) current origin

ax,ay (R): (returned) last scaled and shifted origin point

z     (R): (returned) current zoom scale factor

a     (R): (returned) current plotting angle

rx,ry (R): (returned) resolution of metafile

lu    (I): (returned) FORTRAN file output unit number

(if lu <= 0 metafile package not initialized)

m     (I): (returned) current line type

iw    (I): (returned) current line width

ic    (I): (returned) current line color

3-33

DESCRIPTION OF PLOTTING ROUTINES

3.52  SUBROUTINE WHERERM

WHERERM  returns   information   on   the   Ramtek   plotting

parameters.   If  c  <= 0 then ramtek has not been initialized.  A

routine FIXRM0 (with same parameters) may be  used  to  set  these

variables to absolute values without error checking.

CALL WHERERM (x,y,z,a,rx,ry,nt,ns,i,ic)

x,y   (R): (returned) current origin

z     (R): (returned) current zoom scale factor

a     (R): (returned) current plotting angle

rx,ry (R): (returned) current pixel resolution

nt    (I): (returned) current line bit pixel pattern

ns    (I): (returned) current line bit scale factor

i     (I): (returned) Ramtek color

ic    (I): (returned) Ramtek channel

(if ic <= 0 ramtek is not initialized)

3.53  SUBROUTINE WHEREVT

WHEREVT  returns  information  on   the   terminal   plotting

parameters.   If  nv  <=  0  then  terminal graphics have not been

initialized.  A routine FIXVT0 (with same arguments) may  be  used

to set these variables to absolute values without error checking.

CALL WHEREVT (x,y,z,a,rx,ry,iv,ns,it,iw,ic)

x,y   (R): (returned) current origin

z     (R): (returned) current zoom scale factor

a     (R): (returned) current plotting angle

rx,ry (R): (returned) current pixel resolution

iv    (I): (returned) terminal code

(if nv <= 0 terminal is not initialized)

ms    (I): (returned) internal terminal-type code

= 1 VT100 with Selanar GR100

= 2 VT125

= 3 VT240

= 4 VT220 with Selanar GR220

= 5 Tektronix 4010

= 6 Tektronix 4109

= 7 Graphon GO-235

it    (I): (returned) current line type

iw    (I): (returned) current line width

ic    (I): (returned) current line color

3-34

CHAPTER 4

DESCRIPTION OF 3-D PLOTTING ROUTINES

The following paragraphs contain detailed descriptions of the

subroutines included in the LONGLIB library for 3-d plotting.  For

added flexibility, two distinct families of 3-d plotting  routines

have  been  provided.   One  family  is designed for plotting with

hidden line removal; the other family is more  flexible  but  does

not  perform  hidden line removal.  Both options assume that FRAME

has already been called, i.e.  the plot package is already opened.

The  3-d routines call PLOT as output so that the 2-d plot origin,

scaling,  rotation,  etc.   are  used  in  addition  to  any   3-d

operations.

The nominal Z axis of  the  3-d  plot  packages  for  plotted

objects  runs  out of the screen.  The X and Y axes are defined as

before.

Two separate, independent  3-d  packages  exist.   These  are

identified  by  the initialization routines used for each package.

The hidden line removal package is INIT3DH and is  a  modification

of  the COSMIC hidden line code package (ARC-11446).  The other is

INIT3D.  INIT3D does not perform any hidden line  removal.   These

package  differ  not only in hidden line removal but also in speed

of operation and memory requirements.  The packages are completely

independent.  Only routines designed for a particular package will

work  with  that  package.    It   is   possible   to   use   both

simultaneously.

4.1  INIT3D ROUTINES

The INIT3D 3d plotting package permits 3-d plotting but  does

not include hidden line removal.  The family of routines used with

INIT3D include:

1.  PLOT3D -- the central plot routine for INIT3D

2.  AXIS3D -- plots axes using PLOT3D, NUM3D, and SYM3D

4-1

DESCRIPTION OF 3-D PLOTTING ROUTINES

3.  NUM3D -- plots numbers using PLOT3D

4.  SYM3D -- plots symbols using PLOT3D

5.  WHERE3D -- returns the screen  coordinates  of  the  last

point drawn by PLOT3D.

The INIT3D family of 3-d plotting  routines  are  desiged  to

plot  wireframe  line  plots  with no hidden line removal.  Memory

requirements are modest and plotting is more rapid  than  for  the

INIT3DH routines.  For an example of the use of the INIT3D package

see  the  EXAMP3D  program  included  with  the  LONGLIB  graphics

library.

4.1.1  SUBROUTINE INIT3D

INIT3D sets the absolute origin, rotations, and scale  factor

of  the  3-d  package.   These  functions  are  distinct  from the

functions of PLOT.  INIT3D may be called  at  any  time  to  reset

these  functions  without  closing  the  plot  package.   The plot

package must be opened with FRAME prior to the call to INIT3D.

CALL INIT3D (x,y,z,xa,ya,za,t,ds,sf,i)

x,y,z    (R): coordinates of view point (looking from)

xa,ya,za (R): coordinates of center point (looking to)

t        (R): rotation angle around line from (x,y,z) to

(xa,ya,za) in degrees CCW.

ds       (R): perspective scale factor (image size/viewing distance)

sf       (R): relative scale factor

i        (I): plotting flag ( -1 = do not plot, scaling only)

4.1.2  SUBROUTINE AXIS3D

AXIS3D plots an axis and its markings in 3-d.   In  order  to

draw  a coordinate system, the routine has to be called separately

for the x, y, and z axis.  A possible exponent is  determined  and

placed  behind  the  axis  label  in the form of 10**n in the auto

scaling mode (see AXIS3).  AXIS3D calls PLOT3D, NUM3D, and  SYM3D.

See also AXIS3.

CALL AXIS3D (x,y,x,a,b,g,s,n,ale,xm,xx,t,c,f)

x,y,z (R): location of start of axis

a,b   (R): angles from the x-y, x-z planes (in deg) of the ray

from (x,y,z) along the character string

g     (R): angle of rotation about the ray defined by a,b (deg)

s     (B): alpha string containing the axis title

n     (I): number of characters in the string

> 0 : axis labelling on positive side (anti-clockwise)

4-2

DESCRIPTION OF 3-D PLOTTING ROUTINES

< 0 : axis labelling on negative side (clockwise)

(100's digit) = 0 : axis is labeled

= 1 : line and ticks only--no labeling

ale   (R): length of axis

< 0 : tick marks placed on same side of axis as title

= 0 : no action

< 0 : tick marks placed opposite side of axis from title

xm    (R): value of first marking on the axis

xx    (R): value of last marking on the axis

t     (R): number of tick marks

specification is coded in the form MMM.mmss where

MMM is the number of major tick marks ( MMM > 0), mm is

the number of minor tick marks between major tick marks

(100 > mm => 0), and ss is the number of subminor tick

marks between minor tick marks (100 > ss => 0).

(example 1.0102 produces I_._._i_._._I)

c     (R): size of characters

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

f     (R): number label format (see NUMBER)

4.1.3  SUBROUTINE NUM3D

NUM3D plots a floating point number (see NUMBER) in 3-d using

PLOT3D.  The symbols are plotted in the plane defined by a,b,g.

CALL NUM3D (x,y,z,a,b,g,f,e)

x,y,z (R): lower-left corner of string

If x=999, y=999, z=999 then string is continued from

lower right of previous SYM3D or NUM3D call

a,b   (R): angles from the x-y, x-z planes (in deg) of the ray

from (x,y,z) along the base of the character string

g     (R): angle of rotation about the ray defined by a,b (deg)

h     (R): height of the number to be plotted

f     (R): number to be plotted

e     (R): format of number representation n.j

(see NUMBER for detailed description)

4.1.4  SUBROUTINE PLOT3D

PLOT3D is the 3-d  version  of  PLOT.   A  relative  rotation

matrix  and origin is maintained (separate from viewing matrix and

PLOT parameters).  By setting the plotting option flag i in INIT3D

to  -1,  plotting  will  be  inhibited.   A common block, CPLOT3D,

returns  a  4  element  vector  V  with  the  screen   transformed

coordinates.   PLOT3D  transforms  the  3d input coordinates to 2d

coordinates, clips to a 3d clipping window, and  calls  PLOT  with

the   2d  coordinates  screen  coordinates  of  the  visible  line

4-3

DESCRIPTION OF 3-D PLOTTING ROUTINES

segments.

CALL PLOT3D (x,y,z,i)

x,y,z (R): coordinates of point (in 3 space)

i     (I): plot function parameter

=  0: color control

x is the line color

if x < 0 the screen is cleared

if x >= 0 2d plot angle (PLOT) becomes y

= -1: change relative scale factor by x

=  1: change relative rotation matrix

rotate x degrees CCW around x axis

rotate y degrees CCW around y axis

rotate z degrees CCW around z axis

=  2: draw to (x,y,z) with 'pen down'

= -2: same as i=2. (x,y,z) becomes new origin

=  3: move to (x,y,z) with 'pen up'

= -3: same as i=3. (x,y,z) becomes new origin

=  9: erase to (x,y,z) (erase is color 0)

= -9: same as i=9. (x,y,z) becomes new origin

common /CPLOT3D/V(4) : returned screen coordinates (x,y) of last

call to PLOT3D  v(1)=x, v(2)=y

4.1.5  SUBROUTINE SYM3D

SYM3D plots an ASCII string (see SYMBOL) in 3-d using PLOT3D.

The symbols are plotted in the plane defined by a,b,g.

CALL SYM3D (x,y,z,a,b,g,s,n)

x,y,z (R): lower-left corner of string

If x=999, y=999, z=999 then string is continued from

lower right of previous SYM3D or NUM3D call

a,b   (R): angles from the x-y, x-z planes (in deg) of the ray

from (x,y,z) along the base of the character string

g     (R): angle of rotation about the ray defined by a,b (deg)

h     (R): height of the string to be plotted

s     (B): alpha array containing the text to be plotted (byte array)

n     (I): number of characters in string s

4.1.6  SUBROUTINE WHERE3D

WHERE3D returns the 2d screen coordinates of  the  last  line

drawn using PLOT3D.

CALL WHERE3D (x,y)

4-4

DESCRIPTION OF 3-D PLOTTING ROUTINES

x,y   (R): (returned) screen coordinates of last point

4.2  INIT3DH ROUTINES

The family of plotting routines included in the  hidden  line

3-d packge INIT3DH include:

1.  PLT3DH -- the  central  visible  line  plot  routine  for

INIT3DH

2.  SKETCH --  the  central  hidden  line  plot  routine  for

INIT3DH

3.  AXIS3DH -- plots axes using PLT3DH or SKETCH

4.  CUBE -- plots the polygons of the sides of a "cube"

5.  SYM3DH -- plots symbols using PLT3DH or SKETCH

6.  WHERE3H -- returns the screen  coordinates  of  the  last

PLT3DH call

7.  XFRM3D -- transforms a point (x,y,z) into the perspective

point defined by INIT3D.

The central hidden line plot routine SKETCH is an  adaptation

of  the  COSMIC  routine  ARC-11446 hidden line code.  It has been

modified and extended for additional features.  The INIT3DH can be

used either with or without the hidden line removal.  However, the

memory and computational requirements for hidden line removal  can

be large for complicated scenes.  Essentially, planar polygons are

defined and entered into the SKETCH internal list.  Upon the  last

call  to  SKETCH  the  visible lines are plotted using PLOT.  Line

segments can also be entered into the SKETCH  list  prior  to  the

last call.  They will not "hide" any other lines but can be hidden

by planes.  The SKETCH routine can be set to plot both visible and

invisible  lines.  PLT3DH does not use SKETCH.  Lines plotted with

PLT3DH will always be visible.   The  auxilary  routines  (NUM3DH,

AXIS3DH, and SYM3DH) have options to either use SKETCH or PLT3DH.

For an example of the use of  the  INIT3DH  package  see  the

EXAMP3DH  program  included  with  the  LONGLIB  graphics  library

package or see the MASTER routines TRIG3DH, HIST3D, or T3DH.

4.2.1  SUBROUTINE INIT3DH

INIT3DH sets the relative origin and scale factor of the  3-d

package  which  includes hidden line removal.  These functions are

distinct from the functions of PLOT or  PLOT3D.   INIT3DH  may  be

called  at  any  time to reset these functions without closing the

plot package.  However, a call  to  INIT3DH  resets  the  internal

SKETCH  list  of  polygons.   The plot package must be opened with

FRAME prior to a call INIT3DH.  INIT3DH must be  called  prior  to

SKETCH  or  PLT3DH.   SKETCH  uses  an  internal  common block for

4-5

DESCRIPTION OF 3-D PLOTTING ROUTINES

storage of its internal list.  The user  is  required  to  provide

sufficient  common  block  storage  space.   This  common block is

defined:

COMMON /GO/ ISIZE, WORK(ISIZE)

where

ISIZE >=(25 + 5*MNE + 4*(2+MNE+2*MNH))*NPOLYS

MNE   = maximum number of edges per polygon

MNH   = maximum number of edges per polygon

NPOLYS= maximum number of polygons

If SKETCH is not used, this common block need not be  dimensioned.

Note that ISIZE must be given a value in the user's code.

The angles ya,ro,pi are defined as the sequence of  rotations

ya-ro-pi  (yaw, roll, and pitch) where ya is the angle or rotation

about the z axis (positive is from x to y).  ro is  the  angle  of

rotation  about  the  x axis (positive is from y to z).  pi is the

angle of roation about the y axis (positive is from z to x).

CALL INIT3DH (x,y,z,ya,ro,pi,zom,ds,mne,mnh)

x,y,z    (R): coordinates of relative origin

ya,ro,pi (R): yaw, roll, pitch angles (Eulerian) in degrees

(see above for angle definitions)

zom      (R): relative scale factor

ds       (R): viewing distance (9999. = infinity or no pespective)

mne,mnh  (I): maximum number of edges and holes on one polygon

4.2.2  SUBROUTINE AXIS3DH

AXIS3DH plots an axis and its markings in 3-d.  In  order  to

draw  a coordinate system, the routine has to be called separately

for the x, y, and z axis.  A possible exponent is  determined  and

placed  behind the axis labelling in the form of 10**n in the auto

scaling mode.  AXIS3DH calls PLT3DH or SKETCH, NUM3DH, and SYM3DH.

See AXIS3.

CALL AXIS3DH (x,y,x,a,b,g,s,n,ale,xm,xx,t,c,f,i)

x,y,z (R): location of start of axis

a,b   (R): angles from the x-y, x-z planes (in deg) of the ray

from (x,y,z) along the character string

g     (R): angle of rotation about the ray defined by a,b (deg)

s     (B): alpha string containing the axis title

n     (I): number of characters in the string

> 0 : axis label on positive side (anti-clockwise)

< 0 : axis label on negative side (clockwise)

(100's digit) = 0 : labeled axis

= 1 : line and ticks only--no labeling

ale   (R): length of axis

4-6

DESCRIPTION OF 3-D PLOTTING ROUTINES

> 0 : tick marks placed same side of axis as title

= 0 : no action

< 0 : tick marks placed opposite side of axis from title

xm    (R): value of first marking on the axis

xx    (R): value of last marking on the axis

t     (R): number of tick marks

specification is coded in the form MMM.mmss where

MMM is the number of major tick marks ( MMM > 0), mm is

the number of minor tick marks between major tick marks

(100 > mm => 0), and ss is the number of subminor tick

marks between minor tick marks (100 > ss => 0).

(example 1.0102 produces I_._._i_._._I)

c     (R): size of characters

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

f     (R): number label format (see NUMBER)

i     (I): hidden line option flag

= 0 : use PLT3DH

= 1 : use SKETCH

4.2.3  SUBROUTINE CUBE

CUBE plots the 6 surface polygons of a 3-d  "cube"  into  the

INIT3DH  3-d  plotting  package.   Either  SKETCH or PLT3DH may be

used.

CALL CUBE (x1,x2,y1,y2,z1,z2,i,h)

x1,y1,z1 (R): corner of cube

x2,y2,z2 (R): oposite corner of "cube" from corner 1

i        (I): SKETCH flag for last SKETCH call in routine

Note: (returned) value is the SKETCH return

h        (L): hide flag

= .TRUE.   SKETCH routine used

= .FALSE.  PLT3DH routine used

4.2.4  SUBROUTINE NUM3DH

NUM3D plots a floating point number (see NUMBER) in 3-d using

either  PLT3DH  or  SKETCH.   The symbols are plotted in the plane

defined by a,b,g.

CALL NUM3DH (x,y,z,a,b,g,f,e,i)

x,y,z (R): lower-left corner of string

If x=999, y=999, z=999 then string is continued from

lower right of previous SYM3DH or NUM3DH call

a,b   (R): angles from the x-y, x-z planes (in deg) of the ray

from (x,y,z) along the base of the character string

4-7

DESCRIPTION OF 3-D PLOTTING ROUTINES

g     (R): angle of rotation about the ray defined by a,b (deg)

h     (R): height of the number to be plotted

f     (R): alpha array containing the text to be plotted (byte array)

e     (R): format of number representation n.j

(see NUMBER)

i     (I): hidden line option flag

= 0 : use PLT3DH

= 1 : use SKETCH

4.2.5  SUBROUTINE PLT3DH

PLT3DH is a routine which operates in a similar as  PLOT  but

in 3-d.  It is, however, independent of PLOT3D.  A relative origin

is maintained (separate from the PLOT routine parameters) which is

initalized  by  INIT3DH.   PLT3DH  calls  PLOT  after transforming

(x,y,z).  PLT3DH is also independent of the SKETCH routine.  Lines

plotted  with  PLT3DH  are  always completely visible.  To use the

hidden line removal SKETCH must be used.

CALL PLT3DH (x,y,z,i)

x,y,z (R): coordinates of point (in 3 space)

i     (I): plot function parameter

=  0: line color control

x is the line color

if x < 0 screen is cleared

if x >= 0 plotting angle (in PLOT) becomes y

=  2: draw to (x,y,z) with 'pen down'

= -2: same as i=2. Point (x,y,z) becomes new origin

=  3: move to (x,y,z) with 'pen up'

= -3: same as i=3. Point (x,y,z) becomes new origin

=  9: erase to (x,y,z) (erase on Ramtek is color 0)

= -9: erase to (x,y,z) becomes new origin

4.2.6  SUBROUTINE SKETCH

SKETCH is the central plot routine for  hidden  line  removal

3-d  plotting.  SKETCH is called for each polygon or line segment.

Each polygon or line segment is described by a set  of  points  in

(x,y,z) arrays.  For polygons, these should be in either clockwise

or counter-clockwise direction with the last point  equal  to  the

first.  Holes in polygons are created by continuing the last outer

edge point (which is also the first outer edge point) to the inner

hole  edges and back.  The option flag, i, should be 0 for all but

the last polygon.  When i=0, SKETCH  stores  the  input  polygons.

When  i  is  set  to  be  1, the resulting scene visible lines are

computed and plotted by calls to  PLOT.   This  process  may  take

considerable  cpu  time  as  well as memory storage space when the

number of polygons is large.  The maximum number  of  edges  of  a

4-8

DESCRIPTION OF 3-D PLOTTING ROUTINES

polygon is 160.

Note that SKETCH requires a large working  area  in  a  named

COMMON  area  described  in INIT3DH.  Note SKETCH will fail if the

memory limits are exceeded.  For the most part an error flag  will

be set.  Ocassionally, the scene will be corrupted when the memory

limits are exceeded without the error flag being set.

CALL SKETCH (x,y,z,np,i)

x,y,z   (R): arrays contain polygon edges (max 160)

np      (I): number of points in x,y, and z

i       (I): option flag.  Should be 0 for all input polygons

except the last, when it should be 1.

(Note: if i+10 is used for i then the first z value

is used for all (x,y) points.)

i set to -1 (returned) if a memory error occurs

4.2.7  SUBROUTINE SYM3DH

SYM3DH plots an ASCII string (see SYMBOL) in 3-d using either

PLT3DH or SKETCH.  The symbols are plotted in the plane defined by

a,b,g.

CALL SYM3DH (x,y,z,a,b,g,s,n,i)

x,y,z (R): lower-left corner of string

If x=999, y=999, z=999 then string is continued from

lower right of previous SYM3DH or NUM3DH call

a,b   (R): angles from the x-y, x-z planes (in deg) of the ray

from (x,y,z) along the base of the character string

g     (R): angle of rotation about the ray defined by a,b (deg)

h     (R): height of the string to be plotted in plot units

s     (B): alpha array containing the text to be plotted

n     (I): number of characters in string s

i     (I): hidden line option flag

= 0 : use PLT3DH

= 1 : use SKETCH

4.2.8  SUBROUTINE WHERE3H

WHERE3H returns the 2d screen coordinates of the  last  point

in a call to PLT3DH.

CALL WHERE3H (x,y)

x,y      (R): screen coordinates of last point of call to PLT3DH

4-9

DESCRIPTION OF 3-D PLOTTING ROUTINES

4.2.9  SUBROUTINE XFRM3D

XFRM3D uses the same scaling and origin as PLT3DH and  SKETCH

to transform a 3-d points to screen (PLOT) coordinates.

CALL XFRM3D (x,y,z,x1,y1,z1)

x,y,z    (R): input coordinates of point (in 3 space)

x1,y1,z1 (R): (returned) transformed coordinates of point (in 3 space)

4-10

CHAPTER 5

CURSOR ROUTINES

The routines described in this  chapter  provide  interactive

cursor  control.   When  supported  by the terminal, the Tektronix

Graphics Inputs (GIN), BITCURSOR or  GETCURSOR,  routines  can  be

used.   If  the  terminal  has  a  VT100-compatible text mode, the

routines CURMOTION, CURRECT, and CURBAND, can be used to  simulate

a  GIN  device.   These  routines  use the VT100 keypad and cursor

keys.  They rely on a machine-dependent routine (INXTCHR) to  read

escape  characters  from  the  terminal.   CURMOTION, CURRECT, and

CURBAND return the internal screen "resolution" used in  computing

the location of the cursor.  This may not correspond to the actual

hardware resolution of the terminal screen.

The routine CURLOCATE provides  a  technique  for  placing  a

fixed "cursor mark" on the screen.

A program CURTEST is provided  to  test  and  evaluate  these

cursor routines.

5.1  SUBROUTINE BITCURSOR

BITCURSOR moves a cross-hair cursor on a VT100 equipped  with

a  retro-graphics  card (VT125) and a BIT PAD ONE graphics tablet.

The VT125 is used in the Tek  4010  mode  with  a  graphics  point

returned when a key is pressed on the bit pad puck (or stylus).

CALL BITCURSOR (x,y,k,rx,ry)

x,y (R): (returned) selected cursor position (in plot units)

k   (I): (returned) key code

= 0  (Z) key pressed on bit pad puck

= 1  (1) key pressed on bit pad puck

= 2  (2) key pressed on bit pad puck

= 3  (3) key pressed on bit pad puck

rx  (R): resolution of screen in x direction (returned)

ry  (R): resolution of screen in y direction (returned)

5-1

CURSOR ROUTINES

5.2  SUBROUTINE CURLOCATE

CURLOCATE produces a simulated "x" cursor mark on the  screen

graphics device (Ramtek or terminal device) When only the metafile

is initialized a call to CURLOCATE is a dummy call.  The  terminal

takes precidence over the Ramtek when both are in use.  Erasure of

cursor on  requires  the  correct  location  where  it  was  first

plotted.  CURLOCATE uses the XOR capbility of graphics terminal to

place and remove the cursor.  If the terminal has neither erase or

XOR  capability,  the  graphics  cursor  can not be erased without

clearing the entire screen.  If the  (x,y)  position  is  off  the

screen,  the  cursor will be located on closest edge of the screen

to the desired point.

CALL CURLOCATE (x,y,n,ir)

x,y (R): cursor position

n   (I): cursor number/size (0-4)

< 0  erase cursor

> 0  locate cursor

ir  (I): Ramtek cursor control (recognized if Ramtek is output)

= 0 Ramtek cursor device used

= 1 plotted Ramtek cursor mark used

5.3  SUBROUTINE CURMOTION

CURMOTION  produces  a  graphics  cursor  on  the  Ramtek  or

terminal  When  no  screen  devices  are  initialized  a  call  to

CURMOTION is a dummy call.  Terminal  is  used  in  preference  to

Ramtek.   This  routine  is  supported only on terminals which can

emulate the VT100 numeric keypad.  The terminal  text  cursor  and

VT100  numeric keypad keys are used to move the graphics cursor to

a desired location and a return function key is pressed to  select

the  cursor  location.   Only  <space>,  <return>, PF keys and the

numeric key pad keys will be recognized as  return  command  keys.

PF1 changes the cursor step movement size in three sizes.  As this

happens the cursor  changes  sizes  on  the  screen.   The  cursor

position  is  typed  to the terminal when the Ramtek is used.  The

cursor is moved in multiples of the pixel resolution.  The formula

below shows the conversion.  All other PF keys and the numeric key

pad keys will return the arguments shown below.   Other  keys  are

not  recognized.   NOTE:   The input buffer is 128 characters.  If

you exceed this buffer the program  may  bomb.   Each  cursor  key

input uses 3 characters.

pixel number = (scalefactor * x + origin)/pixelresolution

CALL CURMOTION (x,y,is,rx,ry)

x,y (R): (returned) selected cursor position

is  (I): (returned) status flag

5-2

CURSOR ROUTINES

< 0  error

= 0  return key pressed

= 1  space key pressed

= 2,3,4 PF2,PF3,PF4 keys on VT100 pressed

= 10...19 VT100 numeric key pad keys 0...9 pressed

= 20 numeric key pad period key pressed

= 21 numeric key pad enter key pressed

= 22 numeric key pad comma key pressed

= 23 numeric key pad dash key pressed

rx  (B): resolution of screen in x direction (returned)

ry  (R): resolution of screen in y direction (returned)

5.4  SUBROUTINE CURBAND

CURBAND is similar to the CURMOTION  subroutine  except  that

two  lines  are  "rubber banded" with the simulated cursor motion.

This routine is supported only on terminals which can emulate  the

VT100  numeric  keypad.   The  line  segment  rubberbanding can be

disabled if desired.

CALL CURBAND (x,y,is,rx,ry,x1,y2,x2,y2)

x,y   (R): (returned) selected cursor position

is    (I): (returned) status flag

< 0  error

= 0  return key pressed

= 1  space key pressed

= 2,3,4 PF2,PF3,PF4 keys on VT100 pressed

= 10...19 VT100 numeric key pad keys 0...9 pressed

= 20 numeric key pad period key pressed

= 21 numeric key pad enter key pressed

= 22 numeric key pad comma key pressed

= 23 numeric key pad dash key pressed

rx    (R): resolution of screen in x direction (returned)

ry    (R): resolution of screen in y direction (returned)

x1,y1 (R): starting point of rubber banded line 1

if x1=999, this line segment is not used

x2,y2 (R): starting point of rubber banded line 2

if x2=999, this line segment is not used

5.5  SUBROUTINE CURRECT

CURRECT is similar to the CURBAND subroutine  except  that  a

rectangle  is moved with cursor motion.  This routine is supported

only on terminals which can emulate the VT100 numeric keypad.

CALL CURRECT (x,y,is,rx,ry,x1,y2,x2,y2)

x,y   (R): (returned) selected cursor position/start position

5-3

CURSOR ROUTINES

is    (I): (returned) status flag

< 0  error

= 0  return key pressed

= 1  space key pressed

= 2,3,4 PF2,PF3,PF4 keys on VT100 pressed

= 10...19 VT100 numeric key pad keys 0...9 pressed

= 20 numeric key pad period key pressed

= 21 numeric key pad enter key pressed

= 22 numeric key pad comma key pressed

= 23 numeric key pad dash key pressed

rx    (R): resolution of screen in x direction (returned)

ry    (R): resolution of screen in y direction (returned)

x1,y1 (R): lower left corner of rectangle

x2,y2 (R): upper right corner of rectangle

5.6  SUBROUTINE GETCURSOR

GETCURSOR inquires the Tektronix GIN device for the  location

pointed  to  by  the  GIN  device.   Return  code is screen device

dependent.  This routine is does support all screen devices.  When

a  device  is  not  supported,  the  routine returns without doing

anything.  Screen device must be in the graphics mode.   GETCURSOR

assumes  that the GIN terminator is a CR (carriage return).  Note:

When using an Alpha key for the  return  status  DO  NOT  use  the

<Return>  key  as  this  will  prevent  the correct reading of the

returned data.

CALL GETCURSOR (x,y,k,rx,ry)

x,y (R): (returned) cross-hair location (in LONGLIB coordinates)

k   (I): (returned) GIN status code return (ascii code)

rx  (R): resolution of screen in x direction (returned)

ry  (R): resolution of screen in y direction (returned)

5-4

CHAPTER 6

MAP SUBROUTINES

LONGLIB also supports the plotting of maps  of  the  physical

earth's  surface  by  providing  map  routines.   Two  data  files

(EARTH.DAT and LNDSEA1.DAT) are included in the  LONGLIB  graphics

library  package.   EARTH.DAT  contains the digitized locations of

the edges of the earth's landmasses.  It forms the basis of a  set

of  EARTH  OUTLINE  map  routines  discussed  below.   LNDSEA1.DAT

contains a bit map of land/sea areas.  It forms the basis  of  the

LAND AREA map routines discusses in the second section.

Note:  these  routines  assume  that  the  plot  package  has

already been opened.

6.1  EARTH OUTLINE MAPS

To date only a small number of routines have  been  generated

for  using  this  data  file but more will be added in the future.

Current routines allow for plotting the earth outline map in 3d or

in a linear projection.

A  data  file  (EARTH.DAT)  which  contains  a  map  of   the

land/ocean interface and a set of routines to access this data has

been included in the LONGLIB graphics library.  The data file  (of

unknown origin) contains a list of latitudes and longitudes of the

land/ocean edges at about a  10  km  resolution.   It  is  only  a

geographic map and does not contain political boundries.  Although

imperfect, it is more than adequate for map drawing, etc.  Due  to

the  high  resolution,  the  EARTH.DAT  file contains a lot of pen

motions, requiring a long plotting time.

Fortran file unit 2 should be reserved for accessing the  map

data  file  by these routines.  The logical name LONGLOC:  must be

assigned to be the location of the EARTH.DAT file.

Routines using  the  land  outline  file  include  those  for

plotting on 3d spheres, flat surfaces, etc.

6-1

MAP SUBROUTINES

6.1.1  SUBROUTINE EARTH3D

EARTH3D permits 3d plotting of  the  earth  land  map.   This

routine  uses  the INIT3D and PLOT3D 3d graphics package.  It will

plot the entire earth map with the option of  either  a  spherical

earth  or an ellipsoidal earth.  The radius may be specified.  The

transformation from a latitude/longitude pair (a,b) in radians  on

the earth's suface to the 3d plotting vector v is:

rad = r * (1 - f*sin(a))

call sprect1(v,b,a-pi/2,rad)

call plot3d(v(1),v(2),v(3),2)

where

pi = 3.141592654

v is dimensioned v(3)

CALL EARTH3D(r,f)

r   (R): nominal earth radius (in plotting units)

f   (R): earth flatness

= 0 for spherical earth

= 3.3528132e-3 for an ellipsoidal earth

6.1.2  SUBROUTINE LANDMAP

LANDMAP plots the earth land map using a  linear  projection.

The  latitude  and  longitude are assumed to be a linear grid on a

flat surface.  This routine will plot the entire map surface.  Use

of  the  PLOT  routine  clipping  option will permit plotting only

limited of the map surface if desired.  The  transformation  of  a

latitude/longitude  pair  (a,b)  in  degrees  to an (x,y) pair for

plotting is:

x = (b - s) * long / 360

y = (a + 90) * lat / 180

CALL LANDMAP(alat,along,s)

alat,along (R): latitude, longitude scale factor (degrees/plot unit)

s          (R): longitude of left-mode edge of map (-180 to +180)

6.1.3  SUBROUTINE POLARMAP

POLARMAP plots the earth land map using a  polar  projection.

The  latitude  is  plotted  as a linear radius.  This routine will

plot  the   entire   northern   or   southern   hemisphere.    The

transformation   from   a   visible   point  (in  the  appropriate

hemisphere) latitude/longitude pair (a,b) in degrees to a  plotted

(x,y) pair is:

6-2

MAP SUBROUTINES

a = b * sgn(r) + a

x = cos( a * pi / 180) * r / 90 + x0

y = sin( a * pi / 180) * r / 90 + y0

where

pi = 3.141592654

CALL POLARMAP(x0,y0,r,a)

x0,y0  (R): pole location (in plot units)

r      (R): radius of equator (in plot units)

> northern hemisphere

< southern hemisphere

a      (R): angle of prime meridian from horizontal (deg CCW)

6.1.4  SUBROUTINE SPRECT1

SPRECT1  converts  a  spherical  coordinate   value   (in   a

latitude/longitude   style   spherical   system)   to  rectangular

coordinates.  The transformation from  a  latitude/longitude  pair

(a,b) in degrees to a rectangular (x,y) pair is:

a = b * sgn(r) + a

x = cos( a * pi / 180) * r / 90 + x0

y = sin( a * pi / 180) * r / 90 + y0

where

pi = 3.141592654

CALL SPRECT1(v,t,p,r)

v  (R): output vector containing rectangular (z,y,z) coordinates

dimensioned v(3) (returned)

t  (R): theta (longitude) angle (rad)

p  (R): phi (latitude) angle (rad)

r  (R): radius

6.2  LAND AREA MAP ROUTINES

The LNDSEA1.DAT file contains a bit map of the land/sea  area

of  the  earth.  Using the file, a specified point of latitude and

longitude can be determined to be land or sea.  A routine, LNDSEA,

opens  the  file  and provides a flag to indicate if the specified

point is land or sea.

Fortran file unit 1 should be reserved for accessing the  map

data  file  by these routines.  The logical name LONGLOC:  must be

assigned to be the directory containing the LNDSEA1.DAT file.

6-3

MAP SUBROUTINES

6.2.1  LNDSEA1.DAT Format

The LNDSEA1.DAT is a direct access file is a  world  land/sea

map quantized to every 1/12 degree of both latitude and longitude.

An individual bit is used to indicate whether a  particular  point

is  land  or sea.  The data is stored as 648 records each of which

contains all of the data for a 10  degree  by  10  degree  square.

Each  record  consists  of 14400 (120*120) bits stored 30 bits per

word (4 words per each 1/12 degree strip of data).  The first word

of  the record indicates whether the entire 10 by 10 square is all

land or water using the following definition:

-1 : square contains both land and sea

0 : square contains all land

1 : square contains all water

The next 4 words in each record are the bits for the  bottom  1/12

degree  (lowest latitude) row of the 10 degree by 10 degree square

with longitude bins left to right.  For each  bit  a  0  indicates

land and a 1 indicates water.  The records are ordered:

record #  Latitude range  Longitude range

_________ _______________ _________________

1        -90 to -80         0 to  10

2        -90 to -80        10 to  20

.            .                .

35        -90 to -80       340 to 350

36        -90 to -80       350 to 360

37        -80 to -70         0 to  10

38        -80 to -70        10 to  20

.            .                .

648         80 to  90       350 to 360

See the  source  code  for  LNDSEA  function  for  additional

information.

6.2.2  SUBROUTINE BITMAP

BITMAP plots a land area by testing each point  of  the  area

using  LNDSEA.   The  plotting  area  is  segmented into (nx X ny)

regions.  Each point is tested for the presence of land/sea.   For

each  resolutoin  line  of  latitude,  a  horizontal line is drawn

through all points that are land (or sea as  desired).   A  linear

projection  is used.  The latitude and longitude are assumed to be

a linear grid on a flat  surface.   This  routine  will  plot  the

entire  map surface.  Use of the PLOT routine clipping option will

permit plotting only limited of the map surface if  desired.   The

transformation of a latitude/longitude pair (a,b) in degrees to an

(x,y) pair for plotting is:

6-4

MAP SUBROUTINES

x = (b - s) * along / 360

y = (a + 90) * alat / 180

CALL BITMAP(alat,along,s,nx,ny,i)

alat,along (R): latitude, longitude axis length (plot units)

s          (R): longitude of left-mode edge of map (-180 to +180)

nx,ny      (I): x,y resolution specified as the number of lat/longs

to test for land/sea

i          (I): plot flag

= 0 plot land area

= 1 plot sea area

6.2.3  INTEGER FUNCTION LNDSEA

LNDSEA tests the point (lat, long)  for  land/sea  using  the

LNDSEA1.DAT  file.   It  returns  a  0  for land, 1 for sea.  Uses

Fortran file unit 1.

iflag = LNDSEA(alat,along)

alat  (R): latitude (-90. to +90. degrees)

along (R): longitude (0. to +360. degrees)

iflag (I): land/sea flag (returned)

= -1 : error

=  0 : land

=  1 : sea (ocean, lake, sea)

6-5

CHAPTER 7

MASTER SUBROUTINES

By  popular  request,  a  set  of  the  most  commonly   used

general-purpose  subroutines  for complete function plots, charts,

etc., were  included  in  the  LONGLIB  graphics  library.   These

subroutines  are  called  "MASTER"  Subroutines.   Each  of  these

subroutines is self contained.  When called,  it  will  initialize

the  plotting  package,  plot  the  required  data,  and close the

plotting package.  Normally, only one MASTER subroutine is  called

in  a  program.  However, options are available for multiple calls

to MASTER subroutines.  When  the  LONGLIB  is  initialized  by  a

MASTER  subroutine  (which  calls  FRAME) a metafile using Fortran

file unit 3 is always created.

Programming examples using MASTER routines are given  in  the

chapter on programming examples.

To call a MASTER subroutine more than once in a  program  the

option  flag  must  be set negative for all calls but the last one

which should be positive.  (See also the  PLOTTESTS  program.)  To

use more than one MASTER subroutine in a program:

(1)  On first MASTER  subroutine  call  set  the  option  flag

negative--this does not close plot package.

(2)  on all additional calls to  MASTER  subroutines  set  the

option  negative  and  greater  than 10000--this prevents

re-opening plot package and does not close it.

(3)  On the last call to a MASTER subroutine set  option  flag

positive but greater than zero--this closes plot package.

In summary, when LONGLIB is already open set the magnitude of

the  option  flag  to  greater than 10000.  To prevent closing the

plot package set option flag negative.  When the plot  package  is

closed  in  a  MASTER  subroutine  CTERM(-2)  is  used to ask if a

termina lscreen clear (terminal plotting  only)  should  be  done.

MASTER  subroutines  always  return  the terminal to the text mode

after call.  When color options  are  enabled,  the  color  marked

7-1

MASTER SUBROUTINES

"(return)" is the color in current use after call.

Note:  when passing varables ASCII strings  do  not  use  VAX

FORTRAN  CHARACTER  data  type.   Use BYTE data type arrays or use

%REF().  See the comments under the section on machine  dependency

and the introduction to the plot routine section.

Most MASTER subroutines have common blocks with the same name

prefixed with the letter "C" which contain the scaling information

used to plot the data line.   This  can  be  useful  for  plotting

additional annotations, etc.

The following pages document  the  currently  defined  MASTER

Subroutines.   Additional subroutines may be added as suggested by

the users.

7.1  MASTER ROUTINE INDEX

The sections that follow detail each MASTER routine.  A brief

index  of  the capabilities of each Master routine is given below.

To select a MASTER routine, determine  the  category  of  plotting

needed and look at the available MASTER routine capabilities.  For

general purpose line plotting where flexibility in specifying  the

axes, the routine GLPLOT is recommended.

7.1.1  Pie Chart

1.  PICHRT

7.1.2  Bar Chart

1.  BARCHR (can fill area between lines)

7.1.3  Single Linear/linear Line

1.  PLOTSC (simple)

2.  GLPLOT (flexible axis specification)

3.  PLOTLGXL (allows log lines, software line types)

4.  LSPLOT (options via array)

7-2

MASTER SUBROUTINES

7.1.4  Two Linear/linear Lines On The Same Plot

1.  PLOTSC2 (simple)

2.  GLPLOT (flexible axis specification)

3.  PLOTLG2 (allows log lines)

4.  PLOTLGX (allows log lines, flexible axis specification)

5.  PLOTLGXL (allows log lines, software line types)

6.  BARCHR (can fill area between lines)

7.  LSPLOT (options via array)

7.1.5  Multiple Linear/linear Lines On The Same Plot

1.  GLPLOT (more complex but flexible axis specification)

2.  PLOTLGX (allows log lines, flexible axis specification)

3.  PLOTLGXL (software line types)

4.  SEISPL (special options)

5.  SPLOTS (can show error bars)

6.  SPLOTSX (can show error bars with flexible axis)

7.  BARCHR (can fill area between lines)

8.  LSPLOT (options via array)

7.1.6  Multiple Log/linear Or Log/log Lines On The Same Plot

1.  PLOTLG (relatively simple)

2.  PLOTLGX (more complex, flexible axis specification)

3.  LSPLOT (options via array)

7.1.7  Scatter Plot

1.  SCATPL

2.  SPLOTS (can show error bars)

3.  SPLOTSX (can show error bars with flexible axis)

4.  SEISPL (special options)

5.  LSPLOT (options via array)

7.1.8  1-d Histogram

1.  PHIST (can show mean/standard deviation)

2.  BARCHR (bar chart)

7-3

MASTER SUBROUTINES

7.1.9  Lines/points With Error Bars

1.  SPLOTS (simple)

2.  SPLOTSX (flexible axis specification)

3.  LSPLOT (options via array)

7.1.10  Special Plot Formats

1.  SEISPL (forms used in seismic data plots)

2.  BARCHR (bar chart)

3.  PICHRT (pie chart)

4.  LSPLOT (options via array)

7.1.11  Contour Plot With Equally Spaced Data

1.  CNTRLN (simple, robust)

2.  LCNTR (simple, less-robust, with software line types)

7.1.12  Contour Plots With Unequally Spaced Data

1.  CNTLN (triangulates points then contours)

7.1.13  3-d Surface Slices

1.  VAX3D (simple)

2.  VAX3DX (more complex, with flexible axis specification)

7.1.14  3-d Surface Mesh (no Hidden Line Removal):

1.  MESH3D (simple)

2.  MESH3DX (more complex, with flexible axis specification)

7-4

MASTER SUBROUTINES

7.1.15  3-d Surface Mesh (hidden Line Removal)

1.  MVAX3D (simple)

2.  MVAX3DX (more complex, with flexible axis specification)

7.1.16  3-d Surface Mesh With Contour Plot (hidden Line Removal)

1.  CVAX3D (simple)

2.  CVAX3DX (more complex, with flexible axis specification)

7.1.17  3-d Surface Triangular Mesh (hidden Line Removal)

1.  T3DH (uses INIT3DH)

7.1.18  Unequally Sampled 3-d Surface (hidden Line Removal):

1.  TRIG3DH (uses INIT3DH)

7.1.19  3-d Histogram With Hidden Line Removal

1.  HIST3D (uses INIT3DH)

2.  MVAX3D (simple)

3.  MVAX3DX (more complex, with flexible axis specification)

4.  CVAX3D (simple, can include contour plot)

5.  CVAX3DX (with flexible axis specification,  with  contour

plot)

7.1.20  3-d Contour Plot With Equally Spaced Data (no Hidden  Line

Removal)

1.  CNT3D (simple, robust)

2.  CNT3DX (more complex, with flexible axis specification)

7-5

MASTER SUBROUTINES

7.1.21  4/5-d Surface Plots:

1.  VAX5D (slices)

2.  MVAX5D (mesh/histogram with hidden line removal)

7-6

MASTER SUBROUTINES

7.2  SUBROUTINE BARCHR

BARCHR plots a bar chart with optionally shaded bar  segments

and descriptive legends.  In addition, multiple lines with shading

between lines can be plotted.  Legend can be automatically  placed

or  the  user  can specify the location of the legend.  In the bar

chart mode, bars  can  run  vertically  or  horizontally.   b(i,j)

specifies  the  ith bar and jth segment of bar (or horizontal line

depending on iflag option).  Top of jth segment or height  of  jth

line is computed from:

j

yplotted = ylen * (Sum b(i,k) - bm)/(bx-bm)

k=1

CALL BARCHR(b,nb,ns,sh,iflag,xl,yl,bm,bx,f,nd,sp,sl,nsl,bl,nbl,cs

t,nt,bt,nbt,lt,nlt,tcs,a,d,<p,<c>>)

b     (R): bar data array dimensioned b(nb,ns)

nb    (I): number of bars/number of points in each line

ns    (I): number of segments in each bar/number of lines

sh    (I): shade option for segment/area between lines

dimensioned sh(ns)

sh            shade pattern

____       _________________________

0               no shading

1          -45 deg solid lines

2          horizontal solid lines

3          +45 deg solid lines

4          vertical deg solid lines

5          -45 deg dotted lines

6          horizontal dotted lines

7          +45 deg dotted lines

8          vertical deg dotted lines

9          +/- 45 deg dotted lines

10          vertical/horizontal dotted lines

11          +/- 45 deg solid lines

12          vertical/horizontal solid lines

iflag (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit)  = 1 : color array not used

= 2 : color array used

(10's)       = 0 : bar chart with vertical bars

= 1 : bar chart with horzontal bars

= 2 : multiple line chart

(100's)      = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

xlen  (R): length of horizontal axis

< 0 : chart is enclosed in a box

> 0 : only bottom and left axes plotted

7-7

MASTER SUBROUTINES

ylen  (R): length of vertical axis

bm,bx (R): minimum and maximum values to be shown on chart

note: if bx=bm, values computed from b array will be used

f     (R): format for numeric labels on axis (see NUMBER)

nd    (I): number of divsions of bar length axis

< 0 : division lines shown on chart

= 0 : no division lines or numeric labels

> 0 : division lines shown

sp    (R): width of bar (ignored for line plot)

= 0 : auto scaling with evenly spaced bars

> 0 : bars grouped in groups of int(sp).  Each

bar has width frac(sp).

sl    (C): segment legend labels (CHARACTER data type)

dimensioned sl(ns)

nsl   (I): number of characters to use in plotting sl's

= 0 : no label plotted

bl    (C): bar labels (CHARACTER data type) dimensioned bl(nb)

nbl   (I): number of characters to use in plotting bl's

= 0 : no label plotted

cs    (R): legend/bar label character height

t     (B): title string placed on top of chart

nt    (I): number of characters in t

= 0 : no label plotted

bt    (B): title string placed at base of bars

nbt   (I): number of characters in bt

= 0 : bt not plotted

lt    (B): title string placed on bar length axis

nlt   (I): number of characters in lt

= 0 : lt not plotted

tcs   (R): height of title strings

a     (R): legend location/shading box size dimensioned a(3)

a(1) : legend box size

< 0 : legend placed to right of chart, a(2)

and a(3) are not used

= 0 : no legend

> 0 : a(2) and a(3) used to locate legend

a(2) : x position of lower left corner of legend

a(3) : y position of lower left corner of legend

d     (R): distance between shading lines

< 0 : line width array used

> 0 : line width array not used

p     (R): line width array (used only if d<0)

p(1) : axis line width

p(2) : division line width

p(3) : bar outline/data line width

p(4) : labeling line width

p(5) : title line width (bt,lt)

p(6) : title line width (t)

c     (I): color array (used only if mod(|iflag|,10)=2)

c(1) : t title color (return)

c(2) : bt title color

c(3) : lt title color

c(4) : axis numberic label colors

7-8

MASTER SUBROUTINES

c(5) : sl label color

c(6) : bl label color

c(7) : segment 1 color

c(8) : segment 2 color

...         ...

7-9

MASTER SUBROUTINES

7.3  SUBROUTINE CNT3D

CNT3D plots a simple 3-d contour  of  equally  spaced  points

with  no  hidden  line  removal.   (see CNT3DX) CNT3D calls CNT3DX

using default axis parameters to simplify calling procedure.

CALL CNT3D(d,ndx,ndz,nx,nz,a,b,xh,yh,zh,nl,as,ae,ie,iflag,iax,

<xt,nxt,xs,xe,yt,nyt,zt,nzt,zs,ze,<dm,dx<,ic>,l>>>)

See CNT3DX for parameter description.

(iax is limited to a single digit value)

7.4  SUBROUTINE CNT3DX

CNT3DX is a simple  3-d  contour  plotting  routine.   A  3-d

surface  is  contoured  by  plotting  slices  through  the surface

parallel to the x-z plane of the surface which  have  the  same  y

value.   The  input  consists of a 2 dimensional grid of y values.

For each contour level the input array is scanned cell-by-cell.  A

segment  of  the  contour is determined by linearily interpolating

the edges of the square formed by 4 adjacent points (a cell).  For

example,  if  the  current  contour  value  is  1,  and  y(1,1)=0,

y(1,2)=2, y(2,2)=3, and y(1,2)=4, a contour  line  is  assumed  to

exist for this cell as shown:

y(1,2)      y(2,2)

*          *

+

\

* +        *

y(1,1)      y(1,2)

This line segment is plotted using the same  approach  as  VAX3DX.

No  hidden  line  removal  is  provided.   The calling sequence is

nearly identical for  both  CNT3DX  and  VAX3DX.   The  height  of

plotted contours relative to the y axis is calibrated to z axis so

that scale can be taken from the plot.  No  perspective  is  used.

Options  exist  to  vary  the  plotting  angle  and  to plot axes.

Contour values can be distinguished by color and/or line type.

Origin of the plot is in the lower-left corner.  The  x  axis

runs  plotted  left to right along the plot bottom.  The y axis is

plotted as a vertical displacement offset by  the  z  axis  value.

The  z  axis  appears  to  point  into the screen.  This gives the

illusion of depth in the plot.  See AXIS2 for detailed discription

of axis parameters.

The pathological case of two contour lines within a cell  may

case  the  routine  to  incorrectly trace the contour through that

cell.

7-10

MASTER SUBROUTINES

CALL CNT3DX(d,ndx,ndz,nx,nz,a,b,xh,yh,zh,nl,as,ae,ie,iflag,iax,

<xt,nxt,xs,xe,nmx,nnx,mlx,tsx,ndx,smx,

yt,nyt,nmy,nny,mly,tsy,ndy,smy,

zt,nzt,zs,ze,nmz,nnz,mlz,tsz,ndz,smz,

<dm,dx<,ic>,l>>>)

d        (R): array of y values dimensioned d(ndx,ndz)

ndx,ndz  (I): x and z dimensions of d array

nx,nz    (I): x and z sizes of surface to plot d array

a        (R): angle of x axis from horizontal 0-85 degrees

b        (R): angle of z axis from horizontal 0-90 degrees

note: origin d(1,1) is in lower-left corner

x axis runs left to right on screen

y axis runs up to down on screen

z axis appears to run into the screen but is angled

to the right

xh,yh,zh (R): length of each axis

nl    (I): number of uniformly spaced contour levels,

< 0 : max and min of v are used for as, ae

(j)th contour is (j-1)*(ae-as)/(nl-1)+as

= 0 : int(ae) specifies the number of contour values

where as is an array of the contour values

> 0 : number of uniformly space contour levels,

(j)th contour is (j-1)*(ae-as)/(nl-1)+as

as    (R): first contour level (nl > 0)

array of contour levels (nl=0) dimensioned as(int(ae))

ae    (R): last contour level (nl > 0)

number of contour levels in as (nl=0) ae>0

ie    (I): contour edge option flag

< 0 contour edge added when surface below contour

= 0 no contour edges added

> 0 contour edge added when surface above contour

iflag (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude)  >10000: do not initialize LONGLIB before plotting

(1's digit) = 1 : ignor color and line type arrays

2 : use color array but not line type array

3 : ignore color array, use line type array

4 : use color and line type arrays

(10's digit)= 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

iax   (I): axis format control

< 0 : plot axis, using input scale factors dm and dx

= 0 : do not plot axis, optional axis parameters not used

input scaling is computed from input array

> 0 : plot axis, using scaling computed from input array,

need optional axis parameters

(1's digit)  = 1 : Plot actual max/min or input values for Y axis

= 2 : Plot smoothed values for Y axis

(10's digit) = 0 : Use default axis type

= 1 : Use input AXIS2-type axis parameters

7-11

MASTER SUBROUTINES

(NOTE: the following optional parameters are used only if iax<0 or

mod(iflag,10)=1)

xt    (B): title of x axis (width)

nxt   (I): number of characters in xt

= 0 : no axis plotted

> 0 : normal

xs,xe (R): starting and ending values displayed on x axis

(see AXIS2 for detailed description of axis parameters)

nmx   (I): number of minor ticks between major ticks on x axis

nnx   (I): highlight length of nnx-th minor tick on x axis

mlx   (I): number of major tick marks on x axis

tsx   (R): size of title and numbers on x axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

ndx   (I): number of digits to right of decimal point on x axis

smx   (R): major tick length on x axis

yt    (B): title of y axis (depth)

nyt   (I): number of characters in yt

= 0 : no y axis plotted

> 0 : normal

nmy   (I): number of minor ticks between major ticks on y axis

nny   (I): highlight length of nny-th minor tick on y axis

mly   (I): number of major tick marks on y axis

tsy   (R): size of title and numbers on y axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

ndy   (I): number of digits to right of decimal point on y axis

smy   (R): major tick length on y axis

zt    (B): title of z axis (height)

nzt   (I): number of characters in zt

= 0 : no z axis plotted

> 0 : normal

ze,ze (R): starting and ending valued displayed on z axis

nmz   (I): number of minor ticks between major ticks on z axis

nnz   (I): highlight length of nnz-th minor tick on z axis

mlz   (I): number of major tick marks on z axis

tsz   (R): size of title and numbers on z axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

ndz   (I): number of digits to right of decimal point on z axis

smz   (R): major tick length on z axis

(NOTE: the following are required only if iax<0 or mod(iflag,10)<>0)

dm,dx (R): minimum and maximum values of d array

(NOTE: the following is required only if mod(iflag,10) <> 0)

ic    (I): color array

ic(1) : color for axis lines

ic(2) : color for axis numbers

ic(3) : color for axis titles

ic(4) : color for axis exponents

ic(5) : color for contour line 1

ic(6) : color for contour line 2, etc.

...      ...

l     (I): contour linetype list

7-12

MASTER SUBROUTINES

7.5  SUBROUTINE CNTLN

CNTLN plots a contour plot  of  a  randomly  scatted  set  of

points in three dimensions.  It uses INIT3dH The input consists of

a  list  of  triplets  of  a  surface  value.   The  triplets  are

triangulated  using  TRIANGC  and contours determined by linearily

interpolating the edges of the triangles.  The contour values  may

be  uniformly spaced between the starting and end values or from a

list.  Other than color, the sequence of plotting  (min  to  max),

and  line  typing  of  various  contour  lines,  no  contour  line

identification scheme is provided.  Caution should  be  excercised

when  interpreting plot since the distribution of input points may

affect the placement of the contour lines.

CALL CNTLN(x,y,z,n,xl,yl,iflag,nc,c,ia,xt,nxt,tx,sx,fx,

yt,nyt,ty,sy,fy,t,nt,xm,xx,ym,yx<<,ic>,l>)

x,y,z (R): array of point triplets (x,y,z)

n     (I): number of points

xl    (R): x axis length in inches

yl    (R): y axis length in inches

iflag (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude)  >10000: do not initialize LONGLIB before plotting

(1's digit)   = 1 : do not use color or line type arrays

= 2 : use color but not line type array

= 3 : do not use color but use line type array

= 4 : use both color and line type arrays

(10's digit)  = 0 : just x,y labeled axes

= 1 : axes and axis line/ticks on top and sides

(100's digit) = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

nc    (I): number of contour levels

< 0 : c(1) is the minimum contour level and c(2) is

the contour step size, abs(nc) levels plotted

> 0 : c contains contours levels, c dimensioned c(nc)

c     (R): list of contour levels

ia    (I): axis option flag

< 0 : do not plot axes

> 0 : plot axes

(1's digit)   = 1 : plot y axis using max/min of y array

= 2 : plot y axis using max/min of y array

smoothed by SCALE

= 3 : plot y axis using input max/min

= 4 : plot y axis using input max/min

smoothed by SCALE

(10's digit)  = 1 : plot x axis using max/min of x array

= 2 : plot x axis using max/min of x array

smoothed by SCALE

= 3 : plot x axis using input max/min

= 4 : plot x axis using input max/min

7-13

MASTER SUBROUTINES

smoothed by SCALE

(100's digit) = 0 : normal contouring

= 1 : show triangulation used without contours

xt     (B): x axis title string

nxt    (I): number of characters in title

< 0 : axis ticks on top of x axis

= 0 : no axis

> 0 : axis ticks on bottom of x axis (normal)

tx     (R): number and pattern of axis ticks (see AXIS3)

sx     (R): size of axis labeling (see AXIS3)

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

fx     (R): format of axis number labeling (see AXIS3)

yt     (B): y axis title string

nyt    (I): number of characters in title

< 0 : axis ticks on top of x axis

= 0 : no axis

> 0 : axis ticks on bottom of x axis (normal)

ty     (R): number and pattern of axis ticks (see AXIS3)

sy     (R): size of axis labeling (see AXIS3)

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

fy     (R): format of axis number labeling (see AXIS3)

t      (B): plot title string

nt     (I): number of characters in t (limited to 99 characters)

< 0 : use color array

= 0 : no title

> 0 : do not use color array

if |nt|/100 > 0 : use line type list

xm     (R): minimum value of x axis

xx     (R): maximum value of x axis

ym     (R): minimum value of y axis

yx     (R): maximum value of y axis

ic     (I): color list (optionally used)

ic(1) : color for axis lines

ic(2) : color for axis numbers

ic(3) : color for axis titles

ic(4) : color for axis exponents

ic(5) : color contour (1)

ic(6) : color contour (2), etc.

...      ...

l      (I): line type list for contours (optionally used)

common /ccntrplt/xmr,dxr,ymr,dyr

xmr   (R): returned value of xmin

dxr   (R): returned value of scale factor (xmax-xmin)/xlen

ymr   (R): returned value of ymin

dyr   (R): returned value of scale factor (ymax-ymin)/ylen

7-14

MASTER SUBROUTINES

7.6  SUBROUTINE CNTRPLT

CNTRPLT plots a contour plot of a uniformly sampled 2-d input

array.   The  input  consists of a 2 dimensional grid of y values.

For each contour level the array  is  scanned  cell  by  cell.   A

contour segment is determined by linearily interpolating the edges

of the square formed by 4 adjacent points (a cell).  For  example,

if  the  current  contour  value  is  1,  and  y(1,1)=0, y(1,2)=2,

y(2,2)=3, and y(1,2)=4, a contour line is  assumed  to  exist  for

this cell as shown:

y(1,2)        y(2,2)

*          *

+

\

* +        *

y(1,1)        y(1,2)

The contour values are uniformly spaced between the input starting

and  end  values  or  automatically  selected  values.  Other than

color, the sequence of plotting (min to max), and line  typing  of

various  contour  lines,  no contour line identification scheme is

provided.  Log axes are available  but  data  points  are  plotted

using linear positioning.  (Note:  common block scale factors will

be log values if the log axes are selected.)

The pathological case of two contour lines within a cell  may

case  the  routine  to  incorrectly trace the contour through that

cell.

CALL CNTRPLT(v,ndx,ndy,nx,ny,nl,as,ae,iflag,xl,yl,xt,nxt,yt,nyt,

t,nt,xm,xx,ym,yx<<,ic>,l>)

v       (R): 2-d array dimensioned v(ndx,ndy)

ndx,ndy (I): dimensions of v data array

nx,ny   (I): number of points in each array dimension

nl      (I): number of uniformly spaced contour levels,

< 0 : max and min of v are used for as, ae

(j)th contour is (j-1)*(ae-as)/(nl-1)+as

= 0 : int(ae) specifies the number of contour values

where as is an array of the contour values

> 0 : number of uniformly space contour levels,

(j)th contour is (j-1)*(ae-as)/(nl-1)+as

as      (R): first contour level (nl > 0)

array of contour levels (nl=0) dimensioned as(int(ae))

ae      (R): last contour level (nl > 0)

number of contour levels in as (nl=0) ae>0

iflag   (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude)  >10000: do not initialize LONGLIB before plotting

7-15

MASTER SUBROUTINES

(1's digit)   = 1 : plot x linear, y logarithmic (base 10)

= 2 : plot x logarithmic, y linear

= 3 : plot x logarithmic, y logarithmic

= 4 : plot x linear, y linear

(10's digit)  = 0 : no axes or title plotted

= 1 : plot box with axis tick marks on top and sides

= 2 : plot solid cartesian grid

= 3 : plot ticked cartesian grid without box

= 4 : plot ticked cartesian grid with box

= 5 : plot ticked cartesian grid, box w/axis ticks

= 6 : plot without box or cartesian grid

= 7 : plot solid logarithmic grid

= 8 : plot dotted logarithmic grid

= 9 : plot ticked logarithmic grid

(100's digit) = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

xl    (R): x axis length in inches (integer-valued)

> 0 : use input scaling in xm,xx for axis

< 0 : use smoothed input scaling in xm,xx for axis

yl    (R): y axis length in inches (integer valued)

> 0 : use input scaling in ym,yx for axis

< 0 : use smoothed input scaling in ym,yx for axis

xt    (B): x axis title string

nxt   (I): number of characters in xt

< 0 : axis ticks on top of x axis

= 0 : no axis

> 0 : axis ticks on bottom of x axis (normal)

yt    (B): y axis title string

nyt   (I): number of characters in yt

< 0 : axis ticks on right of y axis

= 0 : no axis

> 0 : axis ticks on left of y axis (normal)

t     (B): plot title string

nt    (I): number of characters in t (limited to 99 characters)

< 0 : use color array

= 0 : no title

> 0 : do not use color array

if |nt|/100 > 0 : use line type list

xm    (R): minimum value of x axis (will be smoothed for xl < 0)

xx    (R): maximum value of x axis (will be smoothed for xl < 0)

ym    (R): minimum value of y axis (will be smoothed for yl < 0)

yx    (R): maximum value of y axis (will be smoothed for yl < 0)

(NOTE: color array required if nt < 0 or |nt|/100 >0)

ic    (I): color array

ic(1) : color for grid

ic(2) : color for axis lines

ic(3) : color for axis numbers

ic(4) : color for axis titles

ic(5) : color for axis exponent

ic(6) : color for title (return)

ic(7) : color for contour line 1

ic(8) : color for contour line 2

ic(9) :    etc. ...

7-16

MASTER SUBROUTINES

l     (I): line type list for contours (required only if |nt|/100>0)

common /ccntrplt/xmr,dxr,ymr,dyr

xmr   (R): returned value of xmin

dxr   (R): returned value of scale factor (xmax-xmin)/xlen

ymr   (R): returned value of ymin

dyr   (R): returned value of scale factor (ymax-ymin)/ylen

7-17

MASTER SUBROUTINES

7.7  SUBROUTINE CVAX3D

CVAX3D plots a 3d surface  with  hidden  line  removal  using

either  a  mesh or a histogram.  Optionally, a contour plot can be

included  beneath  the  surface.   The  vertical  space   may   be

specified.   See CVAX3DX.  CVAX3D calls CVAX3DX using default axis

parameters to simplify the calling procedure.

CALL CVAX3D(d,ndx,ndz,nx,nz,a,b,xh,yh,zh,iflag,

iax,ds,nc,cv,icl,nm,il,ip,

<xt,nxt,xs,xe,yt,nyt,zt,nzt,zs,ze,<dm,dx<,ic>>>)

7.8  SUBROUTINE CVAX3DX

CVAX3DX plots a 3-d surface with hidden lines  removed  using

PLT3D  to  produce  a  mesh  surface  or  HLT3D  to  produce a 2-d

histogram with an optional contour plot made GCONTR and plotted on

a  plane paralell to the surface plane.  Axes and a back panel can

be optionally plotted.  Optionally, a path surface may be  plotted

which  connects  the  surface  and  contour  plots  over  a  users

specified path.  The 3d surface is plotted in a manner similar  to

MVAX3DX.   The  visible  upper side of the surface and the visible

lower side of the surface can be optionally shown using  different

colors and line types.

Origin of the plot is in the upper-left corner.  The  x  axis

runs  left  to right along the plot bottom.  The y axis is plotted

as a vertical displacement offset by the z axis value.  The z axis

appears  to  point out of the screen.  The contour plot is plotted

below the surface with a  user-specified  vertical  spacing.   The

contour  plot  plane  is  plotted paralell to the z=0 plane of the

surface with the (i,j) indicies of the surface  and  contour  plot

aligned  vertically.   The  user  may specify a "path" using "pen"

motions.  The path is plotted as a curve along the surface and the

contour  plot  plane  with  vertical  lines  at  the corresponding

indicies.  No hidden line removal is used for the path.  The  path

permits   the   user  to  specify  a  cut  plane  to  enhance  the

interpretation of the plot.  The path is specified as  a  sequence

of pen motion commands and index points of the form,

ip(1) = 1st path command

ip(2) = 1st i index

ip(3) = 1st j index

ip(4) = 2n path command

ip(5) = 2nd i index

ip(6) = 2nd j index

... etc.

Path commands are interpreted according to:

7-18

MASTER SUBROUTINES

path command      action

______________    ___________________________

0             end of path specification

(indicies ignored)

3             start path at these indicies

2             continue path though indicies

The path specificiation should start with a path command of 3  and

end  with  0.   Note  that several paths can be specified by using

several path command 3's.

CVAX3DX contains an internal working storage array for use by

GCONTR  and  PLT3D.   The  buffer  length  is  sufficient for most

surfaces.  However, for very complex surfaces  the  buffer  length

may  be exceeded.  When this occurs an error message is written to

the terminal and the routine terminates.

CALL CVAX3DX(d,ndx,ndz,nx,nz,a,b,xh,yh,zh,iflag,

iax,ds,nc,cv,icl,nm,il,ip,

<xt,nxt,xs,xe,xap,tsx,fdx,

yt,nyt,yap,tsy,fdy,

zt,nzt,zs,ze,zap,tsz,fdz,<dm,dx<,ic>>>)

d        (R): array of y values dimensioned d(ndx,ndz)

ndx,ndz  (I): x and z dimensions of d array

nx,nz    (I): x and z sizes of surface to plot d array

a        (R): angle of x axis from horizontal 0-85 degrees

b        (R): angle of z axis from horizontal 0-90 degrees

note: origin (1,1) is in upper-left corner

x axis runs left-to-right

y axis runs down-to-up

z axis appears to run outof page screen but

is angled to the right

xh,yh,zh (R): length of each axis

iflag    (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit)   = 2 : use color array (need all parameters)

= 1 : do not use color array

(10's digit)  = 0 : plot surface as a mesh (PLT3D)

= 1 : plot surface as 2-d historgram (HLT3D)

(100's digit) = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

iax   (I): axis format control

< 0 : plot axis, using input scale factors dm and dx

= 0 : do not plot axis, axis parameters (xt...dx) not used

scaling derived from d array is used

> 0 : plot axis, using scaling derived from d array, only

axis parameters xt thru ze required.

7-19

MASTER SUBROUTINES

(1's digit)   = 1 : plot actual max/min or input values for Y axis

= 2 : plot smoothed values for Y axis

(10's digit)  = 0 : plot contour, surface axes with back panel

= 1 : plot contour, surface axes w/o back panel

= 2 : plot contour axes w/o surface axes, back panel

= 3 : plot surface axes w/o back panel, contour axes

= 4 : plot surface axes, back panel w/o contour axes

(100's digit) = 0 : use default axis type

= 1 : use input AXIS3 parameters

ds    (R): vertical spacing between contour plane and minimum

value of a plane

nc    (I): number of contours

< 0 : iabs(nc) contours plotted, the (j)th contour

is (max(a)-min(a))/(iabs(nc)-1). values used

are returned in cv

> 0 : contours specified in cv used

cv    (R): contour level array dimensioned dv(iabs(nc)

icl   (I): contour labeling option

< 0 : label with contour value (number with n digits

to the right of the decimal point)

= 0 : no labels on contours

> 0 : label contours with ASCII characters (nl <= 26)

nm    (I): minimum line segments before contour labeled

< 0 : line type array used

> 0 : line type array not used

il    (I): array of line types dimensioned il(iabs(nc)+2)

il(1) = underside of surface

il(2) = path line

il(3) = contour line 1

il(4) = contour line 2

...   (solid line type on return)

ip    (I): path specification array (see notes above)

if ip(1) = 0, remainder of array ignored

(NOTE: following optional axis paramters are used only if

iax<0 or mod(iflag,10)=1)

xt    (B): title of x axis (width)

nxt   (I): number of characters in xt

= 0 : no axis plotted

> 0 : normal

xs,xe (R): starting and ending values displayed on x axis

xap   (R): axis tick pattern (see AXIS3)

tsx   (R): size of title and numbers on x axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

fdx   (R): axis number label format (see AXIS3)

yt    (B): title of y axis (height)

nyt   (I): number of characters in yt

= 0 : no y axis plotted

> 0 : normal

yap   (R): axis tick pattern (see AXIS3)

tsy   (R): size of title and numbers on x axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

7-20

MASTER SUBROUTINES

fdy   (R): axis number label format (see AXIS3)

zt    (B): title of z axis (depth)

nzt   (I): number of characters in zt

= 0 : no z axis plotted

> 0 : normal

ze,ze (R): starting and ending valued displayed on z axis

zap   (R): axis tick pattern (see AXIS3)

tsz   (R): size of title and numbers on x axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

fdz   (R): axis number label format (see AXIS3)

(NOTE: the following are required only if iax<0 or mod(iflag,10)=1)

dm,dx (R): minimum and maximum scale values for d array

ic    (I): color list

ic(1) : color for axis lines

ic(2) : color for axis numbers

ic(3) : color for axis titles

ic(4) : color for axis exponents

ic(5) : color for upper surface

ic(6) : color for lower surface

ic(7) : color for contour 1

ic(8) : color for contour 2

...        ...

(color for last contour on return)

7-21

MASTER SUBROUTINES

7.9  SUBROUTINE GLPLOT

GLPLOT plots the curve defined in x,y with log and/or  linear

scaling  including  appropriate  axes  and  plot  title.   Various

options select the format of the axes plotting.   This  subroutine

is  designed  to  plot  one or more y value curves simultaneously.

This subroutine permits axis  parameter  flexibility  (AXIS3)  and

dimensioning  for  y  array.   GLPLOT  is  a  good general purpose

plotting routine.

CALL GLPLOT(x,y,nld,npd,nl,np,iflag,isym,xl,yl,xt,xc,xf,xtitle,nxt,

yt,yc,yf,ytitle,nyt,t,nt,<xm,xx,ym,yx<<,ic>,l>>)

x     (R): array of x values dimensioned at least x(np)

y     (R): array of y values dimensioned y(npd,nld)

nld   (I): dimension of y (number of lines dimensioned)

npd   (I): dimension of y (number of points dimensioned)

nl    (I): number of data lines to plot from y array

np    (I): number of points per line

iflag (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude)  >10000: do not initialize LONGLIB before plotting

(1's digit)   = 1 : plot x linear, y logarithmic (base 10)

= 2 : plot x logarithmic, y linear

= 3 : plot x logarithmic, y logarithmic

= 4 : plot x linear, y linear

(10's digit)  = 0 : no axes or title plotted

= 1 : axes with axis line/ticks on top and sides

= 2 : plot solid cartesian grid

= 3 : plot ticked cartesian grid without box

= 4 : plot ticked cartesian grid with box

= 5 : plot ticked cartesian grid, box w/axis ticks

= 6 : plot without box or cartesian grid

= 7 : plot solid logarithmic grid

= 8 : plot dotted logarithmic grid

= 9 : plot ticked logarithmic grid

(100's digit) = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

isym  (I): plot a symbol every isym'th point

< 0 : symbols only plotted, no line

= 0 : no symbols, line only

> 0 : symbol plotted every isym'th point

xl    (R): x axis length in inches

< 0 : use input scaling in xm,xx

> 0 : use auto scaling computed from input array

yl    (R): y axis length in inches

< 0 : use input scaling in ym,yx

> 0 : use auto scaling computed from input array

xt    (R): x axis tick mark pattern (see AXIS3)

xc    (R): x axis character size

< 0 auto exponent scaling (x10 to power) disabled

7-22

MASTER SUBROUTINES

> 0 auto exponent scaling (x10 to power) enabled

xf    (R): x axis number label format (see AXIS3)

xtitle(B): x axis title string

nxt   (I): number of characters in xt

< 0 : axis ticks on top of x axis

= 0 : no axis

> 0 : axis ticks on bottom of x axis (normal)

yt    (R): y axis tick mark pattern (see AXIS3)

yc    (R): x axis character size

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

yf    (R): y axis number label format (see AXIS3)

ytitle(B): y axis title string

nyt   (I): number of characters in yt

< 0 : axis ticks on right of y axis

= 0 : no axis

> 0 : axis ticks on left of y axis (normal)

t     (B): plot title string (limited to 99 characters)

nt    (I): number of characters in t

< 0 : use color array

= 0 : no title

> 0 : do not use color array

if |nt|/100 > 0 : use line type list

xm    (R): minimum value of x array (required if xl or nt < 0)

xx    (R): maximum value of x array (required if xl or nt < 0)

ym    (R): minimum value of y array (required if xl, yl or nt<0)

yx    (R): maximum value of y array (required if xl, yl or nt<0)

(NOTE: color array required if nt < 0 or |nt|/100 >0)

ic    (I): color list

ic(1) : color for grid

ic(2) : color for axis lines

ic(3) : color for axis numbers

ic(4) : color for axis titles

ic(5) : color for axis exponent

ic(6) : color for title (return)

ic(7) : color for plotted line 1

ic(8) : color for plotted line 2

ic(9) :    etc.

l     (I): line type of data lines list (required only if |nt|/100>0)

common /cglplot/xmr,dxr,ymr,dyr

xmr   (R): returned value of xmin

dxr   (R): returned value of scale factor (xmax-xmin)/xlen

ymr   (R): returned value of ymin

dyr   (R): returned value of scale factor (ymax-ymin)/ylen

7-23

MASTER SUBROUTINES

7.10  SUBROUTINE HIST3D

HIST3D plots a 2-d histogram as square columns plotted  as  z

values  from  the  x,y  plane with hidden line removal.  The input

consists of the values of the z axis for each (x,y) point  on  the

plane.   Each square grid is plotted as two triangles using either

SKETCH or  PLT3DH  permitting  hidden  line  removal  if  desired.

HIST3D  calls  INIT3DH  to initialize the 3-d hidden line package.

Default space is provided in the INIT3DH common block.  The  ISIZE

variable  will  be  reset  to  the  internal  size if its value is

smaller than the internal size.  Sufficient space in the  internal

common block is provided for reasonably complex surfaces.

CALL HIST3D(z,ndx,ndy,nx,ny,s,p,t,dv,zref,xl,yl,zl,iflag,ia,

xt,nxt,tx,sx,gx,fx,

yt,nyt,ty,sy,gy,fy,

zt,nzt,tz,sz,gz,fz,

xm,xx,ym,yx,zm,zx<,ic>)

z      (R): array of z values

ndx,ndy(I): dimensions of z array

nx,ny  (I): number of points in each dimension to use of z array

s,p,t  (R): yaw,roll,pitch angles of axes (see INIT3DH)

dv     (R): perspective scale factor for INIT3DH

< 0 : no hidden lines in SKETCH, INIT3DH called

= 0 : INIT3DH not called to initialize 3d package,

(INIT3DH previously called)

> 0 : hidden lines in SKETCH, INIT3DH called

= 9999 : no perspective in INIT3DH

zref   (R): reference value from which columns are drawn from

xl     (R): x axis length in inches

yl     (R): y axis length in inches

zl     (R): z axis length in inches

iflag  (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit)   = 1 : do not use color array

= 2 : use color array

(10's digit)  = 0 : hidden line removal for surface only

= 1 : hidden line removal for axis and surface

= 2 : no hidden line removal (SKETCH not used)

(100's digit) = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

ia     (I): axis option flag

< 0 : plot axis use input z axis scale

= 0 : do not plot axes

> 0 : plot axis use computed z axis scale

(1's digit)  = 1 : plot y axis using input scale

= 2 : plot y axis using input, smoothed scale

(10's digit) = 1 : plot x axis using input scale

= 2 : plot x axis using input, smoothed scale

7-24

MASTER SUBROUTINES

xt     (B): x axis title string

nxt    (I): number of characters in x axis title

< 0 : axis ticks on top of x axis

= 0 : no axis

> 0 : axis ticks on bottom of x axis (normal)

tx     (R): number and pattern of axis ticks (see AXIS3)

sx     (R): size of axis labeling (see AXIS3)

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

gx     (R): rotation angle for x axis (see AXIS3DH)

fx     (R): format of axis number labeling (see AXIS3)

yt     (B): y axis title string

nyt    (I): number of characters in y axis title

< 0 : axis ticks on top of axis

= 0 : no axis

> 0 : axis ticks on bottom of axis (normal)

ty     (R): number and pattern of axis ticks (see AXIS3)

sy     (R): size of axis labeling (see AXIS3)

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

gy     (R): rotation angle for y axis (see AXIS3DH)

fy     (R): format of axis number labeling (see AXIS3)

zt     (B): z axis title string

nzt    (I): number of characters in z axis title

< 0 : axis ticks on top of axis

= 0 : no axis

> 0 : axis ticks on bottom of axis (normal)

tz     (R): number and pattern of axis ticks (see AXIS3)

sz     (R): size of axis labeling (see AXIS3)

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

gz     (R): rotation angle for z axis (see AXIS3DH)

fz     (R): format of axis number labeling (see AXIS3)

xm,xx  (R): minimum, maximum value displayed on x axis

ym,yx  (R): minimum, maximum value displayed on y axis

zm,zx  (R): minimum, maximum value of z axis (required if ia < 0)

ic     (I): color list (required if 1's digit of iflag=2)

ic(1) : color for axes

ic(2) : color for surface

7-25

MASTER SUBROUTINES

7.11  SUBROUTINE LCNTR

LCNTR plots a contour plot of a uniformly sampled  2-d  input

array.   LCNTR  is similar to CNTRPLT but uses software line types

(LINSEQ).  The input consists of a 2 dimensional grid of v values.

A  contour  is  determined by linearily interpolating the edges of

the square formed by  4  adjacent  points  (see  CNTRPLT).   LCNTR

connects the line segments of constant contour before plotting and

will  label  the  contour  lines  with  alphabetic  codes  or  the

numberical  value of the contour.  Log axes are available but data

points are plotted using linear positioning.  (Note:  common block

scale factors will be log values if the log axes are selected.)

When the contour levels are input  (nl>0),  contours  can  be

suppressed  in  a  region  by setting the value of the input array

greater than 1.E20.  LCNTR uses the contouring routine GCONTR.

CALL LCNTR(v,ndx,ndy,nx,ny,nl,cl,n,m,iflag,iw,xl,yl,xt,nxt,yt,nyt,

t,nt,xm,xx,ym,yx<<,ic>,l>)

v       (R): 2-d array of values dimensioned v(ndx,ndy)

ndx,ndy (I): dimensions of data array

nx,ny   (I): number of points in each array dimension

nl      (I): number of uniformly spaced contour levels,

< 0 : max and min of v are used to define contours.

the (j)th contour is computed from max,min of

input array according to,

(j-1)*(max(v)-min(v))/(nl-1)+min(v)

> 0 : number of contour levels specified in cl

cl      (R): array of contour levels dimensioned cl(nl)

if nl < 0 contour levels used are (returned) in cl

n       (I): contour labeling option

< 0 label with contour value (number with n digits

to the right of the decimal point)

= 0 no labels on contours

> 0 label contours with ASCII characters (nl <= 26)

m       (I): minimum line segments before contour labeled (m > 1)

iflag   (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) >10000 : do not initialize LONGLIB before plotting

(1's digit)  = 1 : plot x linear, y logarithmic (base 10)

= 2 : plot x logarithmic, y linear

= 3 : plot x logarithmic, y logarithmic

= 4 : plot x linear, y linear

(10's digit) = 0 : no axes or title plotted

= 1 : axes with axis line/ticks on top and sides

= 2 : plot solid cartesian grid

= 3 : plot ticked cartesian grid without box

= 4 : plot ticked cartesian grid with box

= 5 : plot ticked cartesian grid, box w/axis ticks

= 6 : plot without box or cartesian grid

7-26

MASTER SUBROUTINES

= 7 : plot solid logarithmic grid

= 8 : plot dotted logarithmic grid

= 9 : plot ticked logarithmic grid

(100's)      = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

iw      (I): working array dimensioned at least (2*nx*ny+1)/31

xl      (R): x axis length in inches (integer valued)

> 0 : use input scaling in xm,xx for axis

< 0 : use smoothed input scaling in xm,xx for axis

yl      (R): y axis length in inches (integer valued)

> 0 : use input scaling in ym,yx for axis

< 0 : use smoothed input scaling in ym,yx for axis

xt      (B): x axis title string

nxt     (I): number of characters in xt

< 0 : axis ticks on top of x axis

= 0 : no axis

> 0 : axis ticks on bottom of x axis (normal)

yt      (B): y axis title string

nyt     (I): number of characters in yt

< 0 : axis ticks on right of y axis

= 0 : no axis

> 0 : axis ticks on left of y axis (normal)

t      (B): plot title string

nt     (I): number of characters in t (limited to 99 characters)

< 0 : use color array

= 0 : no title

> 0 : do not use color array

if |nt|/100 > 0 : use line type list

xm     (R): input minimum value of x axis

xx     (R): input maximum value of x axis

ym     (R): input minimum value of y axis

yx     (R): input maximum value of y axis

(NOTE: color array required if nt < 0 or |nt|/100 >0)

ic     (I): color array

ic(1) : color for grid

ic(2) : color for axis lines

ic(3) : color for axis numbers

ic(4) : color for axis titles

ic(5) : color for axis exponent

ic(6) : color for title (return)

ic(7) : color for contour line 1

ic(8) : color for contour line 2

ic(9) :    etc.

l       (I): line type list for contours (required if |nt|/100>0)

common /ccntrplt/xmr,dxr,ymr,dyr

xmr   (R): returned value of xmin

dxr   (R): returned value of scale factor (xmax-xmin)/xlen

ymr   (R): returned value of ymin

dyr   (R): returned value of scale factor (ymax-ymin)/ylen

7-27

MASTER SUBROUTINES

7.12  SUBROUTINE LSPLOT

LSPLOT is a general single/multiple  data  line  routine  for

plotting  lines  or  scatterplots.   Optionally,  error  bars  and

symbols can be added.  LSPLOT  differs  in  philosophy  from  most

other  LONGLIB MASTER routines in that plotting options are passed

via an option array routine.  The routine is designed to produce a

reasonable  plot  with  the  option  array  set to all zeros.  The

output format is changed by initializing selected  array  elements

to  the  values described below.  This permits simple but flexible

specification of the plot format.

CALL LSPLOT(x,y,ndp,ndl,iflag,f,c,nc)

x     (R): x input array dimensioned x(ndp,ndl).  Depending

on option selected, x need only be dimensioned

x(ndp) -- this is the default option.

y     (R): y input array dimensioned y(ndp,ndl).  If

ndl=1 then y may dimensioned y(ndp)

ndp   (I): number of data points/line dimension

ndl   (I): number of data lines dimension

iflag (I): LONGLIB option flag (one's digit arbitrary)

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) >10000 : do not initialize LONGLIB before plotting

(100's digit) = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

f     (R): option array dimensioned at least f(53)

(described below)

c     (C): array of strings dimensioned C(3+ndl)

c(1) : x axis title

c(2) : y axis title

c(3) : top title

c(4) : line 1 legend (optionally used)

c(5) : line 2 legend (optionally used)

...        ...

nc    (I): array of string lengths dimensioned nc(3+ndl)

nc(1) : number of characters to use in C(1)

nc(2) : number of characters to use in C(2)

...       ...

The array elements of the option  array  f  are  interpretted

according  to  the  following table.  Some parameters have default

values (shown in square brackets).  These are used when the  input

value  is  zero.  A simple plot may be produced by setting all the

elements of f to zero.  Note that  user  specified  input  scaling

factors should be powers of ten when the log axis specification is

selected.  An optional legend may be plotted.  The legend consists

of a column of line/simple examples (if selected) and input text.

7-28

MASTER SUBROUTINES

array  range of

index  values   action for each value

------- -------- ------------------------

1    0<=.<=ndp number of points/line to plot [ndp]: 0=ndp used

2    0<=.<=ndl number of lines to plot [nl]: 0=ndl used

3     -1/0/1   x scale: 0=auto,smoothed; 1=auto,nosmooth; -1=user

4      xmin    user supplied scale value (used if f(3)<0)

5      xmax    user supplied scale value (used if f(3)<0)

6     -1/0/1   y scale: 0=auto,smoothed; 1=auto,nosmooth; -1=user

7      ymin    user supplied scale value (used if f(6)<0)

8      ymax    user supplied scale value (used if f(6)<0)

9      0/1     x value usage: 0=first line of x data array

used for all y lines; 1=lines of x,y paired

10     0/1     connect plotted points: 0=yes; 1=no

11     >=0     symbol plotted every ()th point: 0=no symbols

12     >=0     line symbol size [0.1]: 0=use default

13     >=0     symbol number for first data line, each line

then uses next symbol in sequence

14     8<.<8   error bar option (see below): 0=no error bars

16     >=0     error bar size [0.1]: 0=default used

17     0/1     vertical line from points to reference value:

0=no; 1=yes

18     rval    reference value

19    -1/0/1   x axis type: 0=linear; 1=log axis, -1=no axis

20    -1/0/1   y axis type: 0=linear; 1=log axis, -1=no axis

21     >=0     x axis length [7.0]: 0=default used

22     >=0     y axis length [5.0]: 0=default used

23     >=0     x axis tick pattern (see axis3) [7.00]: 0=default

24     >=0     y axis tick pattern (see axis3) [5.00]: 0=default

25     0/1     x axis title side of axis: 0=below; 1=above

26     0/1     y axis title side of axis: 0=left; 1=right

27     0/1     x axis auto exponent enable: 0=enable; 1=disable

28     0/1     y axis auto exponent enable: 0=enable; 1=disable

29     0/1     x axis tick side: 0=below; 1=above

30     0/1     y axis tick side: 0=left; 1=right

31     0/1     x axis number orientation: 0=horizontal; 1=vertical

32     0/1     y axis number orientation: 0=vertical; 1=horizontal

33     0/1     x axis numbers/title: 0=shown; 1=not shown

34     0/1     y axis numbers/title: 0=shown; 1=not shown

35     0/1     use x=log10(abs(x values)+1.e-34): 0=no; 1=yes

36     0/1     use y=log10(abs(y values)+1.e-34): 0=no; 1=yes

37   -1/0/1    add mirror x axis: 0=no; 1=w/labels; -1:w/o labels

38   -1/0/1    add mirror y axis: 0=no; 1=w/labels; -1:w/o labels

(mirrored axes placed on opposite from normal axis)

39     >=0     x axis label size [0.15]: 0=use default

40     >=0     y axis label size [0.15]: 0=use default

41     >=0     top title character size [0.18]: 0=use default

42   0/1/2/3   grid: 0=no grid; 1=solid; 2=dotted; 3=ticked

43   -1/0/1    legend: 0=no legend; 1=right side; -1=user locate

44     xval    user specified lower-left corner of legend

45     yval    user specified lower-left corner of legend

7-29

MASTER SUBROUTINES

46     0/1     show plot symbol on legend: 0=no; 1=yes

47     0/1     show line segment on legend: 0=no; 1=yes

48     >=0     legend character height [0.12]: 0=use default

49     >=0     legend line segment length [0.5]: 0=use default

50   -1/0/1    top title justify: 0=center; -1:left; 1:right

51    0/1      plot horizontal reference line: 0=no; 1=yes

52    0/1      use linetype array values: 0=no; 1=yes

53    0/1      use color array values: 0=no; 1=yes

54    >=0      color index #1: 0=color value 1 used

55    >=0      linetype index #1

56    >=0      color index #2: 0=color value 1 used

57    >=0      linetype index #2

...   ...      ... etc ...

The optional error bar specification, when non-zero,  changes

interpretation  of  lines.   The first line (and every third line)

conidered a "center" line.  The second line specifies the relative

error  (to  be added to the first line) used for plotting the tops

of the error bars.  The third line is used  similarily  to  locate

the  bottoms  of the error bars.  When the error bar specification

is negative the center line points are marked with a  special  "x"

(in  addition  to  any other option).  The absolution value of the

specifiation determines the type of error  bar  according  to  the

following table.

value   type of error bar

------  ---------------------------------------------------

1     line connecting relative errors

2     1 + horizontal bars at relative errs

3     1 + vertical bars at relative errs

4     double line connecting rel. errs+horizontal bars

5     double line connecting rel. errs+vertical bars

6     vertical rectangle w/top and bottom rel. errs

7     rectangle with corners at relative errors

The color  and  line  type  index  (when  enabled)  are  used

according to the following table.

index #        color usage     linetype usage

---------      -------------   ----------------

1              x axis         x axis

2           x axis numbers    y axis

3           x axis title      title

4           x axis exponent   legend titles

5              y axis         reference line

6           y axis numbers    error bars

7           y axis title      line #1 linetype

8           y axis exponent   line #2 linetype

9              title            etc.

10             legend titles     ...

11            reference line     ...

12               grid            ...

13             line #1 color     ...

7-30

MASTER SUBROUTINES

14             line #2 color     ...

...             ... etc. ...      ...

7-31

MASTER SUBROUTINES

7.13  SUBROUTINE MESH3D

MESH3D plots  a  2-d  surface  as  3d  mesh  using  the  same

techniques as VAX3D but without hidden line removal.  MESH3D calls

MESH3DX using default axis  parameters  to  simplify  the  calling

procedure.

CALL MESH3D(d,ndx,ndz,nx,nz,a,b,xh,yh,zh,iflag,iax,<xt,nxt,xs,xe,

yt,yzt,zt,nzt,zs,ze,<dm,dx<,ic>>>)

See MESH3DX for parameter description.

(iax is limited to a single digit value)

7.14  SUBROUTINE MESH3DX

MESH3DX is a simple 3-d  surface  plotting  routine.   A  3-d

surface  is plotted as a simple mesh grid.  The plotting method is

similar to VAX3DX.  No hidden line removal is done.  The height of

plotted  surface  relative  to its y axis value is calibrated to z

axis.  No perspective is  used.   Options  exist  to  varying  the

plotting angle and to plot axes.

Origin of the plot is in the lower-left corner.  The  x  axis

runs  plotted  left to right along the plot bottom.  The y axis is

plotted as a vertical displacement offset by  the  z  axis  value.

The  z  axis  appears  to  point  into the screen.  This gives the

illusion of depth.

CALL MESH3DX(d,ndx,ndz,nx,nz,a,b,xh,yh,zh,iflag,iax,

<xt,nxt,xs,xe,nmx,nnx,mlx,tsx,ndx,smx,

yt,nyt,nmy,nny,mly,tsy,ndy,smy,

zt,nzt,zs,ze,nmz,nnz,mlz,tsz,ndz,smz,

<dm,dx<,ic>>>)

d        (R): array of y values dimensioned d(ndx,ndz)

ndx,ndz  (I): x and z dimensions of d array

nx,nz    (I): x and z sizes of surface to plot d array

a        (R): angle of x axis from horizontal 0-85 degrees

b        (R): angle of z axis from horizontal 0-90 degrees

note: origin (1,1) is in lower-left corner

x axis runs left to right on screen

y axis runs up to down on screen

z axis appears to run into the screen but

is angled to the right

xh,yh,zh (R): length of each axis

iflag    (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude)  >10000: do not initialize LONGLIB before plotting

(1's digit)   = 2 : use color array (need all parameters)

7-32

MASTER SUBROUTINES

= 1 : do not use color array

(10's digit)  = 0 : Plot sides

= 1 : Do not plot sides

(100's digit) = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

iax   (I): axis format control

< 0 : plot axes, using input scale factors dm and dx

= 0 : axes not plotted, parameters (yt...dx) not used.

scaling derived from d array is used

> 0 : plot axes, use max and min of d array to compute

dm and dx, need axis parameters yt thru ze

(1's digit)  = 1 : Plot actual max/min or input values for Y axis

= 2 : Plot smoothed values for Y axis

(10's digit) = 0 : Use default axis type

= 1 : Use input AXIS2-type axis parameters

(NOTE: the following optional paramters are used only if

iax < 0 or mod(iflag,10)=1)

xt    (B): title of x axis (width)

nxt   (I): number of characters in xt

= 0 : no axis plotted

> 0 : axis plotted

xs,xe (R): starting and ending values displayed on x axis

(see AXIS2 for detailed description of axis parameters)

nmx   (I): number of minor ticks between major ticks on x axis

nnx   (I): highlight length of nnx-th minor tick on x axis

mlx   (I): number of major tick marks on x axis

tsx   (R): size of title and numbers on x axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

ndx   (I): number of digits to right of decimal point on x axis

smx   (R): major tick length on x axis

yt    (B): title of y axis (depth)

nyt   (I): number of characters in yt

= 0 : no y axis plotted

> 0 : normal

nmy   (I): number of minor ticks between major ticks on y axis

nny   (I): highlight length of nny-th minor tick on y axis

mly   (I): number of major tick marks on y axis

tsy   (R): size of title and numbers on y axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

ndy   (I): number of digits to right of decimal point on y axis

smy   (R): major tick length on y axis

zt    (B): title of z axis (height)

nzt   (I): number of characters in zt

= 0 : no z axis plotted

> 0 : normal

ze,ze (R): starting and ending valued displayed on z axis

nmz   (I): number of minor ticks between major ticks on z axis

nnz   (I): highlight length of nnz-th minor tick on z axis

mlz   (I): number of major tick marks on z axis

tsz   (R): size of title and numbers on z axis

< 0 auto exponent scaling (x10 to power) disabled

7-33

MASTER SUBROUTINES

> 0 auto exponent scaling (x10 to power) enabled

ndz   (I): number of digits to right of decimal point on z axis

smz   (R): major tick length on z axis

(NOTE: the following optional parameters are required if

iax < 0 or mod(iflag,10)=1)

dm,dx (R): minimum and maximum scale values for d array

ic    (I): color list

ic(1) : color for axis lines

ic(2) : color for axis numbers

ic(3) : color for axis titles

ic(4) : color for axis exponents

ic(5) : color for plot surface (return)

7-34

MASTER SUBROUTINES

7.15  SUBROUTINE MVAX3D

MVAX3D plots a 3d mesh surface or 2-d histogram  with  hidden

line  removal.   See  MVAX3DX.   The  calling format is similar to

VAX3D.  MVAX3D calls MVAX3DX  using  default  axis  parameters  to

simplify calling procedure.

CALL MVAX3D(d,ndx,ndz,nx,nz,a,b,xh,yh,zh,iflag,iax,<xt,nxt,

xs,xe,yt,nyt,zt,nzt,zs,ze,<dm,dx<,ic>>>)

7.16  SUBROUTINE MVAX3DX

MVAX3DX is a 3-d surface plotting routine which plots  a  3-d

surface  as mesh with hidden lines removed using the PLT3D routine

or a 2-d histogram with  hidden  lines  removed  using  the  HLT3D

routin.   Axes  and a axis back panel can be optionally plotted as

well.  The upper side of the visible surface  can  be  shown  with

optional  side plates on the mesh surface.  If the side plates are

not used or for histogram plotting, the lower side of the  visible

surface  may  be  displayed  in different colors or using a dotted

line.

Origin of the plot is in the upper-left corner.  The  x  axis

runs  left  to right along the plot bottom.  The y axis is plotted

as a vertical displacement offset by the z axis value.  The z axis

appears to point out of the screen.

MVAX3DX contains an internal working storage array for use by

PLT3D  and  HLT3D.   The  buffer  length  is  sufficient  for most

surfaces.  However, for very complex surfaces  the  buffer  length

may  be exceeded.  When this occurs an error message is written to

the terminal and the routine terminates.

CALL MVAX3DX(d,ndx,ndz,nx,nz,a,b,xh,yh,zh,iflag,iax,

<xt,nxt,xs,xe,xap,tsx,fdx,

yt,nyt,yap,tsy,fdy,

zt,nzt,zs,ze,zap,tsz,fdz,<dm,dx<,ic>>>)

d        (R): array of y values dimensioned d(ndx,ndz)

ndx,ndz  (I): x and z dimensions of d array

nx,nz    (I): x and z sizes of surface to plot d array

a        (R): angle of x axis from horizontal 0-85 degrees

b        (R): angle of z axis from horizontal 0-90 degrees

note: origin (1,1) is in upper-left corner

x axis runs left-to-right

y axis runs down-to-up

z axis appears to run outof page screen but

is angled to the right

xh,yh,zh (R): length of each axis

iflag    (I): option flag

< 0 : do not close LONGLIB after plotting

7-35

MASTER SUBROUTINES

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit)   = 2 : use color array (need all parameters)

= 1 : do not use color array

(10's digit)  = 0 : mesh surface w/side panels, lower side of

surface not shown

= 1 : mesh surface w/no side panels, lower side of

surface shown using dotted lines

= 2 : mesh surface w/no side panels, lower side of

surface shown using solid lines

= 3 : mesh surface w/no side panels, lower side of

surface not shown

= 4 : histogram surface, lower side of surface

shown using dotted lines

= 5 : histogram surface, lower side of surface

shown using solid lines

= 6 : histogram surface, lower side of surface

shown using solid lines

(100's digit) = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

iax   (I): axis format control

< 0 : plot axes, using input scale factors dm and dx

= 0 : no axes plotted, parameters (xt...dx) not used.

scaling derived from d array is used

> 0 : plot axes, use scaling derived from d array, only

axis parameters xt thru ze required.

(1's digit)   = 1 : Plot actual max/min or input values for Y axis

= 2 : Plot smoothed values for Y axis

(10's digit)  = 0 : Use default axis type

= 1 : Use input AXIS3 parameters

(100's digit) = 0 : Do not plot backplane

= 1 : Plot backplane

(NOTE: following optional axis paramters are used only if

iax<0 or mod(iflag,10)=1)

xt    (B): title of x axis (width)

nxt   (I): number of characters in xt

= 0 : no axis plotted

> 0 : normal

xs,xe (R): starting and ending values displayed on x axis

xap   (R): axis tick pattern (see AXIS3)

tsx   (R): size of title and numbers on x axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

fdx   (R): axis number label format (see AXIS3)

yt    (B): title of y axis (height)

nyt   (I): number of characters in yt

= 0 : no y axis plotted

> 0 : normal

yap   (R): axis tick pattern (see AXIS3)

tsy   (R): size of title and numbers on x axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

7-36

MASTER SUBROUTINES

fdy   (R): axis number label format (see AXIS3)

zt    (B): title of z axis (depth)

nzt   (I): number of characters in zt

= 0 : no z axis plotted

> 0 : normal

ze,ze (R): starting and ending valued displayed on z axis

zap   (R): axis tick pattern (see AXIS3)

tsz   (R): size of title and numbers on x axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

fdz   (R): axis number label format (see AXIS3)

(NOTE: the following optional parameters are required if

iax<0 or mod(iflag,10)=1)

dm,dx (R): minimum and maximum scale values for d array

ic    (I): color list

ic(1) : color for axis lines

ic(2) : color for axis numbers

ic(3) : color for axis titles

ic(4) : color for axis exponents

ic(5) : color for plot surface (return)

7-37

MASTER SUBROUTINES

7.17  SUBROUTINE MVAX5D

MVAX5D plots a 4 or 5-d surface by  plotting  slices  through

the  3rd  and 4th dimensions in a 2-d array of of 3-d plots.  Each

3-d surface plots d(*) as a function of 2 of the dimensions  using

MVAX3D.

Origin of the plot is in the lower-left corner.  The  X  axis

runs  left  to  right  along  the  subplot  bottom.  The Y axis is

plotted out the page of the subplot (see MVAX3D).  The Z axis runs

left to right in subplots with the W axis vertical subplots.

^ W  d

|   |

|   |__X

|  /

| Y

----------> Z

Since the subplots may runoff the edge of the plotting  page,

the  routine  includes  a  page size option to issue a NEWPAGE and

plot the additional  subplots  on  separate  pages.   A  shrinking

factor  is included to shrink the subplots.  Labeling of the W and

Z axis is due in the lower right hand  corner.   Each  subplot  is

further  tagged  with  the  corresponding  W  and Z axis value.  A

multiple page  plot  can  be  pasted  together  to  form  a  large

representation of a 5-d (or 4-d) surface.

CALL MVAX5D(d,nd,n,a,b,iflag,iax,iform,w,xh,yh,zh,ph,pl,fac,iw,iz,

st,en,t1,nt1,t2,nt2,t3,nt3,t4,nt4,dt,ndt,<dm,dx<,ic>>)

d        (R): array to plot dimensioned d(nd(1),nd(2),nd(3),nd(4))

nd       (I): array of containing dimensions of d array

n        (I): array of number of points in each dimension to plot

a,b      (R): angles a,b for MVAX3D subplot

iflag    (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit)   = 2 : use color array (need all parameters)

= 1 : do not use color array

(10's digit)  = 0 : mesh surface w/side panels, lower side of

surface not shown

= 1 : mesh surface w/no side panels, lower side of

surface shown using dotted lines

= 2 : mesh surface w/no side panels, lower side of

surface shown using solid lines

= 3 : mesh surface w/no side panels, lower side of

surface not shown

= 4 : histogram surface, lower side of surface

shown using dotted lines

= 5 : histogram surface, lower side of surface

7-38

MASTER SUBROUTINES

shown using solid lines

= 6 : histogram surface, lower side of surface

shown using solid lines

(100's digit) = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

iax      (I): axis format control

< 0 : plot axes, use input scale factors dm and dx

= 0 : no axes plotted, parameters (xt...dx) not used.

scaling derived from d array

> 0 : plot axes, scaling derived from d array, only

axis parameters xt thru ze required.

(1's digit)  = 1 : Plot actual max/min or input values for Y axis

= 2 : Plot smoothed values for Y axis

(10's digit) = 0 : Axes plotted for all subplots

= 1 : Axes plotted only for first subplot

iform    (I): plot format code

selects which dimension of d array is to become

which output axis

plot axis                     plot axis

Code      X  Y  Z  W            Code    X  Y  Z  W

_____    ____________          _____  ____________

1 input: 1  2  3  4             13     3  1  2  4

2 dimen- 1  2  4  3             14     3  1  4  2

3 sion   1  4  2  3             15     3  2  1  4

4 number 1  4  3  2             16     3  2  4  1

5        1  3  2  4             17     3  4  1  2

6        1  3  4  2             18     3  4  2  1

7        2  1  3  4             19     4  1  2  3

8        2  1  4  3             20     4  1  3  2

9        2  4  3  1             21     4  2  1  3

10       2  4  1  3             22     4  2  3  1

11       2  3  4  1             23     4  3  1  2

12       2  3  1  4             24     4  3  2  1

w        (R): working array dimensioned at least n(x)*n(y)

xh,yh,zh (R): length of each axis of MVAX3D subplot

ph,pl    (R): page height, length when multiple pages required

fac      (R): shrink factor for subplots (2 == FACTOR(1/2))

iw,iz    (I): plot every iw'th and iz'th subplots

(NOTE: the axis titles, number of characters, start/stop

values are permuted along with d array dimensions)

st,en    (R): arrays containing axes start and end values

t1       (B): title corresponding to 1st dimension of d

nt1      (I): number of characters in title t1

= 0 : no axis plotted

> 0 : axis plotted

t2       (B): title corresponding to 2st dimension of d

nt2      (I): number of characters in title t2

= 0 : no axis plotted

> 0 : axis plotted

t3       (B): title corresponding to 3rd dimension of d

nt3      (I): number of characters in title t3

7-39

MASTER SUBROUTINES

= 0 : no axis plotted

> 0 : axis plotted

t4       (B): title corresponding to 4th dimension of d

nt4      (I): number of characters in title t4

= 0 : no axis plotted

> 0 : axis plotted

t5       (B): title corresponding to 5th dimension of d

nt5      (I): number of characters in title t5

= 0 : no axis plotted

> 0 : axis plotted

(NOTE: the following optional parameters are required if

iax<0 or mod(iflag,10)=1)

dm,dx (R): minimum and maximum scale values of plot

ic    (I): color array

ic(1) : color for axis lines

ic(2) : color for axis numbers

ic(3) : color for axis titles

ic(4) : color for axis exponents

ic(5) : color for plot surface (return)

7-40

MASTER SUBROUTINES

7.18  SUBROUTINE PHIST

PHIST  is  a  generalized  histogram  plotting  program  with

various   plot  format  controls.   Shading  may  done  under  the

histogram columns.  The histogram height may be plotted on top  of

each   histogram  column  vertically.   The  mean  and  +/-  sigma

variations may be indicated on the plot output.  Note:  PHIST does

not do the histogramming.  This is left to the user.

CALL PHIST(a,n,t,nt,xl,yl,s,ns,xt,nxt,xm,xx,ax,

iflag,ishad,am,as<,ic>)

a     (R): array of heights of histogram columns

n     (I): number of points in a array to plot

t     (B): title string

nt    (I): number of characters in t

= 0 : no title

< 0 : use color array

xl    (R): length of x axis in inches (integer-valued real number)

yl    (R): length of y axis in inches (integer-valued real number)

s     (B): subtitle string

ns    (I): number of characters in s

xt    (B): x axis title

nxt   (I): number of characters in xt

xm    (R): minimum value displayed on x axis

xx    (R): maximum value displayed on x axis

ax    (R): maximum value of a

iflag (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit)  = 2 : plot number value of column on top of column

= 1 : no numeric values on top of histogram columns

= 0 : no axis or title (histogram columns only)

(10's digit) = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

ishad (I): shade option flag

< 0 : shade with solid line

= 0 : no shading

> 0 : shade with line of type ishad

am    (R): mean value of histogram to plot (in relation to xm,xx)

as    (R): standard deviation of histogram to plot

< 0 : neither mean nor sigma values indicated on plot

= 0 : only mean value indicated on plot

> 0 : mean and +/- sigma values indicated on plot

ic    (I): color list

ic(1) : color for axis lines

ic(2) : color for axis numbers

ic(3) : color for axis titles

ic(4) : color for axis exponents

ic(5) : color for mean label

ic(6) : color for sigma label

7-41

MASTER SUBROUTINES

ic(7) : color for title

ic(8) : color for subtitle

ic(9) : color for histogram columns (return)

common /cphist/xmr,dxr,ymr,dyr

xmr   (R): returned value of xmin

dxr   (R): returned value of scale factor (xmax-xmin)/xlen

ymr   (R): returned value of ymin

dyr   (R): returned value of scale factor (ymax-ymin)/ylen

7-42

MASTER SUBROUTINES

7.19  SUBROUTINE PICHRT

PICHRT plots a circular  pie  chart  with  optionally  shaded

wedges, and descriptive legends.  One or more of the slices may be

"exploded" outward from the pie chart center  for  emphasis.   The

chart  legend  can  be  character labels only or can include shade

legends.  Labels can be automatically placed around the pie  chart

or  located  at  the bottom of the page.  Optionally, the user can

specify the locations of the legends.

CALL PICHRT(x,y,r,d,iflag,as,ae,n,a,sh,iw,l,nl,cs,sl,

t,nt,tcs,d,<p,<ic>>)

x,y   (R): location of pie chart center

r     (R): radius of pie chart segments (r>0)

d     (R): distance from chart center for "exploded" segments

iflag (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit)  = 1 : color array not used

= 2 : color array used

(100's)      = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

as    (R): starting angle of first segment

n     (I): number of segments (n>0)

a     (R): array of segment sizes (angular width of jth segment

is a(j)*360/(Sum a(i),i=1,N))

sh    (R): shade option for each pie segment

sh            shade pattern

____       _________________________

0               no shading

1          -45 deg solid lines

2          horizontal solid lines

3          +45 deg solid lines

4          vertical deg solid lines

5          -45 deg dotted lines

6          horizontal dotted lines

7          +45 deg dotted lines

8          vertical deg dotted lines

9          +/- 45 deg dotted lines

10          vertical/horizontal dotted lines

11          +/- 45 deg solid lines

12          vertical/horizontal solid lines

iw    (I): array of segment outline linewidths (1-9)

l     (C): array of segment labels for legend (CHARACTER)

nc    (I): maximum number of characters in legend string

cs    (R): legend character height

sl    (R): legend shaded box size

< 98: legend located below chart w/o box

< 0 : legend located below chart with shaded box

= 0 : no legend

7-43

MASTER SUBROUTINES

> 0 : legend located around chart with shaded box

> 98: legend located around chart w/o box

t     (B): chart title string

nt    (I): number of characters in t

< 1 : no title plotted

tcs   (R): chart title string height

d     (R): distance between shading lines

p     (R): array containing locations of lower-left corner

of legend box/string (used only if tc < 0)

ic    (I): color array (used only if mod(|iflag|,10)=2)

ic(1) : title color (return)

ic(2) : legend string color

ic(3) : segment 1 color

ic(4) : segment 2 color

...         ...

7-44

MASTER SUBROUTINES

7.20  SUBROUTINE PLOTLG

PLOTLG is a very simple routine which plots  a  single  curve

defined   in   x,y   with  log  and/or  linear  scaling  including

appropriate axes and  plot  title.   Various  options  select  the

format of plotting and type of grid.

CALL PLOTLG(x,y,n,iflag,xl,yl,xt,nxt,yt,nyt,t,nt,

<xm,xx,ym,yx<<,ic>,l>>)

x     (R): array of x values

y     (R): array of y values

n     (I): number of points in x array

iflag (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit)  = 1 : plot x linear, y logarithmic (base 10)

= 2 : plot x logarithmic, y linear

= 3 : plot x logarithmic, y logarithmic

= 4 : plot x linear, y linear

(10's digit) = 0 : no axes or title plotted

= 1 : axes with axis line/ticks on top and sides

= 2 : plot solid cartesian grid

= 3 : plot ticked cartesian grid without box

= 4 : plot ticked cartesian grid with box

= 5 : ticked cartesian grid, box w/axis ticks

= 6 : plot without box or grid

= 7 : plot solid logarithmic grid

= 8 : plot dotted logarithmic grid

= 9 : plot ticked logarithmic grid

(100's)      = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

xl    (R): x axis length in inches (integer-valued)

< 0 : use input scaling in xm,xx

> 0 : use auto scaling computed from input array

yl    (R): y axis length in inches (integer-valued)

< 0 : use input scaling in ym,yx

> 0 : use auto scaling computed from input array

xt    (B): x axis title string

nxt   (I): number of characters in xt

< 0 : axis ticks on top of x axis

= 0 : no axis

> 0 : axis ticks on bottom of x axis (normal)

yt    (B): y axis title string

nyt   (I): number of characters in yt

< 0 : axis ticks on right of y axis

= 0 : no axis

> 0 : axis ticks on left of y axis (normal)

t     (B): plot title string (limited to 99 characters)

nt    (I): number of characters in t

< 0 : use color array

7-45

MASTER SUBROUTINES

= 0 : no title

> 0 : do not use color array

if |nt|/100 > 0 : use line type list

xm    (R): minimum value of x array (required if xl or nt < 0)

xx    (R): maximum value of x array (required if xl or nt < 0)

ym    (R): minimum value of y array (required if xl, yl or nt<0)

yx    (R): maximum value of y array (required if xl, yl or nt<0)

(NOTE: optional color array required if nt<0 or |nt|/100>0)

ic    (I): color array

ic(1) : color for grid

ic(2) : color for axis lines

ic(3) : color for axis numbers

ic(4) : color for axis titles

ic(5) : color for axis exponents

ic(6) : color for plotted line

ic(7) : color for title (return)

(NOTE: optional line type array only required if |nt|/100>0)

l     (I): line type of data line

if |nt|/100 > 0 : use line type list

common /cplotlg/xmr,dxr,ymr,dyr

xmr   (R): returned value of xmin

dxr   (R): returned value of scale factor (xmax-xmin)/xlen

ymr   (R): returned value of ymin

dyr   (R): returned value of scale factor (ymax-ymin)/ylen

7-46

MASTER SUBROUTINES

7.21  SUBROUTINE PLOTLG2

PLOTLG2 is a simple routine to plot multiple  curves  defined

in x,y in log and/or linear scaling including appropriate axes and

plot title.  Various options select the format of plotting.   This

subroutine  is  designed  to plot many y value curves which may be

distinguished by color and/or line type.  This routine is  similar

to PLOTLGX but uses a simpler axis specification.

CALL PLOTLG2(x,y,nl,np,iflag,isym,xl,yl,xt,nxt,yt,nyt,t,nt,<xm,

xx,ym,yx<<,ic>,l>>)

x     (R): array of x values

y     (R): array of y values dimensioned y(np,nl)

nl    (I): number of data lines to plot from y array

np    (I): number of points in x array

iflag (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit)  = 1 : plot x linear, y logarithmic (base 10)

= 2 : plot x logarithmic, y linear

= 3 : plot x logarithmic, y logarithmic

= 4 : plot x linear, y linear

(10's digit) = 0 : no axes or title plotted

= 1 : axes with axis line/ticks on top and sides

= 2 : plot solid cartesian grid

= 3 : plot ticked cartesian grid without box

= 4 : plot ticked cartesian grid with box

= 5 : ticked cartesian grid, box w/axis ticks

= 6 : plot without box or cartesian grid

= 7 : plot solid logarithmic grid

= 8 : plot dotted logarithmic grid

= 9 : plot ticked logarithmic grid

(100's)      = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

isym  (I): plot a symbol every isym'th point

< 0 : symbols only plotted, no line

= 0 : no symbols, line only

> 0 : symbol plotted every isym'th point

xl    (R): x axis length in inches (integer-valued)

< 0 : use input scaling in xm,xx

> 0 : use auto scaling computed from input array

yl    (R): y axis length in inches (integer-valued)

< 0 : use input scaling in ym,yx

> 0 : use auto scaling computed from input array

xt    (B): x axis title string

nxt   (I): number of characters in xt

< 0 : axis ticks on top of x axis

= 0 : no axis

> 0 : axis ticks on bottom of x axis (normal)

yt    (B): y axis title string

7-47

MASTER SUBROUTINES

nyt   (I): number of characters in yt

< 0 : axis ticks on right of y axis

= 0 : no axis

> 0 : axis ticks on left of y axis (normal)

t     (B): plot title string (limited to 99 characters)

nt    (I): number of characters in t

< 0 : use color array

= 0 : no title

> 0 : do not use color array

if |nt|/100 > 0 : use line type list

xm    (R): minimum value of x array (required if xl or nt < 0)

xx    (R): maximum value of x array (required if xl or nt < 0)

ym    (R): minimum value of y array (required if xl, yl or nt<0)

yx    (R): maximum value of y array (required if xl, yl or nt<0)

(NOTE: color array required if nt < 0 or |nt|/100 >0)

ic    (I): color list

ic(1) : color for grid

ic(2) : color for axis lines

ic(3) : color for axis numbers

ic(4) : color for axis titles

ic(5) : color for axis exponents

ic(6) : color for title (return)

ic(7) : color for plotted line 1

ic(8) : color for plotted line 2

ic(9) :    etc.

(NOTE: line type list required only if |nt|/100>0)

l     (I): line type for data lines list

common /cplotlg2/xmr,dxr,ymr,dyr

xmr   (R): returned value of xmin

dxr   (R): returned value of scale factor (xmax-xmin)/xlen

ymr   (R): returned value of ymin

dyr   (R): returned value of scale factor (ymax-ymin)/ylen

7-48

MASTER SUBROUTINES

7.22  SUBROUTINE PLOTLGX

PLOTLGX is a routine for ploting multiple curves  defined  in

x,y  with log and/or linear scaling including appropriate axes and

plot title.  Various options may be  used  select  the  format  of

plotting.   This  routine  may be used to plot many y value curves

simultaneously with the curves distinguished  by  symbols,  color,

and/or   line   type.   This  subroutine  permits  axis  parameter

flexibility and dimensioning for the y array values.

CALL PLOTLGX(x,y,nld,npd,nl,np,iflag,isym,xl,yl,

nmx,nnx,mlx,tsx,ndx,smx,

nmy,nny,mly,tsy,ndy,smy,

xt,nxt,yt,nyt,t,nt,<xm,xx,ym,yx<<,ic>,l>>)

x     (R): array of x values dimensioned at least x(np)

y     (R): array of y values dimensioned y(npd,nld)

nld   (I): dimension of y array (lines)

npd   (I): dimension of y array (points)

nl    (I): number of data lines to plot from y array

np    (I): number of points per line

iflag (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit)  = 1 : plot x linear, y logarithmic (base 10)

= 2 : plot x logarithmic, y linear

= 3 : plot x logarithmic, y logarithmic

= 4 : plot x linear, y linear

(10's digit) = 0 : no axes or title plotted

= 1 : axes with axis line/ticks on top and sides

= 2 : plot solid cartesian grid

= 3 : plot ticked cartesian grid without box

= 4 : plot ticked cartesian grid with box

= 5 : plot ticked cartesian grid, box w/axis ticks

= 6 : plot without box or cartesian grid

= 7 : plot solid logarithmic grid

= 8 : plot dotted logarithmic grid

= 9 : plot ticked logarithmic grid

(100's)      = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

isym  (I): plot a symbol every isym'th point

< 0 : symbols only plotted, no line

= 0 : no symbols, line only

> 0 : symbol plotted every isym'th point

xl    (R): x axis length in inches

< 0 : use input scaling in xm,xx

> 0 : use auto scaling computed from input array

yl    (R): y axis length in inches

< 0 : use input scaling in ym,yx

> 0 : use auto scaling computed from input array

(see AXIS2 for detailed description of axis parameters)

7-49

MASTER SUBROUTINES

nmx   (I): number of minor ticks between major ticks on x axis

nnx   (I): highlight length of nnx-th minor tick on x axis

mlx   (I): number of major tick marks on x axis

tsx   (R): size of title and numbers on x axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

ndx   (I): number of digits to right of decimal point on x axis

smx   (R): major tick length on x axis

nmy   (I): number of minor ticks between major ticks on y axis

nny   (I): highlight length of nny-th minor tick on y axis

mly   (I): number of major tick marks on y axis

tsy   (R): size of title and numbers on y axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

ndy   (I): number of digits to right of decimal point on y axis

smy   (R): major tick length on y axis

xt    (B): x axis title string

nxt   (I): number of characters in xt

< 0 : axis ticks on top of x axis

= 0 : no axis

> 0 : axis ticks on bottom of x axis (normal)

yt    (B): y axis title string

nyt   (I): number of characters in yt

< 0 : axis ticks on right of y axis

= 0 : no axis

> 0 : axis ticks on left of y axis (normal)

t     (B): plot title string (limited to 99 characters)

nt    (I): number of characters in t

< 0 : use color array

= 0 : no title

> 0 : do not use color array

if |nt|/100 > 0 : use line type list

xm    (R): minimum value of x array (required if xl or nt < 0)

xx    (R): maximum value of x array (required if xl or nt < 0)

ym    (R): minimum value of y array (required if xl, yl or nt<0)

yx    (R): maximum value of y array (required if xl, yl or nt<0)

(NOTE: color array required if nt < 0 or |nt|/100 >0)

ic    (I): color list

ic(1) : color for grid

ic(2) : color for axis lines

ic(3) : color for axis numbers

ic(4) : color for axis titles

ic(5) : color for axis exponents

ic(6) : color for title (return)

ic(7) : color for plotted line 1

ic(8) : color for plotted line 2

ic(9) :    etc.

l     (I): data line type list  (required only if |nt|/100 > 0)

common /cplotlgx/xmr,dxr,ymr,dyr

xmr   (R): returned value of xmin

dxr   (R): returned value of scale factor (xmax-xmin)/xlen

7-50

MASTER SUBROUTINES

ymr   (R): returned value of ymin

dyr   (R): returned value of scale factor (ymax-ymin)/ylen

7-51

MASTER SUBROUTINES

7.23  SUBROUTINE PLOTLGL

PLOTLGL plots multiple curves defined in x,y with log  and/or

linear  scaling  including  appropriate  axes and plot title using

software line types.  It  is  similar  in  character  to  PLOTLGX.

Various  options  may be used select the format of plotting.  This

routine may be used to plot many  y  value  curves  simultaneously

with the curves distinguished by symbols, color, and/or line type.

This   subroutine   permits   axis   parameter   flexibility   and

dimensioning for the y array values.

NOTE:  the values in the x and y arrays are  modified.   Upon

return  they contain their original contents scaled by xm,dx,ym,dy

(see LINE).

CALL PLOTLGL(x,y,w,nld,npd,nl,np,iflag,isym,xl,yl,ns,s,l,

nmx,nnx,mlx,tsx,ndx,smx,

nmy,nny,mly,tsy,ndy,smy,

xt,nxt,yt,nyt,t,nt,<xm,xx,ym,yx<,ic>>)

x     (R): array of x values dimensioned at least x(np)

y     (R): array of y values dimensioned y(npd,nld)

w     (R): working array dimensioned at least d(3*np+3)

nld   (I): dimension of y array (lines)

npd   (I): dimension of y array (points)

nl    (I): number of data lines to plot from y array

np    (I): number of points per data line

iflag (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit)  = 1 : plot x linear, y logarithmic (base 10)

= 2 : plot x logarithmic, y linear

= 3 : plot x logarithmic, y logarithmic

= 4 : plot x linear, y linear

(10's digit) = 0 : no axes or title plotted

= 1 : axes with axis line/ticks on top and sides

= 2 : plot solid cartesian grid

= 3 : plot ticked cartesian grid without box

= 4 : plot ticked cartesian grid with box

= 5 : ticked cartesian grid, box w/axis ticks

= 6 : plot without box or cartesian grid

= 7 : plot solid logarithmic grid

= 8 : plot dotted logarithmic grid

= 9 : plot ticked logarithmic grid

(100's)      = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

isym  (I): plot a symbol every isym'th point

< 0 : symbols only plotted, no line

= 0 : no symbols, line only

> 0 : symbol plotted every isym'th point

xl    (R): x axis length in inches

7-52

MASTER SUBROUTINES

< 0 : use input scaling in xm,xx

> 0 : use auto scaling computed from input array

yl    (R): y axis length in inches

< 0 : use input scaling in ym,yx

> 0 : use auto scaling computed from input array

ns    (I): smoothing passes (normally zero--see LINSEQ)

s     (R): nominal interval length (see LINSEQ)

l     (I): linetype array dimensioned l(5*np) (see LINSEQ)

l(1): l1 for line 1

l(2): l2 for line 1

...

l(5): l5 for line 1

l(6): l1 for line 2

l(7): l2 for line 2

...

(see AXIS2 for detailed description of axis parameters)

nmx   (I): number of minor ticks between major ticks on x axis

nnx   (I): highlight length of nnx-th minor tick on x axis

mlx   (I): number of major tick marks on x axis

tsx   (R): size of title and numbers on x axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

ndx   (I): number of digits to right of decimal point on x axis

smx   (R): major tick length on x axis

nmy   (I): number of minor ticks between major ticks on y axis

nny   (I): highlight length of nny-th minor tick on y axis

mly   (I): number of major tick marks on y axis

tsy   (R): size of title and numbers on y axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

ndy   (I): number of digits to right of decimal point on y axis

smy   (R): major tick length on y axis

xt    (B): x axis title string

nxt   (I): number of characters in xt

< 0 : axis ticks on top of x axis

= 0 : no axis

> 0 : axis ticks on bottom of x axis (normal)

yt    (B): y axis title string

nyt   (I): number of characters in yt

< 0 : axis ticks on right of y axis

= 0 : no axis

> 0 : axis ticks on left of y axis (normal)

t     (B): plot title string (limited to 99 characters)

nt    (I): number of characters in t

< 0 : use color array

= 0 : no title

> 0 : do not use color array

if |nt|/100 > 0 : use line type list

xm    (R): minimum value of x array (required if xl or nt < 0)

xx    (R): maximum value of x array (required if xl or nt < 0)

ym    (R): minimum value of y array (required if xl, yl or nt<0)

yx    (R): maximum value of y array (required if xl, yl or nt<0)

(NOTE: color array required if nt < 0)

7-53

MASTER SUBROUTINES

ic    (I): color array

ic(1) : color for grid

ic(2) : color for axis lines

ic(3) : color for axis numbers

ic(4) : color for axis titles

ic(5) : color for axis exponents

ic(6) : color for title (return)

ic(7) : color for plotted line 1

ic(8) : color for plotted line 2

ic(9) :    etc.

common /cplotlgl/xmr,dxr,ymr,dyr

xmr   (R): returned value of xmin

dxr   (R): returned value of scale factor (xmax-xmin)/xlen

ymr   (R): returned value of ymin

dyr   (R): returned value of scale factor (ymax-ymin)/ylen

7-54

MASTER SUBROUTINES

7.24  SUBROUTINE PLOTSC

PLOTSC is a very basic routine which plots the curve  defined

in  x,y  with  axes  and a plot title.  Various options select the

format of the plot.

CALL PLOTSC(x,y,n,iflag,xl,yl,xt,nxt,yt,nyt,t,nt,

<xm,xx,ym,yx<<,ic>,l>>)

x     (R): array of x values

y     (R): array of y values

n     (I): number of points in x array

iflag (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit) = 0 : no axis or title plotted

= 1 : axes with axis line/ticks on top and sides

= 2 : plot solid cartesian grid

= 3 : plot ticked grid without box

= 4 : plot ticked grid with box

= 5 : ticked grid and box with axis tick marks

= 6 : plot without box or grid

(10's)      = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

xl    (R): x axis length in inches (integer valued)

< 0 : use input scaling in xm,xx

> 0 : use auto scaling computed from input array

yl    (R): y axis length in inches (integer valued)

< 0 : use input scaling in ym,yx

> 0 : use auto scaling computed from input array

xt    (B): x axis title string

nxt   (I): number of characters in xt

< 0 : axis ticks on top of x axis

= 0 : no axis

> 0 : axis ticks on bottom of x axis (normal)

yt    (B): y axis title string

nyt   (I): number of characters in yt

< 0 : axis ticks on right of y axis

= 0 : no axis

> 0 : axis ticks on left of y axis (normal)

t     (B): plot title string (limited to 99 characters)

nt    (I): number of characters in t

< 0 : use color array

= 0 : no title

> 0 : do not use color array

if |nt|/100 > 0 : use line type list

xm    (R): minimum value of x array (required if xl or nt < 0)

xx    (R): maximum value of x array (required if xl or nt < 0)

ym    (R): minimum value of y array (required if xl, yl or nt<0)

yx    (R): maximum value of y array (required if xl, yl or nt<0)

(NOTE: color array required if nt < 0 or |nt|/100 >0)

7-55

MASTER SUBROUTINES

ic    (I): color array

ic(1) : color for grid

ic(2) : color for axis lines

ic(3) : color for axis numbers

ic(4) : color for axis titles

ic(5) : color for axis exponents

ic(6) : color for plotted line

ic(7) : color for title (return)

l     (I): data line type (required if nt < 0 or |nt|/100 > 0)

common /cplotsc/xmr,dxr,ymr,dyr

xmr   (R): returned value of xmin

dxr   (R): returned value of scale factor (xmax-xmin)/xlen

ymr   (R): returned value of ymin

dyr   (R): returned value of scale factor (ymax-ymin)/ylen

7-56

MASTER SUBROUTINES

7.25  SUBROUTINE PLOTSC2

PLOTSC2 plots two curves defined by the x,y and  x,y2  arrays

with  axes  and  a  plot title.  An auto-scaling option scales the

axes  to  place  both  curves  within  the   axes.    Curves   are

distinguished  by line type.  The y curve uses a solid line (LINE)

while the y2 curve uses a dashed line  (DASHL).   Various  options

select the format of plotting.

CALL PLOTSC2(x,y,y2,n,iflag,xl,yl,xt,nxt,yt,nyt,t,nt,

<xm,xx,ym,yx<<,ic>,l>>)

x     (R): array of x values

y     (R): array of y values (plotted solid)

y2    (R): second array of y values (plotted dashed)

n     (I): number of points in x array

iflag    (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) >10000 : do not initialize LONGLIB before plotting

(1's digit) = 0 : no axis or title plotted

= 1 : axes with axis line/ticks on top and sides

= 2 : plot solid cartesian grid

= 3 : plot ticked grid without box

= 4 : plot ticked grid with box

= 5 : plot ticked grid and box with axis tick marks

= 6 : plot without grid or box

(1's digit)  = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

xl    (R): x axis length in inches (integer valued)

< 0 : use input scaling in xm,xx

> 0 : use auto scaling computed from input array

yl    (R): y axis length in inches (integer valued)

< 0 : use input scaling in ym,yx

> 0 : use auto scaling computed from input array

xt    (B): x axis title string

nxt   (I): number of characters in xt

< 0 : axis ticks on top of x axis

= 0 : no axis

> 0 : axis ticks on bottom of x axis (normal)

yt    (B): y axis title string

nyt   (I): number of characters in yt

< 0 : axis ticks on right of y axis

= 0 : no axis

> 0 : axis ticks on left of y axis (normal)

t     (B): plot title string (limited to 99 characters)

nt    (I): number of characters in t

< 0 : use color array

= 0 : no title

> 0 : do not use color array

if |nt|/100 > 0 : use line type list

xm    (R): minimum value of x array (required if xl or nt < 0)

7-57

MASTER SUBROUTINES

xx    (R): maximum value of x array (required if xl or nt < 0)

ym    (R): minimum value of y array (required if xl, yl or nt<0)

yx    (R): maximum value of y array (required if xl, yl or nt<0)

(NOTE: color array required if nt < 0 or |nt|/100 >0)

ic    (I): color list

ic(1) : color for grid

ic(2) : color for axis line

ic(3) : color for axis numbers

ic(4) : color for axis titles

ic(4) : color for axis exponents

ic(5) : color for plotted line 1

ic(6) : color for plotted line 2

ic(7) : color for title (return)

l     (I): data line type list  (required only if |nt|/100 > 0)

common /cplotsc2/xmr,dxr,ymr,dyr

xmr   (R): returned value of xmin

dxr   (R): returned value of scale factor (xmax-xmin)/xlen

ymr   (R): returned value of ymin

dyr   (R): returned value of scale factor (ymax-ymin)/ylen

7-58

MASTER SUBROUTINES

7.26  SUBROUTINE SCATPL

SCATPL plots data point pairs (x,y) in a scatter plot  format

using  log  and/or  linear  scaling including appropriate axes and

plot title.  Several different sets of data may be plotted on  the

same plot by specifying different plotting symbols for each set of

data points.  Various options select the format of  plotting.   If

nl=1 then the x and y arrays may be 1d arrays.

CALL SCATPL(x,y,nl,np,iflag,nsym,s,xl,yl,xt,nxt,yt,nyt,

t,nt,<xm,xx,ym,yx<,ic>>)

x     (R): array of x values dimensioned x(np,nl)

y     (R): array of y values dimensioned y(np,nl)

nl    (I): number of symbol types (if nl=1, x,y may be 1d arrays)

np    (I): number of data point pairs (x,y) of same symbol type

iflag (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit)  = 1 : plot x linear, y logarithmic (base 10)

= 2 : plot x logarithmic, y linear

= 3 : plot x logarithmic, y logarithmic

= 4 : plot x linear, y linear

(10's digit) = 0 : no axes or title plotted

= 1 : axes with axis line/ticks on top and sides

= 2 : plot solid cartesian grid

= 3 : plot ticked cartesian grid without box

= 4 : plot ticked cartesian grid with box

= 5 : plot ticked cartesian grid, box w/axis ticks

= 6 : plot without box or cartesian grid

= 7 : plot solid logarithmic grid

= 8 : plot dotted logarithmic grid

= 9 : plot ticked logarithmic grid

(100's)      = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

nsym  (I): array of symbols numbers dimensioned nsym(nl)

nsym(n) <  0 : dots only plotted, no symbols line n

nsym(n) >= 0 : plot symbol number for line n

s     (R): size of symbols (if s <= 0, 0.1 is used)

xl    (R): x axis length in inches (integer-valued)

< 0 : use input scaling in xm,xx

> 0 : use auto scaling computed from input array

yl    (R): y axis length in inches (integer-valued)

< 0 : use input scaling in ym,yx

> 0 : use auto scaling computed from input array

xt    (B): x axis title string

nxt   (I): number of characters in xt

< 0 : axis ticks on top of x axis

= 0 : no axis

> 0 : axis ticks on bottom of x axis (normal)

yt    (B): y axis title string

7-59

MASTER SUBROUTINES

nyt   (I): number of characters in yt

< 0 : axis ticks on right of y axis

= 0 : no axis

> 0 : axis ticks on left of y axis (normal)

t     (B): plot title string

nt    (I): number of characters in t

< 0 : use color array

= 0 : no title

> 0 : do not use color array

xm    (R): minimum value of x array (required if xl or nt < 0)

xx    (R): maximum value of x array (required if xl or nt < 0)

ym    (R): minimum value of y array (required if xl,yl or nt<0)

yx    (R): maximum value of y array (required if xl,yl or nt<0)

(NOTE: color array required if nt < 0 or |nt|/100 >0)

ic    (I): color list

ic(1) : color for grid

ic(2) : color for axis lines

ic(3) : color for axis numbers

ic(4) : color for axis titles

ic(5) : color for axis exponents

ic(6) : color for title (return)

ic(7) : color for plotted line 1

ic(8) : color for plotted line 2

ic(9) :    etc.

common /cscatpl/xmr,dxr,ymr,dyr

xmr   (R): returned value of xmin

dxr   (R): returned value of scale factor (xmax-xmin)/xlen

ymr   (R): returned value of ymin

dyr   (R): returned value of scale factor (ymax-ymin)/ylen

7-60

MASTER SUBROUTINES

7.27  SUBROUTINE SEISPL

SEISPL allows for plotting data in the special formats often  used

in  seismic  data processing using log and/or linear axis scaling.

Appropriate axes and plot titles may be  included.   The  plotting

format  may  be selected by the value of ntype.  Possible plotting

formats include:  multiple "shaded"  waveforms,  connected  lines,

vertical  line  plots,  etc.   Each  line  may  be offset from the

previous line by a specified value for presentation on the plot (z

array).   A line may be added to indicate the zero value, etc.  If

nl=1 then the x and y arrays may be 1d arrays.

CALL SEISPL(x,y,z,nld,npd,nl,np,iflag,ntype,size,zref,xl,yl,

xt,nxt,yt,nyt,t,nt,<xm,xx,ym,yx<,ic>>)

x     (R): array of x values dimensioned x(npd)

y     (R): array of y values dimensioned y(npd,nld)

z     (R): array of y-offset values dimensioned z(nl).  Y value is

offset by the z value before plotting.  When using log

plotting note that offset occurs after taking logs.

nld   (I): dimension of y array

npd   (I): dimension of x,y arrays

nl    (I): number of lines plotted (if nl=1, x,y may be 1d arrays)

np    (I): number of data points to plot per line

iflag (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit)  = 1 : plot x linear, y logarithmic (base 10)

= 2 : plot x logarithmic, y linear

= 3 : plot x logarithmic, y logarithmic

= 4 : plot x linear, y linear

(10's digit) = 0 : no axes or title plotted

= 1 : axes with axis line/ticks on top and sides

= 2 : plot solid cartesian grid

= 3 : plot ticked cartesian grid without box

= 4 : plot ticked cartesian grid with box

= 5 : plot ticked cartesian grid, box w/axis ticks

= 6 : plot without box or cartesian grid

= 7 : plot solid logarithmic grid

= 8 : plot dotted logarithmic grid

= 9 : plot ticked logarithmic grid

(100's)      = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

ntype (I): plot format control

< 0 : line with zref plotted

= 0 : symbols only

> 0 : line without zref plotted

(magnitude) =  1 : symbols only plotted

=  2 : points only plotted

=  3 : connected points plotted

=  4 : vertical lines from points to zref line plotted

7-61

MASTER SUBROUTINES

=  5 : vertical lines plus symbol at point plotted

=  6 : vertical lines and connected points plotted

=  7 : connected points and lines on + side of zref

=  8 : connected points and lines on - side of zref

=  9 : area between connected points and zref filled

= 10 : positive area filled

= 11 : negative area filled

size  (R): size of symbols (ntype : 0,1,4)

spacing between area fill lines (ntype : 9,10,11)

< 0 : indicates center line to be dotted

> 0 : indicates center line solid (if plotted)

zref  (R): offset added to all z values

xl    (R): x axis length in inches

< 0 : use input scaling in xm,xx

> 0 : use auto scaling computed from input array

yl    (R): y axis length in inches

< 0 : use input scaling in ym,yx

> 0 : use auto scaling computed from input array

xt    (B): x axis title string

nxt   (I): number of characters in xt

< 0 : axis ticks on top of x axis

= 0 : no axis

> 0 : axis ticks on bottom of x axis (normal)

yt    (B): y axis title string

nyt   (I): number of characters in yt

< 0 : axis ticks on right of y axis

= 0 : no axis

> 0 : axis ticks on left of y axis (normal)

t     (B): plot title string

nt    (I): number of characters in t

< 0 : use color array

= 0 : no title

> 0 : do not use color array

xm    (R): minimum value of x array (required if xl or nt < 0)

xx    (R): maximum value of x array (required if xl or nt < 0)

ym    (R): minimum value of y array (required if xl,yl or nt<0)

yx    (R): maximum value of y array (required if xl,yl or nt<0)

(NOTE: color array required if nt < 0)

ic    (I): color array

ic(1) : color for grid

ic(2) : color for axis lines

ic(3) : color for axis numbers

ic(4) : color for axis titles

ic(5) : color for axis exponents

ic(6) : color for title (return)

ic(7) : color for plotted line 1

ic(8) : color for plotted line 2

ic(9) :    etc.

common /cseispl/xmr,dxr,ymr,dyr

xmr   (R): returned value of xmin

dxr   (R): returned value of scale factor (xmax-xmin)/xlen

7-62

MASTER SUBROUTINES

ymr   (R): returned value of ymin

dyr   (R): returned value of scale factor (ymax-ymin)/ylen

7-63

MASTER SUBROUTINES

7.28  SUBROUTINE SPLOTS

SPLOTS plots points and/or curves defined by x,y pairs in log

and/or  linear  scaling  with  option  error  bars.   SPLOTS calls

SPLOTSX and is provided to simplify axis specification.

CALL SPLOTS(x,y,nld,npd,nl,np,iflag,nopt,as,xl,yl,

xt,nxt,yt,nyt,t,nt,<xm,xx,ym,yx<,ic>>)

see SPLOTSX for variable description

common /csplots/xmr,dxr,ymr,dyr

xmr   (R): returned value of xmin

dxr   (R): returned value of scale factor (xmax-xmin)/xlen

ymr   (R): returned value of ymin

dyr   (R): returned value of scale factor (ymax-ymin)/ylen

7.29  SUBROUTINE SPLOTSX

Appropriate axes and plot title  may  be  included.   Various

options  select  the  plotting  format of plotting.  Possible plot

formats include scatter plots, connected line  points  with  error

bars  shown,  points  with  error bars, a displacement line from x

axis, etc.

This subroutine may be used to plot several sets  of  curves.

For a given set (or line) of points, the upper error bar value may

be given in the next set (or line) of points.  The lower error bar

value may be given in the following set (or line).  When the error

bar option is used, the value of nl should be the number of points

sets  (or  lines)  to be plotted not including the error bar sets.

This   subroutine   permits   axis   parameter   flexibility   and

dimensioning for y array.

CALL SPLOTSX(x,y,nld,npd,nl,np,iflag,nopt,as,xl,yl,

nmx,nnx,mlx,tsx,ndx,smx,nmy,nny,mly,tsy,ndy,smy,

xt,nxt,yt,nyt,t,nt,<xm,xx,ym,yx<,ic>>)

x     (R): array of x values dimensioned at least x(np)

y     (R): array of y values dimensioned y(npd,nld)

npd   (I): dimension of y

nld   (I): dimension of y

nl    (I): number of y lines to plot (see note below)

np    (I): number of points per line

iflag (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit)  = 1 : plot x linear, y logarithmic (base 10)

7-64

MASTER SUBROUTINES

= 2 : plot x logarithmic, y linear

= 3 : plot x logarithmic, y logarithmic

= 4 : plot x linear, y linear

(10's digit) = 0 : no axes or title plotted

= 1 : axes with axis line/ticks on top and sides

= 2 : plot solid cartesian grid

= 3 : plot ticked cartesian grid without box

= 4 : plot ticked cartesian grid with box

= 5 : ticked cartesian grid, box w/axis ticks

= 6 : plot without box or cartesian grid

= 7 : plot solid logarithmic grid

= 8 : plot dotted logarithmic grid

= 9 : plot ticked logarithmic grid

(100's)      = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

nopt  (I): option flag

(1's digit)  = 0 : disconnected points

= 1 : connected points

= 2 : disconnected symbols

= 3 : connected symbols

= 4 : vertical line from point to x axis

= 5 : connect points, add vertical line from each

data point to x axis

= 6 : symbol plus vertical line from point to x axis

(10's digit) = 0 : no error bars

= 1 : error bars (see above)

= 2 : error bars witout end bars (see above)

(100's)      > 0 : every (*) point shown with a symbol

(0 equivalent to 1)

as    (R): size of plotted symbol and/or error bar

xl    (R): x axis length in inches

< 0 : use input scaling in xm,xx

> 0 : use auto scaling computed from input array

yl    (R): y axis length in inches

< 0 : use input scaling in ym,yx

> 0 : use auto scaling computed from input array

(see AXIS2 for detailed description of axis parameters)

nmx   (I): number of minor ticks between major ticks on x axis

nnx   (I): highlight length of nnx-th minor tick on x axis

mlx   (I): number of major tick marks on x axis

tsx   (R): size of title and numbers on x axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

ndx   (I): number of digits to right of decimal point on x axis

smx   (R): major tick length on x axis

nmy   (I): number of minor ticks between major ticks on y axis

nny   (I): highlight length of nny-th minor tick on y axis

mly   (I): number of major tick marks on y axis

tsy   (R): size of title and numbers on y axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

ndy   (I): number of digits to right of decimal point on y axis

smy   (R): major tick length on y axis

7-65

MASTER SUBROUTINES

xt    (B): x axis title string

nxt   (I): number of characters in xt

< 0 : axis ticks on top of x axis

= 0 : no axis

> 0 : axis ticks on bottom of x axis (normal)

yt    (B): y axis title string

nyt   (I): number of characters in yt

< 0 : axis ticks on right of y axis

= 0 : no axis

> 0 : axis ticks on left of y axis (normal)

t     (B): plot title string

nt    (I): number of characters in t

< 0 : use color array

= 0 : no title

> 0 : do not use color array

xm    (R): minimum value of x array (required if xl or nt < 0)

xx    (R): maximum value of x array (required if xl or nt < 0)

ym    (R): minimum value of y array (required if xl,yl or nt<0)

yx    (R): maximum value of y array (required if xl,yl or nt<0)

(NOTE: color array required if nt < 0)

ic    (I): color list

ic(1) : color for grid

ic(2) : color for axis lines

ic(3) : color for axis numbers

ic(4) : color for axis titles

ic(5) : color for axis exponents

ic(6) : color for title (return)

ic(7) : color for points in line 1

ic(8) : color for symbols/error bars line 1

ic(9) : color for points in line 2

ic(10): color for symbols/error bars line 2

ic(11):    etc.

common /csplotsx/xmr,dxr,ymr,dyr

xmr   (R): returned value of xmin

dxr   (R): returned value of scale factor (xmax-xmin)/xlen

ymr   (R): returned value of ymin

dyr   (R): returned value of scale factor (ymax-ymin)/ylen

7-66

MASTER SUBROUTINES

7.30  SUBROUTINE T3DH

T3DH plots a 3-d surface defined on a  uniformly  spaced  2-d

grid  in  the  x,y  plane using a triangular mesh with hidden line

removal.  The surface can be viewed from  any  angle.   The  input

consists  of  the values of the z axis for each (x,y) point on the

plane.  Each square grid is plotted as two triangles using  either

SKETCH  or PLT3DH permitting hidden line removal if desired.  T3DH

calls INIT3DH to initialize the 3-d hidden line package.   Default

space is provided in the INIT3DH common block.  The ISIZE variable

will be reset to the internal size if its value  is  smaller  than

the  internal size.  Sufficient space is provided for very complex

surfaces.

CALL T3DH(z,ndx,ndy,nx,ny,s,p,t,dv,xl,yl,zl,iflag,ia,

xt,nxt,xm,xx,tx,sx,gx,fx,

yt,nyt,ym,yy,ty,sy,gy,fy,

zt,nzt,tz,sz,gz,fz,zm,zx<,ic>)

z      (R): array of z values

ndx,ndy(I): dimensions of z array

nx,ny  (I): number of points in each dimension to use of z array

s,p,t  (R): yaw,roll,pitch angles of axes (see INIT3DH)

dv     (R): perspective scale factor for INIT3DH

< 0 : no hidden lines in SKETCH, INIT3DH called

= 0 : INIT3DH not called to initialize 3d package,

(INIT3DH previously called)

> 0 : hidden lines in SKETCH, INIT3DH called

= 9999 : no perspective in INIT3DH

xl     (R): x axis length in inches

yl     (R): y axis length in inches

zl     (R): z axis length in inches

iflag  (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit)  = 1 : do not use color array

= 2 : use color array

(10's digit) = 0 : hidden line removal for surface only

= 1 : hidden line removal for axis and surface

= 2 : no hidden line removal (SKETCH not used)

(100's)      = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

ia     (I): axis option flag

< 0 : plot axis use input z axis scale

= 0 : do not plot axes

> 0 : plot axis use computed z axis scale

(1's digit)  = 1 : plot y axis using input scale

= 2 : plot y axis using input, smoothed scale

(10's digit) = 1 : plot x axis using input scale

= 2 : plot x axis using input, smoothed scale

xt     (B): x axis title string

7-67

MASTER SUBROUTINES

nxt    (I): number of characters in x axis title

< 0 : axis ticks on top of x axis

= 0 : no axis

> 0 : axis ticks on bottom of x axis (normal)

tx     (R): number and pattern of axis ticks (see AXIS3)

sx     (R): size of axis labeling (see AXIS3)

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

gx     (R): rotation angle for x axis (see AXIS3DH)

fx     (R): format of axis number labeling (see AXIS3)

yt     (B): y axis title string

nyt    (I): number of characters in y axis title

< 0 : axis ticks on top of axis

= 0 : no axis

> 0 : axis ticks on bottom of axis (normal)

ty     (R): number and pattern of axis ticks (see AXIS3)

sy     (R): size of axis labeling (see AXIS3)

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

gy     (R): rotation angle for y axis (see AXIS3DH)

fy     (R): format of axis number labeling (see AXIS3)

zt     (B): z axis title string

nzt    (I): number of characters in z axis title

< 0 : axis ticks on top of axis

= 0 : no axis

> 0 : axis ticks on bottom of axis (normal)

tz     (R): number and pattern of axis ticks (see AXIS3)

sz     (R): size of axis labeling (see AXIS3)

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

gz     (R): rotation angle for z axis (see AXIS3DH)

fz     (R): format of axis number labeling (see AXIS3)

xm,ym  (R): minimum value displayed on each axis

xx,yx  (R): maximum value displayed on each axis

zm     (R): minimum value of z axis (required if ia < 0)

zx     (R): maximum value of z axis (required if ia < 0)

ic     (I): color array (required if 1's digit of iflag=2)

ic(1) : color of axis

ic(2) : surface color

7-68

MASTER SUBROUTINES

7.31  SUBROUTINE TRIG3DH

TRIG3DH plots a 3-d surface specified by randomly scatted set

of  points  as  a  triangular  mesh with hidden line removal.  The

surface may be viewed from any angle.  The  input  consists  of  a

list   of   triplets  defining  the  surface.   The  triplets  are

triangulated using TRIANGC.  Each triangle is plotted using either

SKETCH  or  PLT3DH  permitting  hidden  line  removal  if desired.

TRIG3DH calls INIT3DH to initialize the 3-d hidden  line  package.

Default  space is provided in the INIT3DH common block.  The ISIZE

variable will be reset to  the  internal  size  if  its  value  is

smaller  than the internal size.  Sufficient space is provided for

very complex surfaces.

CALL TRIG3DH(x,y,z,n,s,p,t,dv,xl,yl,zl,iflag,ia,

xt,nxt,tx,sx,gx,fx,

yt,nyt,ty,sy,gy,fy,

zt,nzt,tz,sz,gz,fz,

xm,xx,ym,yx,zm,zx<,ic>)

x,y,z (R): array of point triplets (x,y,z)

n     (I): number of points

s,p,t (R): yaw,roll,pitch angles of axes (see INIT3DH)

dv    (R): perspective scale factor for INIT3DH

< 0 : no hidden lines in SKETCH, INIT3DH called

= 0 : INIT3DH not called to initialize 3d package,

(INIT3DH previously called)

> 0 : hidden lines in SKETCH, INIT3DH called

= 9999 : no perspective in INIT3DH

xl    (R): x axis length in inches

yl    (R): y axis length in inches

zl    (R): z axis length in inches

iflag (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit)  = 1 : do not use color array

= 2 : use color array

(10's digit) = 0 : hidden line removal for surface only

= 1 : hidden line removal for axis and surface

= 2 : no hidden line removal (SKETCH not used)

(100's)      = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

ia    (I): axis option flag

< 0 : do not plot axes

> 0 : plot axes according to the code,

(1's digit)   = 1 : plot z axis using computed scale

= 2 : plot z axis using computed, smoothed scale

= 3 : plot z axis using input scale

input scale variables required

= 4 : plot z axis using input, smoothed scale

(100's digit) = 1 : plot y axis using computed scale

7-69

MASTER SUBROUTINES

= 2 : plot y axis using computed, smoothed scale

= 3 : plot y axis using input scale

input scale variables required

= 4 : plot y axis using input, smoothed scale

(100's digit) = 1 : plot x axis using computed scale

= 2 : plot x axis using computed, smoothed scale

= 3 : plot x axis using input scale

input scale variables required

= 4 : plot x axis using input, smoothed scale

xt     (B): x axis title string

nxt    (I): number of characters in x axis title

< 0 : axis ticks on top of x axis

= 0 : no axis

> 0 : axis ticks on bottom of x axis (normal)

tx     (R): number and pattern of axis ticks (see AXIS3)

sx     (R): size of axis labeling (see AXIS3)

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

gx     (R): rotation angle for x axis (see AXIS3DH)

fx     (R): format of axis number labeling (see AXIS3)

yt     (B): y axis title string

nyt    (I): number of characters in y axis title

< 0 : axis ticks on top of axis

= 0 : no axis

> 0 : axis ticks on bottom of axis (normal)

ty     (R): number and pattern of axis ticks (see AXIS3)

sy     (R): size of axis labeling (see AXIS3)

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

gy     (R): rotation angle for y axis (see AXIS3DH)

fy     (R): format of axis number labeling (see AXIS3)

zt     (B): z axis title string

nzt    (I): number of characters in z axis title

< 0 : axis ticks on top of axis

= 0 : no axis

> 0 : axis ticks on bottom of axis (normal)

tz     (R): number and pattern of axis ticks (see AXIS3)

sz     (R): size of axis labeling (see AXIS3)

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

gz     (R): rotation angle for z axis (see AXIS3DH)

fz     (R): format of axis number labeling (see AXIS3)

xm,xx  (R): minimum, maximum value of x axis

ym,yx  (R): minimum, maximum value of y axis

zm,zx  (R): minimum, maximum value of z axis

ic     (I): color array (required if 1's digit of iflag=2)

ic(1) : color of axis

ic(2) : surface color

7-70

MASTER SUBROUTINES

7.32  SUBROUTINE VAX3D

VAX3D plots a 3-d surface  by  plotting  2-d  slices  of  the

surface  paralel  to  the x-z plane with hidden line removal.  See

VAX3DX.  VAX3D calls  VAX3DX  using  default  axis  parameters  to

simplify calling procedure.

CALL VAX3D(d,ndx,ndz,nx,nz,a,b,xh,yh,zh,iflag,iax,<xt,nxt,xs,xe,

yt,nyt,zt,nzt,zs,ze,<dm,dx<,ic>>>)

See VAX3DX for parameter description.

(iax is limited to a single digit)

7.33  SUBROUTINE VAX3DX

VAX3DX is a simple  3-d  surface  plotting  routine.   A  3-d

surface  is  plotted  by plotting slices through the surface which

are parallel to the x-y plane.  The y value of the surface at  the

intersection  of  the slice plane and the y value plotted.  Hidden

lines are supressed,  giving  the  illusion  of  a  3  dimensional

surface.   The  height  of  plotted surface relative to its y axis

value is calibrated to x and z  axis.   No  perspective  is  used.

Options exist to varying the plotting angle and to plot axes.

Origin of the plot is in the lower-left corner.  The  x  axis

runs  left  to right along the plot bottom.  The y axis is plotted

as a vertical displacement offset by the z axis value.  The z axis

appears  to  point  into  the  screen.  This, with the hidden line

removal, gives the illusion of depth.

VAX3DX  contains   an   internal   working   storage   arrays

dimensioned  sufficiently  large  for  most sufaces.  However, for

very complex surfaces, the working storage buffer  length  may  be

exceeded.   In  this  case  an  error  message  is  written to the

terminal and the routine terminated.

CALL VAX3DX(d,ndx,ndz,nx,nz,a,b,xh,yh,zh,iflag,iax,

<xt,nxt,xs,xe,nmx,nnx,mlx,tsx,ndx,smx,

yt,nyt,nmy,nny,mly,tsy,ndy,smy,

zt,nzt,zs,ze,nmz,nnz,mlz,tsz,ndz,smz,

<dm,dx<,ic>>>)

d        (R): array of y values dimensioned d(ndx,ndz)

ndx,ndz  (I): x and z dimensions of d array

nx,nz    (I): x and z sizes of surface to plot d array

a        (R): angle of x axis from horizontal 0-85 degrees

b        (R): angle of z axis from horizontal 0-90 degrees

note: origin (1,1) is in lower-left corner

x axis runs left to right on screen

y axis runs up to down on screen

z axis appears to run into the screen but

7-71

MASTER SUBROUTINES

is angled to the right

xh,yh,zh (R): length of each axis

iflag    (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) >10000 : do not initialize LONGLIB before plotting

(1's digit)  = 2 : use color array (need all parameters)

= 1 : do not use color array

(10's digit) = 0 : Plot sides

= 1 : Do not plot sides

(100's)      = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

iax   (I): axis format control

< 0 : plot axes, use input scale factors dm and dx

= 0 : no axes plotted, optional parameters (xt...dx)

not used, scaling computed from input array

> 0 : plot axes, use scaling computed from input array

only axis parameters xt through smz required.

(1's digit)  = 1 : Plot actual max/min or input values for Y axis

= 2 : Plot smoothed values for Y axis

(10's digit) = 0 : Use default axis type

= 1 : Use input AXIS2-type axis parameters

(nmx, nnx, mlx, tsx, ndx, etc.)

(NOTE: the following optional paramters are used if iax < 0

or mod(iflag,10)=1)

xt    (B): title of x axis (width)

nxt   (I): number of characters in xt

= 0 : no axis plotted

> 0 : normal

xs,xe (R): starting and ending values displayed on x axis

(see AXIS2 for detailed description of axis parameters)

nmx   (I): number of minor ticks between major ticks on x axis

nnx   (I): highlight length of nnx-th minor tick on x axis

mlx   (I): number of major tick marks on x axis

tsx   (R): size of title and numbers on x axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

ndx   (I): number of digits to right of decimal point on x axis

smx   (R): major tick length on x axis

yt    (B): title of y axis (depth)

nyt   (I): number of characters in yt

= 0 : no y axis plotted

> 0 : normal

nmy   (I): number of minor ticks between major ticks on y axis

nny   (I): highlight length of nny-th minor tick on y axis

mly   (I): number of major tick marks on y axis

tsy   (R): size of title and numbers on y axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

ndy   (I): number of digits to right of decimal point on y axis

smy   (R): major tick length on y axis

zt    (B): title of z axis (height)

7-72

MASTER SUBROUTINES

nzt   (I): number of characters in zt

= 0 : no z axis plotted

> 0 : normal

ze,ze (R): starting and ending valued displayed on z axis

nmz   (I): number of minor ticks between major ticks on z axis

nnz   (I): highlight length of nnz-th minor tick on z axis

mlz   (I): number of major tick marks on z axis

tsz   (R): size of title and numbers on z axis

< 0 auto exponent scaling (x10 to power) disabled

> 0 auto exponent scaling (x10 to power) enabled

ndz   (I): number of digits to right of decimal point on z axis

smz   (R): major tick length on z axis

(NOTE: the following optional parameters are required only if

iax < 0 or mod(iflag,10)=1)

dm,dx (R): minimum and maximum values of d array

(NOTE: color array required only if mod(iflag,10)=1)

ic    (I): color list

ic(1) : color for axis lines

ic(2) : color for axis numbers

ic(3) : color for axis titles

ic(4) : color for axis exponents

ic(5) : color index for lower plot surface (return)

ic(6) : color index for upper plot surface (return)

7-73

MASTER SUBROUTINES

7.34  SUBROUTINE VAX5D

VAX5D plots a 4 or 5-d surface by plotting slices through the

3rd  and  4th dimensions in a 2-d array of of 3-d plots.  Each 3-d

surface plots d(*) as a function of  2  of  the  dimensions  using

VAX3D.

Origin of the plot is in the lower-left corner.  The  X  axis

runs  left  to  right  along  the  subplot  bottom.  The Y axis is

plotted into the page of the subplot (see VAX3D).  The Z axis runs

left to right in subplots with the W axis vertical subplots.

^ W

|

|  d  Y

|  |/_ X

|

----------> Z

Since the subplots may runoff the edge of the plotting  page,

the  routine  includes  a  page size option to issue a NEWPAGE and

plot the additional  subplots  on  separate  pages.   A  shrinking

factor  is included to shrink the subplots.  Labeling of the W and

Z axis is due in the lower right hand  corner.   Each  subplot  is

further  tagged  with  the  corresponding  W  and Z axis value.  A

multiple page  plot  can  be  pasted  together  to  form  a  large

representation of a 5-d (or 4-d) surface.

CALL VAX5D(d,nd,n,a,b,iflag,iax,iform,w,xh,yh,zh,ph,pl,fac,iw,iz,

st,en,t1,nt1,t2,nt2,t3,nt3,t4,nt4,dt,ndt,<dm,dx<,ic>>)

d        (R): array to plot dimensioned d(nd(1),nd(2),nd(3),nd(4))

nd       (I): array of dimensions of d array

n        (I): array of the number of points from in dimension to plot

a,b      (R): angles a,b for VAX3D subplot

iflag    (I): option flag

< 0 : do not close LONGLIB after plotting

= 0 : close LONGLIB--no plot produced

> 0 : close LONGLIB after plotting

(magnitude) > 10000 : do not initialize LONGLIB before plotting

(1's digit)  = 2 : use color array (need all parameters)

= 1 : do not use color array

(10's digit) = 0 : Plot sides of subplots

= 1 : Do not plot sides

(100's)      = 0 : Ask which screen device to use

<> 0 : Screen Device Number (see FRAME)

iax   (I): axis format control

< 0 : plot axes, use input scale factors dm and dx

= 0 : no axes plotted, optional parameters (t1...dx)

not used, scaling computed from input array

> 0 : plot axes, axis parameters t1 through nt4 used,

scaling computed from input array

(1's digit)  = 1 : Plot actual max/min or input values for Y axis

7-74

MASTER SUBROUTINES

= 2 : Plot SCALE smoothed values for Y axis

(10's digit) = 0 : Axes plotted for all subplots

= 1 : Axes plotted only for first subplot

iform    (I): plot format code

selects which dimension of d array is to become

which output axis

plot axis                     plot axis

Code      X  Y  Z  W            Code    X  Y  Z  W

_____    ____________          _____  ____________

1 input: 1  2  3  4             13     3  1  2  4

2 dimen- 1  2  4  3             14     3  1  4  2

3 sion   1  4  2  3             15     3  2  1  4

4 number 1  4  3  2             16     3  2  4  1

4        1  4  3  2             16     3  2  4  1

5        1  3  2  4             17     3  4  1  2

6        1  3  4  2             18     3  4  2  1

7        2  1  3  4             19     4  1  2  3

8        2  1  4  3             20     4  1  3  2

9        2  4  3  1             21     4  2  1  3

10       2  4  1  3             22     4  2  3  1

11       2  3  4  1             23     4  3  1  2

12       2  3  1  4             24     4  3  2  1

w        (R): working array dimensioned at least n(x)*n(y)

xh,yh,zh (R): length of each axis of subplot

ph,pl    (R): page height, length when multiple pages required

fac      (R): shrink factor for subplots (2 == FACTOR(1/2))

iw,iz    (I): plot every iw'th and iz'th subplots

st,en    (R): arrays containing axes start and end values (permuted

along with d array dimensions)

(NOTE: titles/number of characters permuted with d array dimensions)

t1       (B): title corresponding to 1st dimension of d

nt1      (I): number of characters in title t1

= 0 : no axis plotted

> 0 : axis plotted

t2       (B): title corresponding to 2st dimension of d

nt2      (I): number of characters in title t2

= 0 : no axis plotted

> 0 : axis plotted

t3       (B): title corresponding to 3rd dimension of d

nt3      (I): number of characters in title t3

= 0 : no axis plotted

> 0 : axis plotted

t4       (B): title corresponding to 4th dimension of d

nt4      (I): number of characters in title t4

= 0 : no axis plotted

> 0 : axis plotted

t5       (B): title corresponding to 5th dimension of d

nt5      (I): number of characters in title t5

= 0 : no axis plotted

> 0 : axis plotted

(NOTE: the following optional parameters are required only if

7-75

MASTER SUBROUTINES

iax < 0 or mod(iflag,10) = 1)

dm,dx    (R): minimum and maximum scale values for array

ic       (I): color array

ic(1) : color for axis lines

ic(2) : color for axis numbers

ic(3) : color for axis titles

ic(4) : color for axis exponents

ic(5) : color index for lower plot surface (return)

ic(6) : color index for upper plot surface (return)

7-76

CHAPTER 8

MISCELLANEOUS ROUTINES

The subroutines listed below are miscellaneous functions  and

routines  used  by  the  previously described routines.  These are

low-level routines documented for  the  use  of  advanced  LONGLIB

programers.

8.1  INTEGER FUNCTION IPCLIP

IPCLIP tests a point to determine if it lies in  a  rectangle

defined  by  xm,ym,xx,yx  and  returns an integer value indicating

where point is in relation to rectangle.  The value can be  easily

be  decoded  by "anding" return value with the binary values of 1,

2, 4, 8.

9 | 8  | 10

--------------

1 | 0  | 2

--------------

5 | 4  | 6

iflag = IPCLIP(x,y,xm,ym,xx,yx)

x,y   (R): point to test

xm,ym (R): lower left corner of rectangle

xx,yx (R): upper right corner of rectangle

iflag (I): clip flag (0-10) (see above)

8.2  SUBROUTINE GCONTR

GCONTR draws contour lines of a 2-d array using  a  technique

which produces long, connected contour lines.  It assumes that the

points of the input array are equally spaced  in  each  dimension.

Several  options  for contouring are provided.  When cs, the label

character size is positive, the  origin  point  (1,1)  is  in  the

lower-left corner and the point (i,j) in the array is plotted at,

8-1

MISCELLANEOUS ROUTINES

xplot = (i-1)*xl

yplot = (j-1)*yl

If cs is negative, the x and y  values  plotted  (xp,yp)  for  the

point   (i,j)   in   the   array  are  computed  using  the  PLT3D

transformation common block PLT3B (see PLT3D),

xp = a1 * (yl * j) + a2 * (xl * i) + a3

yp = b1 * (yl * j) + b2 * (xl * i) + b4

where the vertical height (z) is zero.  In this case,  xl  and  yl

should  be  set  to  1.0 and PLT3D should be called before GCONTR.

GCONTR is used in the MASTER routine LCNTR and CVAX3DX.

CALL GCONTR(z,ndx,ndy,nx,ny,xl,yl,cv,nv,zm,iw,n,cs,m,i,ic,il)

z       (R): 2-d array of values dimensioned z(nx,ny)

ndx,ndy (I): dimensions of data array

nx,ny   (I): number of points to use in array

xl,yl   (R): axis length scale factors (inches/array index)

cv      (R): array of contour levels dimensioned cl(nv)

nv      (I): number of contour levels  (note: if nv < 0 then only

one contour level is used.  It will be labeled with

the abs(nv)'th symbol)

zm      (R): maximum value of z for consideration.  A z value which

exceeds this value will be ignored. The cell edges

which include this point will not be included in

contouring.

iw      (I): workspace dimensioned at least (2*nx*ny*nv+1)/31

n       (I): contour labeling option

< 0 label with contour value (number with n digits

to the right of the decimal point)

= 0 no labelling of contours

> 0 label with alphabet (nl should be less than 26)

cs      (R): size of labels

< 0 : plot contours using PLT3B transformation

xl and yl should then be set to 1.0 (see notes)

> 0 : normal location specification

m       (I): minimum number of cells crossed by contour in order

for contour to be labeled

i       (I): color and line type flag

= 0 color and line type arrays not used

= 1 color array used

= 2 line type array used

= 3 color and line type array used

ic      (I): color list for each contour (only requied for i>0)

l       (I): line type list for contours (required only for i>1)

8.3  INTEGER FUNCTION INXTCHR

INXTCHR returns a single key pressed on the terminal.  In the

VAX/VMS environment, it will intercept all control keys (including

8-2

MISCELLANEOUS ROUTINES

<ESC>) except Y,T,R,Q,Z,S without echoing to the screen.  Opens  a

direct  IO  channel  to  the  terminal  driver using SYSQIO.  This

routine is used by the PAUSE and CURMOTION routines.

key  = INXTCHR()

key (I): (returned) terminal character (ASCII value)

if key < 0, an error reading terminal input has occured.

8.4  INTEGER FUNCTION IRMCHAN

IRMCHAN returns the channel  number  and  the  Ramtek  device

number  used  by  the  plot  package  (as assigned by RAMOPEN) for

communicating with the Ramtek.  Returns a negative value when  the

Ramtek  package  is  not initialized or when the Ramtek channel is

not yet assigned.

ich = IRMCHAN(id)

id  (I): (returned) Ramtek device number

ich (I): (returned) Ramtek channel number

if ich <=0 the Ramtek is not is use.

8.5  SUBROUTINE MATMUL4

MATMUL4 multiplies two 4x4 matrixes A and B  and  sets  C=AB.

Does not change the contents of A and B.

CALL MATMUL4 (c,a,b)

c,a,b (R): 4 x 4 matrixes

8.6  SUBROUTINE MTV4

MTV4 multiplies a 4 element row vector by a  4x4  matrix  and

sets V2=A V1.

CALL MTV4 (v2,a,v1)

v2  (R): 4 element row vector

a   (R): 4 x 4 matrix

v1  (R): 4 element row vector

8-3

MISCELLANEOUS ROUTINES

8.7  SUBROUTINE NXTVU

NXTVU is  used  internally  by  PLT3D.   NXTVU  computes  the

maximum (or minimum) of two piecewise linear functions:  the curve

specified in the input d array and the curve stored in the working

array  w.   On return the new maximum (minimum) curve replaces the

old in w.  Any line segments or fractions  thereof  above  (below)

the  maximum  (minimum)  are  plotted.   If iabs(i)=1 the input is

copied into the  working  array  and  plotted.   Subsequent  calls

should use iabs(i)=2.  Using i positive computes the maximum while

using i negative plots the minimum.

A grid of lines in only one dimension may be made by  calling

NXTVU  once for each row, adjusting the d curve to offset each row

by a small amount to give the impression of a surface.

The dimension of the  working  arrays  is  dependent  of  the

surface  complexity  --  the  greater the complexity the larger n2

must be.  ier is used to indicate when n2 is not large enough.  As

a minimum n2>2*n.  Note:  w should not be modified between calls.

CALL NXTVU (i,d,n,w,n2,ier)

i     (I): initialize code

< 0 : plot lower side of surface

=> 0 : plot upper side of surface

= -1 : first call for lower surface plot

= -2 : subsequent calls for lower surface plot

=  1 : first call for upper surface plot

=  2 : subsequent calls for upper surface plot

d     (R): array of (x,y) coordinate pairs dimensioned d(2*n)

d(1) = x(1)

d(2) = y(1)

d(3) = x(2)

...

n     (I): number of coordinate pairs in d array

w     (R): working storage array of dimensioned d(n2)

(should not be modified between calls)

n2    (I): dimension of working array

ier   (I): (returned) error code

= 0 : no error

= 1 : out of space in w

8.8  SUBROUTINE PAUSE

PAUSE prompts terminal without a CTERM(1) for a keystroke  to

continue.  Uses INXTCHR.

CALL PAUSE

(no arguments)

8-4

MISCELLANEOUS ROUTINES

8.9  SUBROUTINE PAUSEP

PAUSEP  prompts  terminal  for  a  keystroke   to   continue.

Includes  appriate CTERM calls to prompt in text mode then returns

terminal to plot mode.  Uses INXTCHR.

CALL PAUSEP

(no arguments)

8.10  SUBROUTINE RAMCLOSE

RAMCLOSE  closes  and  deassigns  the  Ramtek   channel   and

deallocates  the  device.   When  the  REF  routines  are used, it

interatively prompts for the output  device  and  option.   REFDIS

called  prior  to  RAMCLOSE  disables  the  prompting  in  the REF

package.  Note that RAMCLOSE is call by PLOTRM when  a  PLOTND  or

PLOTRM(0.,0.,11) is called.

CALL RAMCLOSE (ic)

ic   (I): channel number (from RAMOPEN or IRMCHAN)

8.11  SUBROUTINE REFDIS

REFDIS is used only in the LONGLIBR version  of  the  longlib

graphics  library.  It is a dummy call for other versions.  REFDIS

is a non-interactive method of specifying the output  device  (REF

file,  terminal  screen,  or  LONGLIB  metafile)  to  be  used for

outputing the REF bit-map Ramtek image array.  When  called  prior

to  PLOTND,  it  outputs  the array to the device specified in the

call without user intervention.  When REFDIS is not  called  prior

to  PLOTND,  PLOTND will call REFDIS and the user will be prompted

for the output device and option.  Note:   REFDIS  may  be  called

multiple times to output to several devices.

When the internal REF data array is output to the terminal or

metafile output each line of the internal array is scanned left to

right.  Connected pixels having the same color are  collected  and

plotted  (using  PLOT)  to the output device.  Pixels with 0 value

are not output.  The pixel-to-inch  output  scaling  can  be  user

selected  to  correspond  to the actual hardware resolution of the

output device.  Normal  resolution  for  the  terminal  output  is

9.5/1024  inch/pixel  (most  terminals  do  not actually have this

resolution).  Normal resolution for the meta file output is  1/300

inch/pixel.   Hence, at normal resolution, the 1280x1024 pixel REF

array more than fills the terminal screen.   The  pixel  image  is

output  with  the  lower-left  corner  at  (0,0).  By changing the

origin prior to call the user can display any desired  portion  of

the image.

8-5

MISCELLANEOUS ROUTINES

CALL REFDIS(id,ot,n,rx,ry)

id    (I): Output device

=-1 : graphics terminal number code (see FRAME)

user-specified (rx,ry) used

= 1 : graphics terminal number code (see FRAME)

default resolution used

= 2 : Ramtek emulation file (REF)

(ot = 1 : absolute file write)

(ot = 2 : write out only non-zero pixels)

(ot = 3 : write out only zero pixels)

=-3 : LONGLIB metafile output

user-specified (rx,ry) used

= 3 : LONGLIB metafile output

default resolution used

ot    (I): Output option code (see above)

n     (C): REF file name

rx,ry (R): user-specified output resolution (inch/pixel)

8.12  SUBROUTINE RMCLEAR

RMCLEAR clears Ramtek screen.

CALL RMCLEAR (ic,ie)

ic   (I): channel number (from RAMOPEN or IRMCHAN)

ie   (I): (returned) error code

8.13  SUBROUTINE RMDIR

RMDIR sets the write direction for image array  data  on  the

Ramtek display.  This routine is supported in the REF package.

CALL RMDIR (ic,is,ie)

ic   (I): channel number (from RAMOPEN or IRMCHAN)

is   (I): scan sequence code

code pix-to-pix  line-to-line

0      L-R        T-B

1      R-L        T-B

2      L-R        B-T

3      R-L        B-T

4      T-B        L-R

5      B-T        L-R

6      T-B        R-L

7      B-T        R-L

ie   (I): (returned) error code

8-6

MISCELLANEOUS ROUTINES

8.14  SUBROUTINE RMFNTSIZE

RMFNTSIZE changes the size of Ramtek text  displayed  on  the

Ramtek display.  Not supported in REF package.

CALL RMFNTSIZE (ic,ih,iv,ihs,ivs,ie)

ic      (I): channel number (from RAMOPEN or IRMCHAN)

ih,iv   (I): horizontal,vertical dimension

ihs,ivs (I): horizontal,vertical spacing

ie      (I): (returned) error code

8.15  SUBROUTINE RAMOPEN

RAMOPEN (1) translates the local name "RM" to  determine  the

ramtek device number, (2) allocates the ramtek device, (3) assigns

a channel to the Ramtek device, and (4)  opens  the  channel  I/O.

Returns the channel number or -1 if device is not available.  This

is routine is called by RPLOTS which is called by FRAME.   RAMOPEN

initializes the REF array.

CALL RAMOPEN(ic,it,id,ie)

ic   (I): returned channel number

it   (I): Ramtek device code input

= 1 1280x1024 Ramtek

= 2 512x512 Ramtek

id   (I): returned Ramtek device number

ie   (I): (returned) error code

8.16  SUBROUTINE RAMOUT

RAMOUT outputs  a  command  and  data  array  to  the  Ramtek

display.   (see  Ramtek  manual  for  command  formats).   The REF

package uses RAMOUT for a different purpose.

CALL RAMOUT (ic,m,n,ie)

ic   (I): channel number (from RAMOPEN or IRMCHAN)

m    (B): array of bytes to output

n    (I): number of bytes

ie   (I): (returned) error code

8.17  SUBROUTINE RMPAN

RMPAN pans Ramtek display.  Not supported in REF package.

8-7

MISCELLANEOUS ROUTINES

CALL RMPAN (ic,il,ir,ie)

ic    (I): channel number (from RAMOPEN or IRMCHAN)

il    (I): left x pixels

ir    (I): right y pixels

ie    (I): (returned) error code

8.18  SUBROUTINE RMPLOT

RMPLOT plots  an  array  of  connected  vectors  using  pixel

locations  on  Ramtek  or  REF package.  This routine is called by

PLOTRM which is called by  PLOT.   RMPLOT  simulates  line  widths

using  the width information stored in an internal common block by

RMTEXTURE by replicating the line several times with pixel offsets

to  produce  a  "thick" line.  Note:  x is in pixels from right to

left.  y is in pixels from top of display.

CALL RMPLOT (ic,n,ia,k,ie)

ic   (I): channel number (from RAMOPEN or IRMCHAN)

n    (I): number of point pairs (<129)

ia   (I): array of point pairs (in pixels)

a(1)=x1,a(2)=y1,a(3)=x2,...

k    (I): color table index to use

ie   (I): (returned) error code

8.19  SUBROUTINE RMREADBYTE

RMREADBYTE reads the Ramtek image array.  Supported by REF.

CALL RMREADBYTE (ic,a,n,ie)

ic   (I): channel number (from RAMOPEN)

a    (B): (returned) image data

n    (I): number of words of a to read

ie   (I): (returned) error code

8.20  SUBROUTINE RMREADCOL

RMREADCOL reads  the  Ramtek  color  table  from  the  Ramtek

display.   The  color  table is INTEGER*4 words.  Not supported by

REF.

CALL RMREADCOL (ic,ia,n,ie)

ic   (I): channel number (from RAMOPEN)

ia   (I): (returned) color table array

8-8

MISCELLANEOUS ROUTINES

n    (I): number of words of a to read

ie   (I): (returned) error code

8.21  SUBROUTINE RMREADCURSOR

RMREADCURSOR reads the current Ramtek cursor device position.

Called  by CURMOTION, etc.  See RMSETCURSOR.  Not supported on REF

package.

CALL RMREADCURSOR (ic,id,ix,iy,it,iv,ien,ie)

ic        (I): channel number (from RAMOPEN)

id        (I): cursor device number

ix,iy     (I): (returned) pixel location of cursor (pixels)

it,iv,ien (I): (returned) codes for track, visible, enter

switches, see RMSETCURSOR

ie        (I): (returned) error code

8.22  SUBROUTINE RMREADWORD

RMREADWORD reads INTEGER*2 words from the Ramtek image array.

Supported by REF.

CALL RMREADWORD (ic,id,n,ie)

ic   (I)  : channel number (from RAMOPEN)

id   (I*2): (returned) image data

n    (I)  : number of words to read

ie   (I)  : (returned) error code

8.23  SUBROUTINE RMSETCUR

RMSETCUR moves specified ramtek cursor device to a  specified

position  and  sets  it  as  visible  and/or  blinking.  Called by

CURLOCATE.  Not supported by REF

CALL RMSETCUR (ic,i,ix,iy,ib,iv,ie)

ic    (I): channel number (from RAMOPEN)

i     (I): cursor device number (0-3)

ix,iy (I): pixel position of cursor (see RMPLOT)

ib    (I): blink flag  (1=no blink,2=blink)

iv    (I): visible flag (2=visible, 0=invisible)

ie    (I): returned error code

8-9

MISCELLANEOUS ROUTINES

8.24  SUBROUTINE RMSTART

RMSTART sets the start pixel  of  image  mode  write  on  the

Ramtek display.  Supported by REF.

CALL RMSTART (ic,ix,iy,ie)

ic    (I): channel number (from RAMOPEN)

ix,iy (I): pixel position to start next image write

ie    (I): (returned) error code

8.25  SUBROUTINE RMTEXT

RMTEXT places text on Ramtek display  using  Ramtek  hardware

text support.  Not supported on REF.

CALL RMTEXT (ic,icol,ix,iy,is,t,nt,ie)

ic   (I): channel number (from RAMOPEN)

icol (I): color

ix   (I): x pixel location

iy   (I): y pixel location

is   (I): size in pixels

t    (B): byte array of text

nt   (I): number of bytes in the array t

ie   (I): (returned) error code

8.26  SUBROUTINE RMTEXTURE

RMTEXTURE changes the bit texturing pattern for  vector  line

drawing  on the Ramtek.  Called by RMPEN which is called by NEWPEN

and also by CURMOTION et.  al.  Supported by REF.

CALL RMTEXTURE (ic,it,iw,is,ie)

ic    (I): opened channel number (from RAMOPEN)

it    (I): line type number (0-15)

iw    (I): line width used in RMPLOT (1-7)

is    (I): bit width scale factor (0-15)

(0= [1 bit=1 pixel], 1=[1 bit=2 pixels], etc.)

ie    (I): (returned) error code

8.27  SUBROUTINE RMWIND

RMWIND sets the image area of the Ramtek display.   Supported

by REF.

8-10

MISCELLANEOUS ROUTINES

CALL RMWIND (ic,ix,iy,mx,my,ie)

ic    (I): channel number (from RAMOPEN)

ix,iy (I): starting corner of image pixels (u-r corner)

mx,my (I): ending corner of image pixels (l-l corner)

ie    (I): (returned) error code

8.28  SUBROUTINE RMWRITEBYTE

RMWRITEBYTE write byte image data to the Ramtek image  array.

Supported by REF.

CALL RMWRITEBYTE (ic,a,n,ie)

ic   (I): channel number (from RAMOPEN)

a    (B): image data

n    (I): number of words of a to read

ie   (I): (returned) error code

8.29  SUBROUTINE RMWRITECOL

RMWRITECOL writes Ramtek color display  data  to  the  Ramtek

display  color  table.   The  color table is INTEGER*4 words.  Not

supported by REF.

CALL RMWRITECOL (ic,a,n,ie)

ic   (I): channel number (from RAMOPEN)

a    (I): new color table array

n    (I): number of words of array a

ie   (I): (returned) error code

8.30  SUBROUTINE RMWRITEWORD

RMWRITEWORD writes INTEGER*2 image data to the  Ramtek  image

display.  Supported by REF.

CALL RMWRITEWORD (ic,id,n,ie)

ic   (I)  : channel number (from RAMOPEN)

id   (I*2): image data

n    (I)  : number of words to read

ie   (I)  : (returned) error code

8-11

MISCELLANEOUS ROUTINES

8.31  SUBROUTINE RMZOOM

RMZOOM zooms Ramtek display.  Not supported by REF.

CALL RMZOOM (ic,iz,ie)

ic    (I): channel number (from RAMOPEN)

iz    (I): zoom factor in powers of 2

ie    (I): (returned) error code

8.32  SUBROUTINE VTPLOT

VTPLOT plots an  array  of  connected  vectors  to  terminal.

Terminal must be in graphics mode prior to call.  VTPLOT is called

by PLOTVT and by CURLOCATE and CURMOTION., et.  al.  An erase flag

is  used  to  indicate  whether  vector  string should be visible,

erased, or XOR'ed.  Since not all terminal support XOR, two  flags

for  XOR  are  provided--one  which if XOR is not supported writes

visible vectors with the other which erases (if supported).   Line

width is simulated by replotting adjacent lines.

CALL VTPLOT (n,m,ie,iw)

n   (I): number of point pairs in m

m   (I): array of points to be connected with line

m(1)=x1,m(2)=y1,m(3)=x2,...

ie  (I): erase flag (0=normal, 1=XOR (on), 2=erase, 3=XOR (off))

iw  (I): width

8.33  SUBROUTINE TRIANGC

TRIANGC triangulates a set of  (x,y)  points  such  that  the

boundry  is  a  convex  polygon.   This  routine is an adaption of

COSMIC routine ARC-11441.  Used by some MASTER routines.

CALL TRIANGC(x,y,n,nt,nzz,m,i,j,ni,l,nz,ie,ibe,ite)

x,y   (R): arrays of x,y points

n     (I): number of points

< 0 : ie,ibe,ite arrays not used

> 0 : ie,ibe,ite arrays used (normal)

nt    (I): array of indicies of triangulated points t(nzz,3)

corner 1 of triangle K = (x,y) = (x(t(K,1),y(t(K,1)))

corner 2 of triangle K = (x,y) = (x(t(K,2),y(t(K,2)))

corner 3 of triangle K = (x,y) = (x(t(K,3),y(t(K,3)))

nzz   (I): dimension of t array (>3*n)

m     (I): number of triangles stored in t

i,j   (I): working arrays (dimensioned i(ni),j(ni))

ni    (I): dimension of i,j arrays (ni>=n)

8-12

MISCELLANEOUS ROUTINES

l     (I): number of edges in ie,ibe,ite

nz    (I): dimension of ie,ibe,ite array (>3*n)

note: these arrays only needed if n>0

ie    (I): array of indicies of each triangle edge ie(nz,2)

ibe   (I): edge flag array dimensioned ibe(nz)

= 0 for interior edge

= 1 if ie is a boundry edge

ite   (I): array of indicies of the neighbor edges of

each triangle, dimensioned ite(nz,4)

8.34  REAL FUNCTION XVMUL3D

XVMUL3D returns  one  element  of  an  input  vector  (x,y,z)

multiplied by an input rotation matrix r(4,4).

value = XVMUL3D (n,x,y,z,v,r)

n     (I) : which coordinate value to return (1=x,2=y,3=z)

x,y,z (R) : input x,y,z

v     (R) : working vector (4 elements)

r     (R) : rotation matrix (4,4)

value (R) : desired element value (see n)

8-13

CHAPTER 9

LONGLIB LIBRARY NAMES

This chapter lists the LONGLIB  graphics  library  subroutine

names  as well as the subroutines (outside of the standard FORTRAN

routines)  called  by  each  subroutine.   The  calling   routines

(excluding  MASTER  routines)  are indicated as well.  An asterick

indicates that the routine is documented in the documentation  and

may called by the user.

9.1  SUBROUTINE CALLS

1.  ABSPLT

Calls:  WHEREVT, FIXVT0, WHEREPR, FIXPR0, WHERERM, FIXRM0

Called by:  *

2.  ANXTVU

Calls:  (none)

Called by:  NXTVU

3.  ARCALC

Calls:  (none)

Called by:  LINSEQ

4.  ARCPLT

Calls:  SPIFUN, PLOT, SPIDER

Called by:  LINSEQ

5.  ARCSET (Entry of ARCPLT)

Calls:  PLOT

Called by:  LINSEQ

6.  ARROW

Calls:  PLOT

Called by:  *

9-1

LONGLIB LIBRARY NAMES

7.  ASTEXIT

Calls:

Called by:  (qio -- system routine)

8.  ASTINTER

Calls:  (none)

Called by:  *, PLOT

9.  AXIS

Calls:  PLOT, SYMBOL, NUMBER

Called by:  *

10.  AXIS2

Calls:  PLOT, SYMBOL, NUMBER

Called by:  *

11.  AXIS3

Calls:  PLOT, SYMBOL, NUMBER

Called by:  *

12.  AXIS3D

Calls:  ROTEM, MATMUL4, PLOT3D, NUM3D, SYM3D, XVMUL3D

Called by:  *

13.  AXIS3DH

Calls:  ROTEM, MATMUL4, PLT3D,  SKETCH,  NUM3DH,  SYM3DH,

XVMUL3D

Called by:  *

14.  BARCHR

Calls:   FRAME,  CTERM,  PLOT,  SYMBOL,  NUMBER,  PLOTND,

NEWPEN, SHADE

Called by:  *

15.  BITCURSOR

Calls:  (none)

Called by:  *

16.  BITMAP

Calls:  PLOT, LNDSEA

Called by:  *

17.  CHECK3D

Calls:  (none)

Called by:  LINHID

18.  CHLSKYS

Calls:  (none)

Called by:  SMOOTHC

19.  CIRCLE

Calls:  PLOT

Called by:  *, PLRAX

9-2

LONGLIB LIBRARY NAMES

20.  CLIP3D

Calls:  (none)

Called by:  IPCLP3

21.  CLPIT

Calls:  IPCLIP

Called by:  PLOTVT, PLOTRM, PPLOT

22.  IPCLIP

Calls:  (none)

Called by:  CLPIT, PLOTVT, PLOTRM, PPLOT

23.  CNCELPLT

Calls:  SEGCODE, POLY1INT, PLOT

Called by:  CNTRPLT

24.  CNCELPLT3D

Calls:  SEGCODE, RVXPT3D, PLOT, POLY1INT

Called by:  CNT3DX

25.  CNDRAW

Calls:  PLOT, NEWPEN, SYMBOL, NUMBER

Called by:  GCONTR

26.  CNT3D

Calls:  CNT3DX

Called by:  *

27.  CNT3DX

Calls:   SCALE,  FRAME,  CTERM,  AXIS2,   AXIS,   VXPT3D,

CNCELPLT3D, TRCELPLT3D, PLOTND, NEWPEN

Called by:  *, CNT3D

28.  CNTLN

Calls:   FRAME,  CTERM,  SCALE,  AXIS3,   PLOT,   SYMBOL,

CNCELPLT, TRIANGC, INTERPC, CNTOUR, PLOTND, NEWPEN

Called by:  *

29.  CNTOUR

Calls:  PLOT

Called by:  CNTLN

30.  CNTRPLT

Calls:  FRAME, CTERM, NEWPAGE, SCALG, SCALE, LGRID, GRID,

LGAXS, AXIS, CNCELPLT, PLOTND, NEWPEN

Called by:  *

31.  CSHADE

Calls:  PLOT, NEWPEN, PICHRT

Called by:  *

9-3

LONGLIB LIBRARY NAMES

32.  CTERM

Calls:  (none)

Called by:  *, PLOTVT

33.  CUBE

Calls:  PLT3DH, SKETCH

Called by:  *, HIST3D

34.  CURBAND

Calls:  RMTEXTURE, RMSETCUR, RMPLOT, INXTCHR, VTPLOT

Called by:  *

35.  CURLOCATE

Calls:  RMSETCUR, VTPLOT

Called by:  *

36.  CURMOTION

Calls:  INXTCHR, RMSETCUR, VTPLOT

Called by:  *

37.  CURRECT

Calls:  RMTEXTURE, RMSETCUR, RMPLOT, INXTCHR, VTPLOT

Called by:  *

38.  CVAX3D

Calls:  CVAX3DX

Called by:  *

39.  CVAX3DX

Calls:  NXTVU, PLOT, FRAME, GCONTR, PLT3D, HLT3D,  CTERM,

AXIS3

Called by:  *, CVAX3D

40.  DASHL

Calls:  PLOT, SYMBOL

Called by:  *

41.  DRAW3D

Calls:  PLOT

Called by:  PLOT3D

42.  EARTH3D

Calls:  SPRECT1, PLOT3D

Called by:  *

43.  ELLIPSE

Calls:  PLOT

Called by:  *

44.  ENABLEAST

Calls:  ENAST

Called by:  *, FRAME

9-4

LONGLIB LIBRARY NAMES

45.  ENAST

Calls:  (qio system routine if control-c  interrupt  code

is used in package)

Called by:  ENABLEAST

46.  FACTOR

Calls:  VFACTOR, RFACTOR, PFACTOR

Called by:  *

47.  FIXPR0

Calls:  PPLOTP

Called by:  *, ABSPLT

48.  FIXRM0

Calls:  RMTEXTURE

Called by:  *, ABSPLT

49.  FIXVT0

Calls:  NEWVPEN

Called by:  *, ABSPLT

50.  FRAME

Calls:  VPLOTS, RPLOTS, PPLOTS, ENABLEAST,  EXIT  (system

routine)

Called by:  *, PLOTS

51.  GCONTR

Calls:  CNDRAW

Called by:  *, LCNTR

52.  GETCURSOR

Calls:  (none)

Called by:  *

53.  GLPLOT

Calls:  FRAME, CTERM, NEWPAGE, SCALG, SCALE, LGRID, GRID,

LGAXS, AXIS3, LINE, LGLIN, PLOTND

Called by:  *

54.  GRID

Calls:  PLOT

Called by:  *

55.  HELPME

Calls:  LIB$DISPLAY_OUTPUT, LIB$STATUS

Called by:  *

56.  HIST3D

Calls:  SCALE, FRAME,  CTERM,  AXIS3DH,  PLT3DH,  SKETCH,

PLOT, PLOTND, INIT3DH

Called by:  *

9-5

LONGLIB LIBRARY NAMES

57.  HLT3D

Calls:  NXTVU, PLOT

Called by:  *, CVAX3DX, MVAX3DX

58.  HPLT

Calls:  PLOT

Called by:  LINHID

59.  INIT3D

Calls:  MATMUL4, ROTEM

Called by:  *

60.  INIT3DH

Calls:  (none)

Called by:  *

61.  INTERPC

Calls:  (none)

Called by:  CNTLN

62.  INTERSECT

Calls:  (none)

Called by:  VAX3DX

63.  INXTCHR

Calls:  SYS$QIO

Called by:  *, PAUSE, PAUSEP, CURMOTION, CURRECT, CURBAND

64.  IPCLP3

Calls:  CLIP3D

Called by:  PLOT3D

65.  IRMCHAN

Calls:  (none)

Called by:  *

66.  ISEGCODE

Calls:  (none)

Called by:  TRCELPLT3D

67.  ISOL3D

Calls:  (none)

Called by:  LINHID

68.  JPLTAG

Calls:  SHADE, PLOT

Called by:  *

69.  LANDMAP

Calls:  PLOT

Called by:  *

9-6

LONGLIB LIBRARY NAMES

70.  LCNTR

Calls:  FRAME, CTERM, NEWPAGE, SCALG, SCALE, LGRID, GRID,

LGAXS, AXIS, GCONTR, PLOTND, NEWPEN

Called by:  *

71.  LCOEF

Calls:  (none)

Called by:  LINHID

72.  LGAXS

Calls:  PLOT, SYMBOL, NUMBER

Called by:  *

73.  LGLIN

Calls:  PLOT, SYMBOL

Called by:  *

74.  LGRID

Calls:  PLOT

Called by:  *

75.  LINE

Calls:  PLOT, SYMBOL

Called by:  *

76.  LINHID

Calls:   LCOEF,  VSRT1,  VSRTR,  HPLT,  CHECK3D,  ISOL3D,

STAT3D

Called by:  LINHID

77.  LINSEQ

Calls:  PRETRP, ARCALC, SMOOTHC, SPISET, ARCSET, ARCPLT

Called by:  *, SYMSS

78.  LNDSEA

Calls:  (none)

Called by:  BITMAP

79.  LSPLOT

Calls:  FRAME, CTERM, PLOTND, SYMBOL, SCALE, NEWPEN, PLOT

Called by:  *

80.  MATMUL4

Calls:  (none)

Called by:  PLOT3D, SYM3D, SYM3DH

81.  MESH3D

Calls:  MESH3DX

Called by:  *

82.  MESH3DX

Calls:  SCALE, FRAME, CTERM, VXPT3D, AXIS2,  AXIS,  PLOT,

PLOTND

9-7

LONGLIB LIBRARY NAMES

Called by:  *, MESH3D

83.  MIDC

Calls:  (none)

Called by:  TRIANGC

84.  MTV4

Calls:  (none)

Called by:  PLOT3D, SYM3D, SYM3DH, XVMUL3D

85.  MVAX3D

Calls:  MVAX3DX

Called by:  *, MVAX5D

86.  MVAX3DX

Calls:  SCALE, FRAME, CTERM, PLT3D, HLT3D,  AXIS3,  PLOT,

PLOTND

Called by:  *, MVAX3D

87.  MVAX5D

Calls:   SCALE,  FRAME,  CTERM,  RTERM,  SYMBOL,  NUMBER,

MVAX3D, PLOT, FACTOR, PLOTND

Called by:  *

88.  NEWPAGE

Calls:  PPLOT

Called by:  *

89.  NEWPEN

Calls:  RMPEN, PPEN

Called by:  *, CNDRAW, SHADE, CSHADE, GCONTR

90.  NEWVCOL

Calls:  (none)

Called by:  VPLOTS, PLOTVT

91.  NEWVPEN

Calls:  (none)

Called by:  VPEN

92.  NUM3D

Calls:  SYM3D

Called by:  *, AXIS3D

93.  NUM3DH

Calls:  SYM3DH

Called by:  *, AXIS3DH

94.  NUMBER

Calls:  SYMBOL

Called by:  *, AXIS, AXIS2, AXIS3, CNDRAW

9-8

LONGLIB LIBRARY NAMES

95.  NXTVU

Calls:  PLOT, NXT0VU, ANXTVU

Called by:  *, PLT3D, HLT3D, MVAX3DX, CVAX3DX

96.  OLDNUMB

Calls:  SYMBOL

Called by:  *

97.  NXT0VU

Calls:  (none)

Called by:  NXTVU

98.  PAUSE

Calls:  INXTCHR

Called by:  *

99.  PAUSEP

Calls:  INXTCHR, CTERM

Called by:  *

100.  PFACTOR

Calls:  (none)

Called by:  *, FACTOR

101.  PHIST

Calls:  FRAME, AXIS, PLOT, SYMBOL, SHADE,  RECT,  PLOTND,

CTERM

Called by:  *

102.  PICHRT

Calls:   FRAME,  SYMBOL,  PLOT,  NEWPEN,  CSHADE,  SHADE,

PLOTND, CTERM

Called by:  *

103.  PLOT

Calls:  PLOTVT, PLOTRM, PPLOT, ASTINTER

Called by:  *, AXIS, AXIS2, AXIS3, ARROW, BITMAP, CIRCLE,

CNDRAW,   DASHL,  DARW3D,  ELLIPSE,  FRACT,  GRID,  HPLT,

JPLTAG,  LANDMAP,  LGAXS,  LGLIN,  LGRID,  LINE,   NXTVU,

PLOTND,  PLRAX,  PLRLN,  PLT3DH,  POLARMAP,  RECT, SHADE,

SYMBOL, SYMS, SYMSS, TRCELPLT3D, CSHADE

104.  PLOT3D

Calls:  MTV4, ROTEM, MATMUL4, VCPY, DRAW3D, IPCLP3

Called by:  *, AXIS3D, EARTH3D, SYM3D

105.  PLOTLG

Calls:  FRAME, CTERM, NEWPAGE, SCALE, SCALG, LGRID, GRID,

PLOT, LGAXS, AXIS, LINE, LGLIN, PLOTND, SYMBOL, NEWPEN

Called by:  *

9-9

LONGLIB LIBRARY NAMES

106.  PLOTLG2

Calls:  FRAME, CTERM, NEWPAGE, SCALE, SCALG, LGRID, GRID,

PLOT, LGAXS, AXIS, LINE, LGLIN, PLOTND, SYMBOL, NEWPEN

Called by:  *

107.  PLOTLGL

Calls:  FRAME, CTERM, NEWPAGE, SCALG, SCALE, LGRID, GRID,

LGAXS, AXIS2, LINSEQ, PLOTND, SYMBOL, NEWPEN

Called by:  *

108.  PLOTLGX

Calls:  FRAME, CTERM, NEWPAGE, SCALG, SCALE, LGRID, GRID,

LGAXS, AXIS2, LGLIN, LINE, PLOTND, SYMBOL, NEWPEN

Called by:  *

109.  PLOTND

Calls:  PLOT

Called by:  *

110.  PLOTRM

Calls:  RMCLEAR, RMPLOT, IPCLIP, RAMCLOSE, CLPIT

Called by:  *, PLOT

111.  PLOTS

Calls:  FRAME

Called by:  *

112.  PLOTSC

Calls:  FRAME, CTERM, NEWPAGE, SCALE, GRID,  LINE,  AXIS,

PLOT, LINE, SYMBOL, PLOTND, NEWPEN

Called by:  *

113.  PLOTSC2

Calls:  FRAME, CTERM, NEWPAGE, SCALE, GRID,  LINE,  AXIS,

PLOT, LINE, SYMBOL, PLOTND, NEWPEN

Called by:  *

114.  PLOTVT

Calls:  CTERM, VTPLOT, IPCLIP, NEWVCOL, CLPIT

Called by:  *, PLOT

115.  PLRAX

Calls:  PLOT, SYMBOL, CIRCLE

Called by:  *

116.  PLRLN

Calls:  PLOT, SYMBOL

Called by:  *

117.  PLT3D

Calls:  NXTVU

Called by:  *, MVAX3DX, CVAX3DX

9-10

LONGLIB LIBRARY NAMES

118.  PLT3DH

Calls:  PLOT

Called by:  *,  SYM3DH,  AXIS3DH,  CUBE,  T3DH,  TRIG3DH,

CNTOUR, HIST3D

119.  POLARMAP

Calls:  PLOT

Called by:  *

120.  POLY1INT

Calls:  (none)

Called by:  CENCELPLT, CNCELPLT3D

121.  PPEN

Calls:  PPLOTP

Called by:  *, NEWPEN

122.  PPLOT

Calls:  PPLOTP, IPCLIP, CLPIT

Called by:  *, PLOT

123.  PPLOTP

Calls:  (none)

Called by:  NEWMSK, PPEN, PPLOT, PPLOTS

124.  PPLOTS

Calls:  PPLOTP

Called by:  FRAME

125.  PRESPL

Calls:  PPLOTP

Called by:  *, RESPL

126.  PRETRP

Calls:  SPISET, SPIFUN

Called by:  LINSEQ

127.  PSAVPL

Calls:  (none)

Called by:  *, SAVPL

128.  PSUBPRO

Calls:  SUBPROC

Called by:  *

129.  PXPCGT

Calls:  (none)

Called by:  SYMBOL, SYM3D, SYM3DH

130.  RECT

Calls:  PLOT

Called by:  *

9-11

LONGLIB LIBRARY NAMES

131.  RESPL

Calls:  PRESPL, RRESPL, VRESPL, WHERE, PLOT

Called by:  *

132.  RFACTOR

Calls:  (none)

Called by:  *, FACTOR

133.  RAMOPEN

Calls:  SYS$TRANSLOG, SYS$ALLOC, SYS$DALLOC,  SYS$DASSGN,

SYS$ASSIGN, SYS$QIOW

Called by:  RPLOTS

134.  RAMCLOSE

Calls:  SYS$DALLOC, SYS$DASSGN

Called by:  PLOTRM

135.  RAMOUT

Calls:  SYS$QIOW

Called by:  RMSTART, RMWIND, RMPLOT, RMCLEAR,  RMTEXTURE,

RMZOOM, RMPAN, RMSETCUR, RMTEXT, RMFNTSIZE

136.  RAMOUTIN

Calls:  SYS$QIOW

Called   by:    RMREADCURSOR,   RMREADCOL,    RMREADBYTE,

RMREADWORD

137.  REFDIS (entry of RAMCLOSE)

Calls:  (none)

Called by:  *

138.  RMCLEAR

Calls:  RAMOUT

Called by:  PLOTRM

139.  RMDIR

Calls:  RAMOUT

Called by:  *

140.  RMFNTSIZE

Calls:  RAMOUT

Called by:  *

141.  RMPAN

Calls:  RAMOUT

Called by:  *

142.  RMPLOT

Calls:  RAMOUT

Called by:  PLOTRM

9-12

LONGLIB LIBRARY NAMES

143.  RAMOUTIN

Calls:  SYS$QIOW

Called by:  RMREADBYTE, RMREADCOL, RMREADWORD

144.  RMREADBYTE

Calls:  RAMOUTIN

Called by:  *

145.  RMREADCOL

Calls:  RAMOUTIN

Called by:  *

146.  RMREADCURSOR

Calls:  RAMOUTIN

Called by:  *

147.  RMREADWORD

Calls:  RAMOUTIN

Called by:  *

148.  RMSETCUR

Calls:  RAMOUT

Called by:  CURBAND, CURRECT, CURMOTION, CURLOCATE

149.  RMRESET

Calls:  RAMOUT

Called by:  *

150.  RMSTART

Calls:  RAMOUT

Called by:  *

151.  RMTEXT

Calls:  RAMOUT

Called by:  *

152.  RMTEXTURE

Calls:  RAMOUT

Called by:  RMPEN, RPLOTS, FIXRM0, RRESPL

153.  RMWIND

Calls:  RAMOUT

Called by:  *

154.  RMWRITEBYTE

Calls:  RAMOUT

Called by:  *

155.  RMWRITECOL

Calls:  RAMOUT

Called by:  *

9-13

LONGLIB LIBRARY NAMES

156.  RMWRITEWORD

Calls:  RAMOUT

Called by:  *

157.  RMZOOM

Calls:  RAMOUT

Called by:  *

158.  RMPEN

Calls:  RMTEXTURE

Called by:  NEWPEN

159.  ROTEM

Calls:  (none)

Called by:  AXIS3D, INIT3D, PLOT3D, SYM3D, SYM3DH

160.  RPLOTS

Calls:  PLOTRM, RMTEXTURE

Called by:  FRAME

161.  RRESPL

Calls:  RMTEXTURE

Called by:  *, RESPL

162.  RSAVPL

Calls:  (none)

Called by:  *, SAVPL

163.  RTERM

Calls:  RMCLEAR, RAMCLOSE, RAMOPEN

Called by:  *

164.  RVXPT3D

Calls:  (none)

Called by:  CNCELPLT3D

165.  SAVPL

Calls:  PSAVPL, RSAVPL, VSAVPL, WHERE, PLOT

Called by:  *

166.  SCALE

Calls:  (none)

Called by:  *

167.  SCALG

Calls:  (none)

Called by:  *

168.  SCATPL

Calls:  FRAME, CTERM, NEWPAGE, SCALG, SCALE, LGRID, GRID,

LGAXS, AXIS, PLOT, SYMBOL, PLOTND

Called by:  *

9-14

LONGLIB LIBRARY NAMES

169.  SEGCODE

Calls:  (none)

Called by:  CNCELPLT, CNCELPLT3D

170.  SEISPL

Calls:  FRAME, CTERM, NEWPAGE, SCALG, SCALE, LGRID, GRID,

LGAXS, AXIS, PLOT, NEWPEN, SYMBOL, PLOTND

Called by:  *

171.  SFPLOT

Calls:  FRAME, CTERM, NEWPAGE, SCALG, SCALE, LGRID, GRID,

LGAXS, AXIS, PLOT, NEWPEN, SYMBOL, PLOTND

Called by:  *

172.  SHADE

Calls:  PLOT, NEWPEN

Called by:  *, JPLTAG

173.  SKETCH

Calls:  LINHID

Called by:  *

174.  SMOOTHC

Calls:  CHLSKYS

Called by:  LINSEQ

175.  SPIDER

Calls:  (none)

Called by:  ARCPLT

176.  SPIFUN

Calls:  (none)

Called by:  ARCPLT, PRETRP

177.  SPISET

Calls:  (none)

Called by:  LINSEQ, PRETRP

178.  SPLOTS

Calls:  SPLOTSX

Called by:  *

179.  SPLOTSX

Calls:  FRAME, CTERM, NEWPAGE, SCALG, SCALE, LGRID, GRID,

LGAXS, AXIS2, PLOT, SYMBOL, PLOTND

Called by:  *, SPLOTS

180.  SPRECT1

Calls:  (none)

Called by:  EARTH3D

9-15

LONGLIB LIBRARY NAMES

181.  STAT3D

Calls:  (none)

Called by:  LINHID

182.  STAT3D2

Calls:  (none)

Called by:  LINHID

183.  SUBPROC

Calls:  LIB$SPAWN, LIB$SIGNAL

Called by:  PSUBPRO

184.  SYM3D

Calls:  PLOT3D, ROTEM, MATMUL4, MTV4, PXPCGT

Called by:  *, AXIS3D, NUM3D

185.  SYM3DH

Calls:  MTV4, PLT3DH, PXPCGT, ROTEM, MATMUL4

Called by:  *, NUM3DH, AXIS3DH

186.  SYMBOL

Calls:  PLOT, PXPCGT

Called by:  *, AXIS, AXIS2, AXIS3, CNDRAW, DASHL,  LGAXS,

LGLIN, LINE, NUMBER, OLDNUM, PLRAX

187.  SYMS

Calls:  PLOT

Called by:  *

188.  SYMSS

Calls:  PLOT, LINSEQ

Called by:  *

189.  TR3DH

Calls:  SCALE, FRAME,  CTERM,  AXIS3DH,  PLT3DH,  SKETCH,

PLOT, PLOTND, INIT3DH

Called by:  *

190.  TRCELPLT3D

Calls:  RVXPT3D, PLOT, ISEGCODE

Called by:  CNT3DX

191.  TRIANGC

Calls:  MIDC

Called by:  *, CNTLN, TRIG3DH

192.  TRIG3DH

Calls:  SCALE, FRAME,  CTERM,  AXIS3DH,  PLT3DH,  SKETCH,

PLOT, PLOTND, INIT3DH

Called by:  *

9-16

LONGLIB LIBRARY NAMES

193.  VAX3D

Calls:  VAX3DX

Called by:  *, VAX5D

194.  VAX3DX

Calls:  SCALE, FRAME, CTERM, VXPT3D, AXIS2,  AXIS,  PLOT,

INTERSECT, PLOTND

Called by:  *, VAX3D

195.  VAX5D

Calls:   SCALE,  FRAME,  CTERM,  RTERM,  SYMBOL,  NUMBER,

VAX3D, PLOT, FACTOR, PLOTND

Called by:  *

196.  VCPY

Calls:  (none)

Called by:  PLOT3D

197.  VFACTOR

Calls:  (none)

Called by:  *, FACTOR

198.  VPEN

Calls:  NEWVPEM

Called by:  NEWPEN

199.  VPLOTS

Calls:  PLOTVT, NEWVPEN, NEWVCOL

Called by:  FRAME

200.  VRESPL (entry of VSAVPL)

Calls:  NEWVPEN, PLOTVT

Called by:  *, SAVPL

201.  VSAVPL

Calls:  (none)

Called by:  *, SAVPL

202.  VSRT1

Calls:  (none)

Called by:  LINHID

203.  VSRTR

Calls:  (none)

Called by:  LINHID

204.  VTPLOT

Calls:  (none)

Called by:  CURRECT, PLOTVT

205.  VXPT3D

Calls:  (none)

Called by:  CNT3DX, VAX3DX, MESH3DX

9-17

LONGLIB LIBRARY NAMES

206.  WHERE

Calls:  WHEREVT, WHERERM, WHEREPR

Called by:  *, SAVEPL, RESPL

207.  WHERE3D

Calls:  (none)

Called by:  *

208.  WHERE3H

Calls:  (none)

Called by:  *

209.  WHEREPR

Calls:  (none)

Called by:  *, WHERE

210.  WHERERM

Calls:  (none)

Called by:  *, WHERE

211.  WHEREVT

Calls:  (none)

Called by:  *, WHERE

212.  XFRM3D

Calls:  (none)

Called by:  *

213.  XVMUL3D

Calls:  MTV4

Called by:  AXIS3D

9-18

LONGLIB LIBRARY NAMES

9.2  COMMON BLOCK NAMES

The following is a list of all the named  common  block  used

internally  by the LONGLIB graphics library.  Those marked with an

asterick are documented under the first listed routine.   and  are

designed  to  be  accessable  to the user.  Those unmarked are for

internal use only and may change at later revisions.

1.  VT100 -- Accessed by:  BITCURSOR,  CURMOTION,  CURLOCATE,

CURBAND,  CURRECT,  GETCURSOR,  FRAME,  VTPLOTS,  VTPLOT,

CTERM, PLOTVT, WHEREVT, VFACTOR, FACTOR,  FIXVT0,  WHERE,

VSAVPL, VRESPL, NEWVPEN, NEWVCOL

2.  RMTEK -- Accessed  by:   CURMOTION,  CURLOCATE,  CURBAND,

CURRECT,   FRAME,   PLOTRM,   RPLOTS,  IRMCHAN,  WHERERM,

RFACTOR, RMPEN, FACTOR, FIXRM0, WHERE, RSAVPL, RRESPL

3.  PXPCOM -- Accessed by:  FRAME, PPLOT,  WHEREPR,  PFACTOR,

PPLOTS, PPEN, FACTOR, FIXPR0, WHERE, PSAVPL, PRESPL

4.  RAMTEKIO --  Accessed  by:   RAMOPEN,  RAMOUT,  RAMOUTIN,

RMCLEAR

5.  CHIDE -- Accessed by:  INIT3DH, PLT3DH, WHERE3H,  SKETCH,

LINHID

6.  HEDG -- Accessed by:  LINHID, CHECK3D

7.  GO3 -- Accessed  by:   LCOEF,  LINHID,  STAT3D,  STAT3D2,

CHECK3D, ISOL3D

8.  GO * -- Accessed by:  LINHID

9.  DAVE -- Accessed by:  CHECK3D, ISOL3D, STAT3D2

10.  LSTPLT -- Accessed by:  PLOT, WHERE

11.  PLT3B * -- Accessed by:  PLT3D, HLT3D,  CNDRAW,  MVAX3DX,

CVAX3DX

12.  LAST3D -- Accessed by:  DRAW3D, WHERE3D

13.  CPLOT3D -- Accessed by:  INIT3D, PLOT3D

14.  CGLPLOT * -- Accessed by:  GLPLOT

15.  CPLOTLG * -- Accessed by:  PLOTLG

16.  CPHIST * -- Accessed by:  PHIST

17.  CPLOTSC * -- Accessed by:  PLOTSC

9-19

LONGLIB LIBRARY NAMES

18.  CPLOTSC2 * -- Accessed by:  PLOTSC2

19.  LOCATE -- Accessed by:  VXPT3D, VAX3DX, CNT3DX,  RVXPT3D,

MESH3DX

20.  CPLOTLG * -- Accessed by:  PLOTLG

21.  CPLOTLG2 * -- Accessed by:  PLOTLG2

22.  CPLOTLGL * -- Accessed by:  PLOTLGL

23.  CPLOTLGX * -- Accessed by:  PLOTLGX

24.  CCNTRPLT * -- Accessed by:  CNTRPLT, CNTLN, LCNTR

25.  CSPLOTS * -- Accessed by:  SPLOTS

26.  CSPLOTSX * -- Accessed by:  SPLOTSX, SPLOTS

27.  CSEISPL * -- Accessed by:  SEISPL

28.  CSCATPL * -- Accessed by:  SCATPL

29.  TT_IO -- Accessed by:  INXTCHR, ICHRCHK

30.  ASTC  --  Accessed   by:    ASTINTER,   ENAST,   ASTEXIT,

ENABLEAST, PLOT

9-20

CHAPTER 10

PLOT EXAMPLES AND PLOTTING SYMBOLS

Shown in the sections that follow are outputs (and  listings)

of  various programs included with LONGLIB.  This include programs

which produce plotting symbols and line types/widths.

10.1  PLOTTING SYMBOLS AND CHARACTER FONTS EXAMPLES

The program SYMBOLS  was  used  to  create  a  table  of  the

available plotting symbols and character fonts available using the

subroutines SYMBOLS, SYMS and SYMSS.  These tables  are  shown  on

the succeeding pages.  A listing of SYMBOLS follows the tables.

10-1

PLOT EXAMPLES AND PLOTTING SYMBOLS

10.2  LINE TYPE/WIDTH EXAMPLES

The program LINETYPE was to create a table of  the  available

line  types,  widths,  and colors for each plotting device.  These

are shown on the succeding pages.  A listing of  LINETYPE  follows

the tables.

10-2

PLOT EXAMPLES AND PLOTTING SYMBOLS

10.3  MASTER ROUTINE OUTPUT EXAMPLES

The program PLOTTESTS was used to obtain examples of some  of

the MASTER routine outputs.  A listing follows the output pages.

10-3

PLOT EXAMPLES AND PLOTTING SYMBOLS

10.4  3-D ROUTINE OUTPUT EXAMPLES

EXAMP3D and EXAMP3DH demonstrate the 3d routines while  WORLD

demonstrates  the  some  of  the  3d map capabilities.  Output and

listings follow.

10-4

PLOT EXAMPLES AND PLOTTING SYMBOLS

10.5  CURSOR ROUTINE TEST PROGRAM

A listing of the program CURTEST follows.  The program can be

used  to test the operation of the graphics input cursor routines.

Only a listing is given.

10-5

PLOT EXAMPLES AND PLOTTING SYMBOLS

10.6  REF OUTPUT EXAMPLE

This section describes an  example  of  how  the  Ramtek  REF

subpackage  can  be  used  to  produce  a  grey scale "image".  An

example of the output image produced by first running the  example

program  REFTEST,  outputing  the result to a REF file, then using

the program REFLAS to produce a grey scale image on the QMS  laser

printer.  A listing of the REFTEST program is also provided.  Note

that REFTEST must be linked to the  REF  version  of  the  library

(LONGLIBR)  rather  than  the  "normal"  version.   Also note that

REFTEST, when linked to the "normal" LONGLIB, will produce a color

image on the Ramtek display.

10-6

CHAPTER 11

INDEX

11-1

Page Index-1

INDEX

4010, 1-6                           Cncelplt3d, 9-3

4014, 1-6                           Cndraw, 9-3

4107, 1-6                           Cnt3d, 7-8, 9-3

4109, 1-6                           Cnt3dx, 7-8, 9-3

Cntln, 7-11, 9-3

Absolute, 3-32, 4-2                 Cntour, 9-3

Absplt, 3-1, 9-1                    Cntrplt, 7-13, 9-3

Anxtvu, 9-1                         Color, 1-2, 1-11, 10-2

Applewriter, 1-10                   Color table, 8-8, 8-10

Arcalc, 9-1                         Common, 4-8, 7-2

Arcplt, 9-1                         Common block, 7-2

Arcset, 9-1                         Contour plot, 7-11, 7-13, 7-24

Area, 3-6, 3-26                     Cosmic, 3-14, 4-1, 4-5, 8-12

Arrow, 3-2, 9-1                     Cphist, 9-18

Ascii, 3-1, 3-16, 3-26 to 3-27,     Cplot3d, 4-3, 9-18

4-4, 7-2                        Cplotlg, 9-18 to 9-19

Astc, 9-19                          Cplotlg2, 9-19

Astexit, 9-1                        Cplotlgl, 9-19

Astinter, 9-2                       Cplotlgx, 9-19

Axis, 3-2, 3-11, 3-20, 3-27, 9-2    Cplotsc, 9-18

Axis., 3-11                         Cplotsc2, 9-18

Axis2, 3-3, 9-2                     Cscatpl, 9-19

Axis3, 3-4, 4-2, 4-6, 9-2           Cseispl, 9-19

Axis3d, 4-2, 9-2                    Cshade, 3-6, 9-3

Axis3dh, 4-6, 9-2                   Csplots, 9-19

Csplotsx, 9-19

Barchr, 7-6, 9-2                    Cterm, 1-3 to 1-4, 3-6, 3-24, 8-4,

Bit pad one, 5-1                        9-3

Bitcursor, 5-1, 9-2                 Cterm(2), 3-7

Bitmap, 6-4, 9-2                    Cube, 4-7, 9-3

Byte, 1-3, 3-1, 3-27, 7-2           Curband, 5-3, 9-4

Curlocate, 5-2, 8-9, 8-11, 9-4

C, 1-13, 2-1 to 2-2                 Curmotion, 5-2 to 5-3, 8-2, 8-8,

C language, 2-3                         8-10, 9-4

Cartesian, 3-10                     Currect, 5-3, 9-4

Ccntrplt, 9-19                      Cursor, 1-2 to 1-3, 5-1 to 5-2,

Cglplot, 9-18                           8-8 to 8-9

Channel, 8-2 to 8-3                 Curtest, 10-5

Character, 1-2 to 1-3, 1-14, 3-1,   Cvax3d, 7-16, 9-4

3-27, 7-2                       Cvax3dx, 7-16, 8-2, 9-4

Character fonts, 3-29

Character plots, 3-27, 3-30         3-d contour plot, 7-8

Characters, 3-26                    3-d plotting, 4-1

Check3d, 9-2                        5-d surface, 7-35, 7-69

Chide, 9-18                         2-d surface plot, 7-16, 7-29,

Chlskys, 9-2                            7-32, 7-66

Circle, 3-5, 9-2                    4-d surface plot, 7-35, 7-69

Clear, 1-4                          2-d surface plotting, 3-10, 3-20

Clip3d, 9-2                         Dashl, 3-7, 7-53, 9-4

Clpit, 9-3                          Dave, 9-18

Cncelplt, 9-3                       Dcl, 1-4, 1-8

Page Index-2

Descriptor, 3-27                    Image, 8-6, 8-8 to 8-11

Dot matrix printers, 1-7            Init3d, 4-1 to 4-3, 6-2, 9-5

Draw3d, 9-4                         Init3dh, 4-1 to 4-2, 4-5, 4-7,

7-22, 7-62, 7-64, 9-6

Earth, 6-1                          Integer, 3-1, 3-27

Earth.dat, 6-1                      Integer*2, 1-2, 8-9, 8-11

Earth3d, 6-2, 9-4                   Integer*4, 1-2, 3-1, 8-8, 8-10

Ellipse, 3-7, 9-4                   Interpc, 9-6

Enableast, 9-4                      Intersect, 9-6

Enast, 9-4                          Inxtchr, 1-3, 5-1, 8-2, 8-4, 9-6

Error bars, 7-59                    Ipclip, 8-1, 9-3

Examp3d, 4-2                        Ipclp3, 9-6

Examp3dh, 4-5                       Irmchan, 8-3, 9-6

Example, 2-4                        Isegcode, 9-6

Examples, 2-1                       Isol3d, 9-6

Factor, 3-1, 3-8, 3-17, 3-23,       Jpltag, 3-11, 9-6

3-30, 4-2, 4-5, 9-4

Fixpr0, 3-31, 9-5                   La50, 1-7

Fixrm0, 3-32, 9-5                   Landmap, 6-2, 9-6

Fixvt0, 3-32, 9-5                   Laser, 1-9

Form feed, 3-15                     Lasers, 1-9

Fortran, 1-3, 1-13, 2-1 to 2-2,     Last3d, 9-18

3-27, 7-2                       Lcntr, 7-24, 8-2, 9-6

Fortran,, 3-1                       Lcoef, 9-6

Fortran-77, 1-2                     Lgaxs, 3-11, 9-6

Frame, 1-3 to 1-4, 1-11, 3-8,       Lglin, 3-11 to 3-12, 3-25, 9-6

3-18, 4-1 to 4-2, 9-5           Lgrid, 3-10, 3-13, 9-7

Line, 3-7, 3-12 to 3-13, 3-25,

Gcontr, 7-16, 7-24, 8-1, 9-5            7-53, 9-7

Getcursor, 1-5, 5-4, 9-5            Line type, 1-2, 3-7, 3-14 to 3-15,

Gin, 5-4                                3-22, 3-24, 3-31, 10-2

Glplot, 7-20, 9-5                   Line width, 10-2

Go, 9-18                            Linear, 7-20, 7-42, 7-44, 7-46,

Go3, 9-18                               7-48, 7-55, 7-57

Graphics tablet, 5-1                Linetype, 10-2

Graphon, 1-6                        Linhid, 9-7

Greek, 3-27                         Linseq, 1-2, 3-14, 3-30, 9-7

Grid, 3-10, 3-13, 9-5               Lndsea, 6-3 to 6-5, 9-7

Lndsea1.dat, 6-1

Hedg, 9-18                          %loc(), 1-3

Help, 1-13, 3-11                    Locate, 9-19

Help library, 1-13                  Log, 7-20, 7-42, 7-44, 7-46, 7-48,

Helpme, 1-14, 3-11, 9-5                 7-55, 7-57

Hewlett-packard, 1-9                Logarithmic, 3-11 to 3-13

Hidden line removal, 4-1, 4-5 to    Logical, 3-1

4-9                             Longlibr, 8-5

Hist3d, 4-5, 7-22, 9-5              Longloc:, 1-12, 6-1, 6-3

Histogram, 7-38                     Lsplot, 7-26, 9-7

Hlt3d, 3-10, 3-21, 7-16, 7-32,      Lstplt, 9-18

9-5

Hpgl, 1-7, 1-9                      Machine dependent, 1-2

Hpgl2, 1-9                          Map, 6-1, 6-3

Hpgls, 1-9                          Map routines, 6-1

Hplt, 9-5                           Master, 1-2, 2-1, 3-27, 4-5, 7-1

Page Index-3

Math, 3-29                          Plt3d, 3-10, 3-20, 7-16, 7-32,

Matmul4, 8-3, 9-7                       8-2 to 8-3, 9-10

Mesh3d, 7-29, 9-7                   Plt3dh, 4-5 to 4-7, 4-9, 7-22,

Mesh3dx, 7-29, 9-7                      7-62, 7-64, 9-10

Metafile, 1-1, 1-6, 1-10            Polar axis, 3-20

Midc, 9-7                           Polar line, 3-20

Miscellaneous, 8-1                  Polarmap, 6-2, 9-10

Mtv4, 8-3, 9-7                      Poly1int, 9-10

Mvax3d, 7-32, 7-35, 9-7             Polygon, 4-8

Mvax3dx, 7-16, 7-32, 9-7            Postscript, 1-7, 1-10

Mvax5d, 1-3, 7-35, 9-8              Ppen, 3-15, 3-22, 3-24, 3-31,

9-10

Newpage, 3-15, 9-8                  Pplot, 3-17, 3-21, 9-10

Newpen, 3-15, 3-22, 3-24, 3-31,     Pplotp, 9-10

8-10, 9-8                       Pplots, 9-10

Newvcol, 1-3, 9-8                   Prespl, 3-23, 9-11

Newvpen, 1-3, 9-8                   Pretrp, 9-11

Num3d, 4-2 to 4-3, 4-7, 9-8         Printronix, 1-7

Num3dh, 4-6, 9-8                    Projection, 6-2, 6-4 to 6-5

Number, 3-16, 3-27, 4-3, 4-7, 9-8   Psavpl, 3-24, 9-11

Nxt0vu, 9-8                         Psubpro, 1-8, 9-11

Nxtvu, 3-10, 3-21, 8-3, 9-8         Pxpcgt, 9-11

Pxpcom, 9-18

Oldnumb, 9-8

Origin, 3-17, 4-3, 4-5, 4-7         Qms, 1-9, 1-12, 10-6

Quic, 1-7, 1-9

Page, 3-15

Pause, 8-2, 8-4, 9-8                Ramclose, 8-5, 9-11

Pausep, 8-4, 9-8                    Ramlib, 1-2

Pen plotter, 1-9                    Ramopen, 8-3, 8-7, 9-11

Pfactor, 3-8, 3-17, 3-23, 3-30,     Ramout, 8-7, 9-11

9-8                             Ramoutin, 9-11 to 9-12

Phist, 7-38, 9-9                    Ramtek, 1-10 to 1-11, 3-1, 3-9,

Pichrt, 7-40, 9-9                       3-15, 3-24, 3-31 to 3-32, 5-2,

Plot, 1-3, 1-11, 3-1, 3-17 to           8-5 to 8-11

3-19, 3-21, 4-1, 4-3, 9-9       Ramtek emulation, 1-10

Plot3d, 4-2 to 4-3, 6-2, 9-9        Ramtek emulation file, 1-11

Plotlg, 2-4, 7-42, 9-9              Ramtek image data, 8-6, 8-8 to

Plotlg2, 7-44, 9-9                      8-11

Plotlgl, 7-48, 9-9                  Ramtekio, 9-18

Plotlgx, 7-44, 7-46, 7-48, 9-9      Raster scan, 1-7

Plotnd, 1-12, 3-18, 9-9             Raster scan conversion, 1-7

Plotrm, 1-10 to 1-11, 3-17 to       Real, 3-1

3-18, 8-5, 9-9                  Rect, 3-23, 9-11

Plots, 3-8, 3-18, 9-9               Rectangle, 5-3

Plotsc, 2-1, 2-4 to 2-6, 7-51,      Rectangular, 6-3

9-10                            Ref, 1-10 to 1-11, 3-24, 8-5

Plotsc2, 7-53, 9-10                 %ref(), 1-3, 3-1, 3-27

Plottests, 2-4, 10-3                Refdis, 1-12, 3-24, 8-5, 9-11

Plotting window, 3-17               Reflas, 1-12, 10-6

Plotvt, 1-3, 3-17, 3-19, 8-11,      Reflas2, 1-12

9-10                            Reflib, 1-2

Plrax, 3-20, 9-10                   Refterm, 1-12

Plrln, 3-20, 9-10                   Reftest, 10-6

Plt3b, 3-10, 3-21, 8-2, 9-18        Replot, 1-7, 1-10

Page Index-4

Respl, 3-23 to 3-24, 9-11           Spiset, 9-14

Rfactor, 3-8, 3-17, 3-23, 3-30,     Splots, 7-59, 9-14

9-11                            Splotsx, 9-15

Rlaser, 1-9                         Sprect1, 6-3, 9-15

Rmclear, 8-6, 9-12                  Stat3d, 9-15

Rmdir, 8-6, 9-12                    Stat3d2, 9-15

Rmfntsize, 8-6, 9-12                String, 1-2

Rmpan, 8-7, 9-12                    Stripping, 1-7

Rmpen, 3-15, 3-22, 3-24, 3-31,      Strips, 1-7

8-10, 9-13                      Subproc, 1-8 to 1-9, 9-15

Rmplot, 8-7, 9-12                   Subprocess, 1-8 to 1-9

Rmreadbyte, 8-8, 9-12               Surface plot, 7-22, 7-62, 7-64

Rmreadcol, 8-8, 9-12                Sym3d, 4-2, 4-4, 9-15

Rmreadcursor, 8-8, 9-12             Sym3dh, 4-6, 4-9, 9-15

Rmreadword, 8-9, 9-12               Symbol, 3-26 to 3-27, 4-4, 4-9,

Rmreset, 9-12                           9-15

Rmsetcur, 8-9, 9-12                 Symbols, 4-4, 10-1

Rmsetcursor, 8-8                    Syms, 3-27, 3-30, 9-15, 10-1

Rmstart, 8-9, 9-12                  Symss, 3-30, 9-15, 10-1

Rmtek, 9-18

Rmtext, 8-9, 9-12                   T3dh, 4-5, 7-62

Rmtexture, 8-8, 8-10, 9-13          Tclear, 1-5

Rmwind, 8-10, 9-13                  Tekclr, 1-6

Rmwritebyte, 8-10, 9-13             Tektronix, 1-3, 1-6, 5-4

Rmwritecol, 1-11, 8-10, 9-13        Tektronix 4010/4014, 3-8

Rmwriteword, 1-11, 8-11, 9-13       Tektronix 4107/4109, 3-8

Rmzoom, 8-11, 9-13                  Terminal, 1-3, 3-6, 3-9, 3-15,

Rotation, 4-3                           3-31 to 3-32, 5-2, 8-11

Rotem, 9-13                         Text, 3-18, 8-6

Rplots, 1-10, 9-13                  Tr3dh, 9-15

Rrespl, 3-23, 9-13                  Transformation, 3-10, 3-21, 8-2

Rsavpl, 3-24, 9-13                  Trcelplt3d, 9-15

Rterm, 3-24, 9-13                   Triangc, 7-11, 7-64, 8-12, 9-15

Rvxpt3d, 9-13                       Trig3dh, 4-5, 7-64, 9-16

Trilghi, 1-7

Savpl, 3-23 to 3-24, 9-13           Trilglo, 1-7

Scale, 3-13, 3-25, 4-2, 4-5, 9-14   Trilog, 1-7

Scalg, 3-12, 3-25, 9-14             Tt_io, 9-19

Scatpl, 7-55, 9-14

Scatter plot, 7-55                  Vax, 1-2 to 1-3, 1-7 to 1-13,

Screen coordinates, 4-3                 1-15, 2-1, 2-3, 3-1, 3-11,

Segcode, 9-14                           3-27, 7-2

Seismic plotting, 7-57              Vax/vms, 1-2, 1-11 to 1-14

Seispl, 7-57, 9-14                  Vax3d, 7-29, 7-66, 7-69, 9-16

Selanar, 1-3 to 1-4, 3-8, 3-32      Vax3dx, 7-29, 7-66, 9-16

Sfplot, 9-14                        Vax3dx., 7-8

Shade, 3-26, 9-14                   Vax5d, 1-3, 7-69, 9-16

Sketch, 4-5 to 4-9, 7-22, 7-62,     Vclear, 1-5

7-64, 9-14                      Vcpy, 9-16

Smoothc, 9-14                       Vfactor, 3-8, 3-17, 3-23, 3-30,

Software line type, 3-7, 3-14           9-16

Source code, 1-13                   View port, 3-17

Spherical, 6-3                      Vms, 1-8, 1-15, 3-11

Spider, 9-14                        Vpen, 3-15, 3-22, 3-24, 3-31,

Spifun, 9-14                            9-16

Page Index-5

Vplots, 1-3, 9-16                   Where, 3-31, 9-17

Vrespl, 3-23, 9-16                  Where3d, 4-4, 9-17

Vsavpl, 3-24, 9-16                  Where3h, 4-9, 9-17

Vsrt1, 9-16                         Wherepr, 3-31, 9-17

Vsrtr, 9-16                         Whererm, 3-32, 9-17

Vt100, 1-3 to 1-5, 3-8, 3-32, 8-2,  Wherevt, 3-32, 9-17

9-18                            Window, 3-17

Vt125, 1-3 to 1-4, 3-8, 3-32, 5-1

Vt220, 1-5, 3-8, 3-32

Vt240, 1-3, 1-5, 3-8, 3-32          Xfrm3d, 4-9, 9-17

Vtplot, 1-3, 8-11, 9-16             Xor, 5-2

Vxpt3d, 9-17                        Xvmul3d, 8-12, 9-17